[image: image1.jpg]


FLASH

[image: image2.jpg]



FLASH FL 745
Manufacturer: RIM CZ a.s.
Design:
	FLASH FL 745


Swivel office chair with separate seat and high backrest structure, with synchronous mechanism and on castors. 
All materials may be easily separated for waste separation and recycling.
The chair meets the requirements for easy use, maintenance, and service.
	Mechanism
	Synchronous mechanism STA with movement and active seat and backrest tilt
Synchronous mechanism with active seat tilt - the seat movement is not blocked and actively tilts (balances) with the movement of seated person. The range of seat tilt is +2 up to -9°. The seat can be locked in the basic position 0°.
The mechanism’s continual counter-pressure may be set within the range of approx. 45 – 120 kg of body weight. Setting the stiffness of mechanism from minimum to maximum with two turns by a small wheel. 


	Seat
	Bowl shaped seat upholstered with 55 mm PUR foam, bulk weight of 70 kg/m3, free of CFCs and phthalate. The seat is coated with fabric or leather without using glue. Depth-adjustable within the range of 60 mm, seat tilt adjustment +2 up to  -9° (including backrest).
Height-adjustable seat in the range of 120 mm, from 410 to 530 mm. 


	Backrest

	Ergonomically shaped backrest upholstered with PUR foam thickness of 50 mm, bulk weight of 70 kg/m³ and coated with fabric or leather. Height-adjustable within the range of 70 mm with 8 locking positions.


	Upholstery
	To be selected from fabric or leather collection. 


	Base
	Five-star stable base ø 68 cm, material polyamide.


	Castors
	Black hard castors Ø 60 mm, braked – depending on the load, for soft or hard floor surface (as per DIN EN 12529). 
The chair does not move when released which reduces the risk of an accident.


	Plastic components
	Coloured, black. It provides colour stability even with deep scratches.


	Armrests
	· 086 – multifunctional T-shaped armrests with aluminium holder. Height-adjustable within the range of 8 cm and with pressing the side button armpad may be adjusted forward in the range of 5 cm and to a width of 3 cm. The top of the armrests is covered with softened PU. Armrests are attached to the mechanism (not seat). 
· 088 - multifunctional T-shaped armrest either all plastic or with polished aluminium holder. Height-adjustable within the range of 10 cm, armrest angle-adjustable within the range of 30°, and with a simple movement (eccentric) width-adjustable within the range of 45 – 53 cm to increase the seating area. The top of armrests is covered with softened PU.


	Options
	· Height-adjustable headrest
· Depth-adjustable lumbar support in the lower back area (30 mm)
· Seat-depth adjustment in the range of 10 cm (possible only with black plastic base  Ø 71 cm)
· Base of  Ø 68 cm, polished Alu or painted to RAL 9006
· Black plastic base of  Ø 71 cm 
· Gas unit with suspension
· Gas unit for seat height 450 – 570 mm
· Castors with chrome ring for hard or soft floor
· Glides 60 mm high
· Felt glides 60 mm high


	Certificate
	This product complies with European dimension and safety standards. This product complies with ergonomic standards as per 2 PfG 947/07.13. 
All materials may be easily separated for waste separation and recycling.
The chair meets the requirements for easy use, maintenance, and service.


	Warranty
	5 years (see Business conditions)



	Dimensions (in mm)


[image: image3.jpg]



[image: image4.jpg]



	Concept FLASH


[image: image5.png]TUVRheinland

www.tuv.com
ID 1111220837

epriifte
icherheit




��
��
��
�
FL 745.089 NPR�
        FL 750E�
        FL 760E�
�
 








[image: image6.png]. ®
TUVRheinland
CERTIFIED

Ergonomics
Regular
Product
Surveillance

www.tuv.com
1D 1111220874



[image: image7.png]QUALITY

OFFICE



[image: image8.jpg]


[image: image9.jpg]0€S-0L¥

380-440

490
680

GG9-089

680




[image: image10.jpg]




