Specification Table

| Description |  | L403-6W | L403-9W |  | L403-12W | L403-15W |
| --- | --- | --- | --- | --- | --- | --- |
| Input Driver Voltage (V) | 24V |  |  | 24V |  |  |
| Input Driver Current (A) |  | 1.5A |  |  | 1.5A |  |
| Power-Watts |  | 6W | 9W |  | 12W | 15W |
| LED Light Bar Sections |  | 2pcs/set |  |  | 2pcs/set |  |
| Lumens |  | 2x570LM | 2x855LM |  | 2x1110LM | 2x1425LM |
| CCT Rating |  | 4000K |  |  | 4000K |  |
| CRI Rating |  | 80 |  |  | 80 |  |
| LED Count/Type |  | 154pcs/2835 | 224pcs/2835 |  | 350pcs/2835 | 420pcs/2835 |
| IR Type |  | IR+touch(dimmable) |  |  | IR+touch(dimmable) |  |
| Input Connection |  | T-Type connect cable (18x32x4) |  |  |  |  |
| Dimensions |  | L711(28”) 24x10mm | L1067(42”) 24x10mm |  | L1550(61”) 24x10mm | L1854(73”) 24x10mm |

Power Supply Features

| Model Number | LYD2401500UB |
| --- | --- |
| Part number | 209021903 |
| Input Voltage Rated Range/Frequency | 100-240VAC 50/60Hz |
| Input Voltage Variation Range/Frequency | 90-264VAC 47/63Hz |
| Input Current (A) | 1.6A |
| Input Inrush Current (A) | 50A Max. Cold start at 240Vac input with rated load and 25oC ambient |
| Output Voltage (Vdc) | 24Vdc |
| Output Minimum Current (A) | 0A |
| Output Maximum Current (A) | 2A |
| Output Power (W) | 48W |
| Output Combined Minimum Load (A) | 0A |
| Output Combined Maximum Load (A) | 2A |
| Output Load Regulation (V) | 22.8V-25.2V |
| Output Standby Power (W) | 0.1W Max |
| Output Ripple | 240mVp-p |
| Output Noise | 480mVp-p |
| Efficiency | 87.4% at 100Vac/87.4% at 240Vac |
| Short Circuit Protection | The power supply will auto recover when short circuit faults are removed. |
| Over Current Protection | The power supply will auto recover when over current faults are removed. |
| Operating Temperature | 0oC ~ 40oC |
| Certifications | UL, FCC, CE, GS |
| Dimensions (LxWxH) | 123x55x35mm |
| DC Cable Length | 1.83mm |
| DC Cable Gauge/VW-1 Rating | 20AWG/300V |
| DC Cable Plug Polarity | Inner-positive/outer-negative *see photo |
| Surge Rating | 1KV |

Light Bar Certifications

| Certifications | FCC Class B, RoHS,CE |
| --- | --- |

Warranty

| Light Products | 3 Years |
| --- | --- |

For complete warranty information, refer to http://www.middleatlantic.com/company/about-us.aspx#warranty

*DC Cable Plug Polarity: