Middle Atlantic Products, Inc.	August 2006

Phone	(973) 839-1011

Fax	(973) 839-1976

Website	middleatlantic.com

E-mail	info@middleatlantic.com

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, including MasterFormat, SectionFormat, and PageFormat, as described in The Project Resource Manual—CSI Manual of Practice.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” when editing this section.

Section numbers and titles are from MasterFormat 2004 Edition.

SECTION 27 11 16

CABLE MANAGEMENT RACKS

Specifier Notes:  This section covers Middle Atlantic Products, Inc. MK Series Cable Management Rack.  Consult Middle Atlantic Products for assistance in editing this section for the specific application.

PART 1	GENERAL

1.1	SECTION INCLUDES

A.	Cable management racks.

1.2	RELATED SECTIONS

Specifier Notes:  List sections with work directly related to this section.

A.

1.3	REFERENCES

Specifier Notes:  List standards referenced in this section, complete with designations and titles.  This article does not require compliance with standards, but is merely a listing of those used.

A.	EIA/TIA 310D – Cabinets, Racks, Panels and Associated Equipment.

B.	ISO 9001:2000 – Quality Management Systems – Requirements.

1.4	SUBMITTALS

A.	Comply with Section 01 33 00 – Submittal Procedures.

B.	Product Data:  Submit manufacturer’s product data, including options.

C.	Warranty:  Submit manufacturer’s standard warranty.

1.5	QUALITY ASSURANCE

A.	Manufacturer’s Qualifications:  ISO 9001:2000 registered company.

1.6	DELIVERY, STORAGE, AND HANDLING

A.	Delivery:  Deliver materials to site in manufacturer’s original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

B.	Storage:  Store materials in clean, dry area indoors in accordance with manufacturer’s instructions.

C.	Handling:  Protect materials and finish during handling and installation to prevent damage.

1.7	WARRANTY

A.	Warranty Period:  Product lifetime.

PART 2	PRODUCTS

2.1	MANUFACTURER

A.	Middle Atlantic Products, Inc.  Phone (973) 839-1011.  Fax (973) 839-1976.  Website middleatlantic.com.  E-mail info@middleatlantic.com.

2.2	CABLE MANAGEMENT RACKS

A.	Cable Management Racks:  MK Series Cable Management Rack.

Specifier Notes:  Specify cable management rack model number.  Consult Middle Atlantic Products product information for available models.

1.	Model:  MK-__________ .

2.	Type:  19-inch cable management rack.

3.	Compliance:  EIA/TIA 310D.

4.	Overall Dimensions:

a.	Height:  84.25 inches (2,140 mm).

b.	Width:  24 inches (610 mm).

Specifier Notes:  Specify overall depth.

c.	Depth:

1)	At Base:  _____ inches ( _____ mm).

2)	Rail Channels:  _____ inches ( _____ mm).

3)	At Top, Including Integrated Cable Trough:  _____ inches ( _____ mm).

5.	Useable Height:  45 rackspaces.

6.	Construction:  Fully welded.

7.	Weight Capacity:

a.	Evenly Distributed with Base Securely Fastened:  500 pounds.

b.	Evenly Distributed with Base Securely Fastened and Top Braced to Structural Building Component:  800 pounds.

8.	Cable Management Rings:  Steel, multi position, front mount, 20 places.

9.	7-Inch Obround Cable Pass-Through Openings:

a.	Front:  12.

b.	Each Side:  8.

10.	Integrated Top Cable Trough:  Front and rear.

11.	Multiple Raised Cable Lacing Points:  Cable management.

12.	Cable Fastening Straps:  Hook and loop.

13.	Front Cable Feed-Through Hole Plugs:  8.

14.	Rack Screws:  10-32 threads, 100.

15.	Finish:  Phosphate pretreated, black textured powder coat.

2.3	OPTIONS

Specifier Notes:  Specify required options.  Delete options not required.  Consult Middle Atlantic Products for more information.

A.	Horizontal lacer bars.

B.	Horizontal cable managers.

C.	Cable ladder.

D.	Cable ladder adapter.

E.	Rackmount Cable Tray:  [One rackspace]  [Two rackspaces].

F.	Vented center mount shelf.

PART 3	EXECUTION

3.1	EXAMINATION

A.	Examine areas to receive cable management racks.

B.	Notify Architect of conditions that would adversely affect installation or subsequent use.

C.	Do not begin installation until unacceptable conditions are corrected.

3.2	INSTALLATION

A.	Install cable management racks in accordance with manufacturer’s instructions at locations indicated on the Drawings.

B.	Install cable management racks plumb, level, square, and secure.

3.3	ADJUSTING

A.	Repair minor damages to finish in accordance with manufacturer’s instructions and as approved by Architect.

3.4	PROTECTION

A.	Protect installed cable management racks from damage during construction.

END OF SECTION