Middle Atlantic Products, Inc.	August 2006

Phone	(973) 839-1011

Fax	(973) 839-1976

Website	middleatlantic.com

E-mail	info@middleatlantic.com

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, including MasterFormat, SectionFormat, and PageFormat, as described in The Project Resource Manual—CSI Manual of Practice.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” when editing this section.

Section numbers and titles are from MasterFormat 2004 Edition.

SECTION 27 11 16

PORTABLE EQUIPMENT RACKS

Specifier Notes:  This section covers Middle Atlantic Products, Inc. PTRK Series Portable Rack.  Consult Middle Atlantic Products for assistance in editing this section for the specific application.

PART 1	GENERAL

1.1	SECTION INCLUDES

A.	Portable equipment racks.

1.2	RELATED SECTIONS

Specifier Notes:  List sections with work directly related to this section.

A.

1.3	REFERENCES

Specifier Notes:  List standards referenced in this section, complete with designations and titles.  This article does not require compliance with standards, but is merely a listing of those used.

A.	EIA/TIA 310D – Cabinets, Racks, Panels and Associated Equipment.

B.	ISO 9001:2000 – Quality Management Systems – Requirements.

1.4	SUBMITTALS

A.	Comply with Section 01 33 00 – Submittal Procedures.

B.	Product Data:  Submit manufacturer’s product data, including options.

C.	Warranty:  Submit manufacturer’s standard warranty.

1.5	QUALITY ASSURANCE

A.	Manufacturer’s Qualifications:  ISO 9001:2000 registered company.

1.6	DELIVERY, STORAGE, AND HANDLING

A.	Delivery:  Deliver materials to site in manufacturer’s original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

B.	Storage:  Store materials in clean, dry area indoors in accordance with manufacturer’s instructions.

C.	Handling:  Protect materials and finish during handling and installation to prevent damage.

1.7	WARRANTY

A.	Warranty Period:  Product lifetime.

PART 2	PRODUCTS

2.1	MANUFACTURER

A.	Middle Atlantic Products, Inc.  Phone (973) 839-1011.  Fax (973) 839-1976.  Website middleatlantic.com.  E-mail info@middleatlantic.com.

2.2	PORTABLE EQUIPMENT RACKS

A.	Portable Equipment Racks:  PTRK Series Portable Rack.

Specifier Notes:  Specify portable equipment rack model number.  Consult Middle Atlantic Products product information for available models.

1.	Model:  PTRK-__________ .

2.	Type:  19-inch portable equipment rack.

3.	Compliance:  EIA/TIA 310D.

Specifier Notes:  Specify overall and useable dimensions.

4.	Overall Dimensions:

a.	Height:  _____ inches ( _____ mm).

b.	Width:  22.59 inches (574 mm).

c.	Depth:  _____ inches ( _____ mm).

5.	Useable Dimensions:

a.	Height:  _____ rackspaces.

b.	Depth:  _____ inches ( _____ mm).

6.	Weight Capacity:  500 pounds.

7.	Materials:

Specifier Notes:  Specify top.

a.	Top:  [16-gauge steel]  [Graphite laminate wood].

b.	Bottom and Sides:  16-gauge steel.

Specifier Notes:  Specify front and rear doors.

c.	Front Door:  18-gauge steel, [solid]  [plexi].

d.	Rear Door:  18-gauge steel, [solid]  [vented].

e.	Internal Braces:  1/8-inch thick, structural steel.

8.	Finish of Structural Elements:  Black textured powder coat.

9.	Rackrail:

a.	11-gauge steel with tapped 10-32 mounting holes in universal EIA spacing.

b.	Fully adjustable.

c.	Finish:  Black e-coat.

d.	Rackspaces:  Numbered.

10.	Casters:  4-inch, smooth-rolling, locking, cushion casters.

11.	Keylocks:  Front and rear doors.

12.	Handles:

a.	Each side.

b.	Rubber padded, spring loaded, recessed.

13.	Rear Knockouts:  Top and bottom, for cable pass-through.

14.	Removable Decora-Style Plate:  Top front.

2.3	OPTIONS

Specifier Notes:  Specify required options.  Delete options not required.  Consult Middle Atlantic Products for more information.

A.	Rear Rackrail:  11-gauge steel with tapped 10-32 mounting holes in universal EIA spacing.

B.	Single LCD Monitor Mount:  Accommodates VESA 75 and VESA 100 monitor.

C.	Universal Connector Panel Knockouts:  Accommodates commonly used connectors.

D.	Document pocket.

PART 3	EXECUTION

3.1	EXAMINATION

A.	Examine areas to receive portable equipment racks.

B.	Notify Architect of conditions that would adversely affect installation or subsequent use.

C.	Do not begin installation until unacceptable conditions are corrected.

3.2	INSTALLATION

A.	Install equipment racks in accordance with manufacturer’s instructions at locations indicated on the Drawings.

B.	Install equipment racks plumb, level, and square.

3.3	ADJUSTING

A.	Adjust operating hardware to operate smoothly without binding.

B.	Repair minor damages to finish in accordance with manufacturer’s instructions and as approved by Architect.

3.4	PROTECTION

A.	Protect installed portable equipment racks from damage during construction.

END OF SECTION