Middle Atlantic Products, Inc.	January 2007

Phone	(973) 839-1011

Fax	(973) 839-1976

Website	middleatlantic.com

E-mail	info@middleatlantic.com

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, including MasterFormat, SectionFormat, and PageFormat, as described in The Project Resource Manual—CSI Manual of Practice.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” when editing this section.

Section numbers and titles are from MasterFormat 2004 Edition.

SECTION 27 11 16

OPEN-FRAME EQUIPMENT RACKS

Specifier Notes:  This section covers Middle Atlantic Products, Inc. R4 Series Four Post Open-Frame Rack.  Consult Middle Atlantic Products for assistance in editing this section for the specific application.

PART 1	GENERAL

1.1	SECTION INCLUDES

A.	Four post open-frame equipment racks.

1.2	RELATED SECTIONS

Specifier Notes:  List sections with work directly related to this section.

A.

1.3	REFERENCES

Specifier Notes:  List standards referenced in this section, complete with designations and titles.  This article does not require compliance with standards, but is merely a listing of those used.

A.	EIA/TIA 310D – Cabinets, Racks, Panels and Associated Equipment.

B.	ISO 9001:2000 – Quality Management Systems – Requirements.

1.4	SUBMITTALS

A.	Comply with Section 01 33 00 – Submittal Procedures.

B.	Product Data:  Submit manufacturer’s product data, including options.

C.	Warranty:  Submit manufacturer’s standard warranty.

1.5	QUALITY ASSURANCE

A.	Manufacturer’s Qualifications:  ISO 9001:2000 registered company.

1.6	DELIVERY, STORAGE, AND HANDLING

A.	Delivery:  Deliver materials to site in manufacturer’s original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

B.	Storage:  Store materials in clean, dry area indoors in accordance with manufacturer’s instructions.

C.	Handling:  Protect materials and finish during handling and installation to prevent damage.

1.7	WARRANTY

A.	Warranty Period:  Product lifetime.

PART 2	PRODUCTS

2.1	MANUFACTURER

A.	Middle Atlantic Products, Inc.  Phone (973) 839-1011.  Fax (973) 839-1976.  Website middleatlantic.com.  E-mail info@middleatlantic.com.

2.2	FOUR POST OPEN-FRAME EQUIPMENT RACKS

A.	Four Post Open-Frame Equipment Racks:  R4 Series Four Post Open-Frame Rack.

Specifier Notes:  Specify open-frame equipment rack model number.  Consult Middle Atlantic Products product information for available models.

1.	Model:  R4__________ .

2.	Type:  19-inch four post open-frame equipment rack.

3.	Compliance:  EIA/TIA 310D.

Specifier Notes:  Specify overall height, width, depth and useable height.

4.	Overall Dimensions:

a.	Height:  _____ inches (_____ mm).

b.	Width:  _____ inches (_____ mm).

c.	Depth:  _____ inches (_____ mm).

5.	Useable Height:  _____ rackspaces.

6.	Assembly:  Field assembly, self-aligning design.

Specifier Notes:  Specify weight capacity. R412 models have 2,500 pound weight capacity, R4CN models have 1,000 pound weigh capacity.

7.	Weight Capacity: [2,500] [1,000] pounds.

Specifier Notes:  Specify rackrail type.

8.	Rackrail: [12-24 threaded] [cage nut] mounting holes in universal EIA spacing.

9.	Top Angles:  14-gauge steel.

10.	Gusseted Foot Base:  Reversible for increased stability or to save floor space.

11.	Mounting Hardware:  100.

12.	Finish:  Black powder coat.

2.3	OPTIONS

Specifier Notes:  Specify required options.  Delete options not required.  Consult Middle Atlantic Products for more information.

- Removable Keylocked Side Panels: 20-gauge steel with recessed lift handles.

- Vertical Cable Duct: [single] [dual] duct, cable grommets, solid front door.

- Additional Rail Kit: [12-24 threaded] [cage nut], in pairs, includes hardware.

- Cage Nut Z-Rail Adapter Kit.

- Cable Ladder: 12-inch wide, [6-foot long] [10-foot long].

- Vertical Lacer Strip.

- Vertical Power Strip.

PART 3	EXECUTION

3.1	EXAMINATION

A.	Examine areas to receive four post open-frame equipment racks.

B.	Notify Architect of conditions that would adversely affect installation or subsequent use.

C.	Do not begin installation until unacceptable conditions are corrected.

3.2	INSTALLATION

A.	Install four post open-frame equipment racks in accordance with manufacturer’s instructions at locations indicated on the Drawings.

B.	Install four post open-frame equipment racks plumb, level, square, and secure.

3.3	ADJUSTING

A.	Repair minor damages to finish in accordance with manufacturer’s instructions and as approved by Architect.

3.4	PROTECTION

A.	Protect installed four post open-frame equipment racks from damage during construction.

END OF SECTION