Middle Atlantic Products, Inc.	January 2019

Phone	(973) 839-1011

Fax	(973) 839-1976

Website	middleatlantic.com

E-mail	info@middleatlantic.com

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, including MasterFormat, SectionFormat, and PageFormat, as described in The Project Resource Manual—CSI Manual of Practice.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” when editing this section.

Section numbers and titles are from MasterFormat 2004 Edition.

SECTION 27 11 16

EQUIPMENT ENCLOSURES

Specifier Notes:  This section covers Middle Atlantic Products, Inc. Slim 5 Series 19-Inch Equipment Enclosure.  Consult Middle Atlantic Products for assistance in editing this section for the specific application.

PART 1	GENERAL

1.1	SECTION INCLUDES

A.	Equipment enclosures.

1.2	RELATED SECTIONS

Specifier Notes:  List sections with work directly related to this section.

A.

1.3	REFERENCES

Specifier Notes:  List standards referenced in this section, complete with designations and titles.  This article does not require compliance with standards, but is merely a listing of those used.

A.	EIA/TIA 310D – Cabinets, Racks, Panels and Associated Equipment.

B.	ISO 9001:2000 – Quality Management Systems – Requirements.

C.	UL – Underwriters Laboratories.

1.4	SUBMITTALS

A.	Comply with Section 01 33 00 – Submittal Procedures.

B.	Product Data:  Submit manufacturer’s product data, including options.

C.	Warranty:  Submit manufacturer’s standard warranty.

1.5	QUALITY ASSURANCE

- Manufacturer’s Qualifications:  ISO 9001:2000 registered company.

- Conform to ISO 14025 and EN 15804 or ISO 21930 in the development of life cycle assessments (LCA) and Type III environmental product declarations (EPD).

1.6	DELIVERY, STORAGE, AND HANDLING

A.	Delivery:  Deliver materials to site in manufacturer’s original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

B.	Storage:  Store materials in clean, dry area indoors in accordance with manufacturer’s instructions.

C.	Handling:  Protect materials and finish during handling and installation to prevent damage.

1.7	WARRANTY

A.	Warranty Period:  Product lifetime.

PART 2	PRODUCTS

2.1	MANUFACTURER

A.	Middle Atlantic Products, Inc.  Phone (973) 839-1011.  Fax (973) 839-1976.  Website middleatlantic.com.  E-mail info@middleatlantic.com.

2.2	EQUIPMENT ENCLOSURES

A.	Equipment Enclosures:  Slim 5 Series 19-Inch Equipment Enclosure.

Specifier Notes:  Specify equipment enclosure model number.  Consult Middle Atlantic Products product information for available models.

1.	Model:  5-__________ .

2.	Type:  19-inch gangable equipment enclosure.

3.	Compliance:  EIA/TIA 310D.

4.	UL Listed:  US and Canada.

Specifier Notes:  Specify overall and useable dimensions.

5.	Overall Dimensions:

a.	Width:  19.125 inches (486 mm).

b.	Depth:  _____ inches ( _____ mm).

c.	Height:  _____ inches ( _____ mm).

6.	Useable Dimensions:

a.	Useable Depth:  _____ inches ( _____ mm).

b.	Useable Height:  _____ rackspaces.

7.	Construction:  Knock down.

8.	Weight Capacity:  400 pounds.

9.	Materials:

a.	Rack Top and Bottom:  14-gauge steel.

b.	Horizontal Braces:  14-gauge steel.

10.	Finish of Top, Bottom, and Braces:  Black powder coat.

11.	Rack Uprights:

a.	11-gauge steel rackrail with tapped 10-32 mounting holes in universal EIA spacing.

b.	Finish:  Black e-coat.

c.	Rackspaces:  Bracketed.

12.	Cable Knockouts:  1/2 inch and 3/4 inch, 4 in top, 4 in base.

2.3	OPTIONS

Specifier Notes:  Specify required options.  Delete options not required.  Consult Middle Atlantic Products for more information.

A.	Front Doors:  [Solid, 16-gauge steel]  [Plexi].

B.	Removable Side Panels:

1.	18-gauge steel with recessed lift pulls.

2.	Steel Locking Clips:  Secure side panels.

C.	Rear Access Pan:

1.	18-gauge steel.

2.	Keylock.

D.	Cable Chase Kit:

1.	16-gauge steel.

2.	3 inches wide for wire clearance when ganging.

E.	Adjustable Split Rackrail:

1.	Rear equipment support.

2.	11-gauge steel.

3.	Mount to accompanying 14-gauge steel brackets.

F.	Casters:

1.	Set of 4.

2.	Type:  [Heavy duty]  [Fine-floor friendly].

3.	Total Weight Capacity:  [1,300 pounds]  [700 pounds].

4.	Overall Height:  3.875 inches (98 mm).

G.	Skirted Caster Base:

1.	Type:  [Heavy duty]  [Fine-floor friendly].

2.	Total Weight Capacity:  [1,300 pounds]  [700 pounds].

3.	Overall Height:  3.875 inches (98 mm).

H.	Leveling Feet:

1.	3/8-inch threaded steel.

2.	Adjustment Range:  1.25 inches (32 mm).

I.	Copper buss bar.

J.	Door latch.

PART 3	EXECUTION

3.1	EXAMINATION

A.	Examine areas to receive equipment enclosures.

B.	Notify Architect of conditions that would adversely affect installation or subsequent use.

C.	Do not begin installation until unacceptable conditions are corrected.

3.2	INSTALLATION

A.	Install equipment enclosures in accordance with manufacturer’s instructions at locations indicated on the Drawings.

B.	Install equipment enclosures plumb, level, square, and secure.

3.3	ADJUSTING

A.	Adjust operating hardware to operate smoothly without binding.

B.	Repair minor damages to finish in accordance with manufacturer’s instructions and as approved by Architect.

3.4	PROTECTION

A.	Protect installed equipment enclosures from damage during construction.

END OF SECTION