Middle Atlantic Products, Inc.	August 2006

Phone	(973) 839-1011

Fax	(973) 839-1976

Website	middleatlantic.com

E-mail	info@middleatlantic.com

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, including MasterFormat, SectionFormat, and PageFormat, as described in The Project Resource Manual—CSI Manual of Practice.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” when editing this section.

Section numbers and titles are from MasterFormat 2004 Edition.

SECTION 27 11 16

SEISMIC-CERTIFIED RISER BASES

Specifier Notes:  This section covers Middle Atlantic Products, Inc. SRB Series Seismic-Certified Riser Bases.  Consult Middle Atlantic Products for assistance in editing this section for the specific application.

PART 1	GENERAL

1.1	SECTION INCLUDES

A.	Seismic-certified riser bases.

1.2	RELATED SECTIONS

Specifier Notes:  List sections with work directly related to this section.

A.

1.3	REFERENCES

Specifier Notes:  List standards referenced in this section, complete with designations and titles.  This article does not require compliance with standards, but is merely a listing of those used.

A.	ASCE 7 – Minimum Design Loads for Buildings and Other Structures.

B.	CBC – California Building Code.

C.	IBC – International Building Code.

D.	ISO 9001:2000 – Quality Management Systems – Requirements.

E.	NFPA 5000 – Building Construction and Safety Code.

F.	UBC – Uniform Building Code.

1.4	SUBMITTALS

A.	Comply with Section 01 33 00 – Submittal Procedures.

B.	Product Data:  Submit manufacturer’s product data, including options.

C.	Warranty:  Submit manufacturer’s standard warranty.

1.5	QUALITY ASSURANCE

A.	Manufacturer’s Qualifications:  ISO 9001:2000 registered company.

1.6	DELIVERY, STORAGE, AND HANDLING

A.	Delivery:  Deliver materials to site in manufacturer’s original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

B.	Storage:  Store materials in clean, dry area indoors in accordance with manufacturer’s instructions.

C.	Handling:  Protect materials and finish during handling and installation to prevent damage.

1.7	WARRANTY

A.	Warranty Period:  Product lifetime.

PART 2	PRODUCTS

2.1	MANUFACTURER

A.	Middle Atlantic Products, Inc.  Phone (973) 839-1011.  Fax (973) 839-1976.  Website middleatlantic.com.  E-mail info@middleatlantic.com.

2.2	SEISMIC-CERTIFIED RISER BASES

A.	Seismic-Certified Riser Bases:  SRB Series Seismic-Certified Riser Bases.

Specifier Notes:  Specify riser base model number.  Consult Middle Atlantic Products product information for available models.

1.	Model:  SRB-__________ .

2.	Satisfies the following codes for protecting essential equipment in upper floor installations when used with MRK/VRK-Z4 floor anchor brackets with Ip value of 1.5.

a.	1997 UBC and 2001 CBC:  Seismic Zone 4.

b.	2000 IBC, 2003 IBC, ASCE 7-02, and 2003 NFPA 5000:  Seismic Use Group III. lateral force requirements.

3.	Weight Capacity:

a.	One Bay:  1,000 pounds.

b.	Two Bay:  1,500 pounds.

c.	Three Bay:  2,250 pounds.

Specifier Notes:  Specify dimensions.  Consult Middle Atlantic Products for information regarding custom heights.

4.	Dimensions:

a.	Height:  [24.00 inches (610 mm)]  [ _____ inches ( _____ mm)].

b.	Width:  _____ inches ( _____ mm).

c.	Depth:  _____ inches ( _____ mm).

5.	Construction:  Fully welded.

6.	Material:  11-gauge steel.

7.	Finish:  Black-textured powder coat.

8.	Rack-Mounting Holes:  Top.

9.	Floor-Mounting Holes:  Bottom.

2.3	OPTIONS

Specifier Notes:  Specify required options.  Delete options not required.  Consult Middle Atlantic Products for more information.

A.	Support Angles:  11-gauge steel.

1.	Finish:  Black powder coat.

B.	Custom Heights:  9 inches to 24 inches.

PART 3	EXECUTION

3.1	EXAMINATION

A.	Examine areas to receive seismic-certified riser bases.

B.	Notify Architect of conditions that would adversely affect installation or subsequent use.

C.	Do not begin installation until unacceptable conditions are corrected.

3.2	INSTALLATION

A.	Install seismic-certified riser bases in accordance with manufacturer’s instructions at locations indicated on the Drawings.

B.	Install seismic-certified riser bases plumb, level, square, and secure.

3.3	ADJUSTING

A.	Repair minor damages to finish in accordance with manufacturer’s instructions and as approved by Architect.

3.4	PROTECTION

A.	Protect installed seismic-certified riser bases from damage during construction.

END OF SECTION