Middle Atlantic Products, Inc.	January 2019

Phone	(973) 839-1011

Fax	(973) 839-1976

Website	middleatlantic.com

E-mail	info@middleatlantic.com

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, including MasterFormat, SectionFormat, and PageFormat, as described in The Project Resource Manual—CSI Manual of Practice.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” when editing this section.

Section numbers and titles are from MasterFormat 2004 Edition.

SECTION 27 11 16

TILT-OUT WALL RACKS

Specifier Notes:  This section covers Middle Atlantic Products, Inc. TOR Series Tilt-Out Wall Rack.  Consult Middle Atlantic Products for assistance in editing this section for the specific application.

PART 1	GENERAL

1.1	SECTION INCLUDES

A.	Tilt-out wall racks.

1.2	RELATED SECTIONS

Specifier Notes:  List sections with work directly related to this section.

A.

1.3	REFERENCES

Specifier Notes:  List standards referenced in this section, complete with designations and titles.  This article does not require compliance with standards, but is merely a listing of those used.

A.	EIA/TIA 310D – Cabinets, Racks, Panels and Associated Equipment.

B.	ISO 9001:2000 – Quality Management Systems – Requirements.

C.	UL – Underwriters Laboratories.

1.4	SUBMITTALS

A.	Comply with Section 01 33 00 – Submittal Procedures.

B.	Product Data:  Submit manufacturer’s product data, including options.

C.	Warranty:  Submit manufacturer’s standard warranty.

1.5	QUALITY ASSURANCE

- Manufacturer’s Qualifications:  ISO 9001:2000 registered company.

- Conform to ISO 14025 and EN 15804 or ISO 21930 in the development of life cycle assessments (LCA) and Type III environmental product declarations (EPD).

1.6	DELIVERY, STORAGE, AND HANDLING

A.	Delivery:  Deliver materials to site in manufacturer’s original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

B.	Storage:  Store materials in clean, dry area indoors in accordance with manufacturer’s instructions.

C.	Handling:  Protect materials and finish during handling and installation to prevent damage.

1.7	WARRANTY

A.	Warranty Period:  Product lifetime.

PART 2	PRODUCTS

2.1	MANUFACTURER

A.	Middle Atlantic Products, Inc.  Phone (973) 839-1011.  Fax (973) 839-1976.  Website middleatlantic.com.  E-mail info@middleatlantic.com.

2.2	TILT-OUT WALL RACKS

A.	Tilt-Out Wall Racks:  TOR Series Tilt-Out Wall Rack.

Specifier Notes:  Specify wall rack model number.  Consult Middle Atlantic Products product information for available models.

1.	Model:  TOR-__________ .

2.	Type:  19-inch panel width, tilt-out wall rack.

3.	Compliance:  EIA/TIA 310D.

4.	UL Listed:  US and Canada.

Specifier Notes:  Specify overall depth and racking height.

5.	Overall Dimensions:

a.	Width:

1)	Wall Mount:  24.5 inches (622 mm).

2)	Flush Mount and Fascia:  25.7 inches (635 mm).

b.	Height:  30.2 inches (767 mm).

c.	Depth:  _____ inches ( _____ mm).

6.	Racking Height:  _____ rackspaces.

Specifier Notes:  Specify flush or surface mount.

7.	Mounting:  [Flush mount]  [Surface mount].

8.	Weight Capacity:  100 pounds.

9.	Materials:

a.	Rack:  18-gauge steel.

b.	Ears and Fascia:  16-gauge steel.

c.	Finish:  Putty powder coat.

10.	Rackrail:

a.	11-gauge steel with tapped 10-32 mounting holes in universal EIA spacing.

b.	Finish:  Black e-coat.

c.	Rackspaces:  Marked.

11.	Front Rail:  Adjustable from 1-1/2 inches to 3-1/2 inches from top.

12.	Clamping Kit:  Secures non-rackmount equipment.

Specifier Notes:  Specify size of electrical box.

13.	Electrical Box:  [4 inches by 4 inches]  [4 inches by 2-1/8 inches].

14.	Removable Panels:  For universal connector panels.

15.	Knockouts:  2 Decora-style, at top face of rack.

16.	Cable Management:

a.	Cable Pass-Through on Backpan:  8.5 inches by 8.5 inches.

b.	Cable tie points.

c.	Electrical Conduit Knockouts:  Top and bottom.

17.	Pivot:

Specifier Notes:  Specify operating position.

a.	Operating Position:  _____ degrees, positive stop.

b.	Service Position:  105 degrees, positive stop.

18.	Keylock.

2.3	OPTIONS

Specifier Notes:  Specify required options.  Delete options not required.  Consult Middle Atlantic Products for more information.

A.	Partial Recess Trim Kit:  For mounting between 2 by 4 or 2 by 6 studs.

PART 3	EXECUTION

3.1	EXAMINATION

A.	Examine areas to receive tilt-out wall racks.

B.	Notify Architect of conditions that would adversely affect installation or subsequent use.

C.	Do not begin installation until unacceptable conditions are corrected.

3.2	INSTALLATION

A.	Install tilt-out wall racks in accordance with manufacturer’s instructions at locations indicated on the Drawings.

B.	Install tilt-out wall racks plumb, level, square, and secure.

3.3	ADJUSTING

A.	Adjust operating hardware to operate smoothly without binding.

B.	Repair minor damages to finish in accordance with manufacturer’s instructions and as approved by Architect.

3.4	PROTECTION

A.	Protect installed tilt-out wall racks from damage during construction.

END OF SECTION