Duet Module Interface Specification

for a

Middle Atlantic RackLink Premium

Power Management System

TABLE OF CONTENTS

Introduction	3

Overview	4

Implementation	5

Port Mapping	8

Channels	9

Command Control	11

Command Feedback	16

Important Notes	21

Programming Notes	22

Revision History

| ###### Date | Initials | Comments |
| --- | --- | --- |
| 10-19-2018 | DGG | v1.0.0 Initial Release |

## Introduction

This is a reference manual to describe the interface provided between an AMX NetLinx system and a Middle Atlantic RackLink Premium. The RackLink Premium supports an Ethernet connection only and utilizes REST communication.

This module was written using Café Duet firmware version v3.21.343. NetLinx Studio version 4 build 4.4.1626, Café Duet application platform and runtime version 1.8.190 and Café Duet application version 1.8.85.

The Comm. module must be informed of the IP address of the device it is to connect to, the username and password credentials for the device and the page size of the event log (if used) on the touchpanel. This must be done manually via a telnet session, or can be automated from the UI module. These properties must be set using the ‘PROPERTY-‘ command. The ‘REINIT’ command is used to notify the Comm. module that new properties are now available and to start using them. When these commands are used in the appropriate order, the Comm. module will attempt to connect to the specified IP address periodically until a connection is made or a new IP address is submitted using the ‘PROPERTY-‘ and ‘REINIT’ commands. Please see the Programming Notes section for additional information.

## Overview

The COMM module translates between the standard interface described below and the RackLink protocol. It parses the buffer for responses from the device, sends strings to control the device, and receives commands from the UI module or telnet sessions.

A User Interface (UI) module is also provided. This module uses the standard interface described below and parses the command responses for feedback.

The following diagram gives a graphical view of the interface between the interface code and the Duet module.

Middle Atlantic RackLink

Duet

COMM Module

NetLinx

UI Module

Virtual

Device

SNAPI

A sample UI module and a touch panel file are provided in the module package.  These are not intended to cover every possible application, but can be expanded as needed by a dealer to meet the requirements of a particular installation.

## Implementation

To interface to the AMX Middle Atlantic RackLink Premium module, the programmer must perform the following steps:

- Define the device ID for the RackLink Premium that will be controlled.

- Define the main virtual device ID that the device COMM module will use to communicate with the main program and User Interface. Duet virtual devices use device numbers 41000 - 42000.

- If a touch panel interface is desired, a touch panel file (Middle Atlantic RackLink Premium Demo v1_0.TP5) and module (Middle Atlantic RackLink Premium Demo v1_0.axs) have been created for testing.

- The Duet Middle_Atlantic_RackLink Premium_dr1_0_0 module must be included in the program with a DEFINE_MODULE command. This command starts execution of the module and passes in the following key information: the device ID of the RackLink Premium to be controlled, and the virtual device ID for communicating to the main program.

An example of how to do this is shown below.

DEFINE_DEVICE

/////////////////////////////////////////////////////////////

// PHYSICAL DEVICES

/////////////////////////////////////////////////////////////

dvRackLink	= 0:4:0;

/////////////////////////////////////////////////////////////

// VIRTUAL DEVICES

/////////////////////////////////////////////////////////////

// MAIN

vdvRackLink	= 41001:1:0;

// OUTLETS

vdvOutlet1		= 41001:1:0;

vdvOutlet2		= 41001:2:0;

vdvOutlet3		= 41001:3:0;

vdvOutlet4		= 41001:4:0;

vdvOutlet5		= 41001:5:0;

vdvOutlet6		= 41001:6:0;

vdvOutlet7		= 41001:7:0;

vdvOutlet8		= 41001:8:0;

vdvOutlet9		= 41001:9:0;

vdvOutlet10	= 41001:10:0;

vdvOutlet11	= 41001:11:0;

vdvOutlet12	= 41001:12:0;

vdvOutlet13	= 41001:13:0;

vdvOutlet14	= 41001:14:0;

vdvOutlet15	= 41001:15:0;

vdvOutlet16	= 41001:16:0;

// DRY CONTACTS

vdvDryContact1	= 41001:17:0;

vdvDryContact2	= 41001:18:0;

vdvDryContact3	= 41001:19:0;

vdvDryContact4	= 41001:20:0;

vdvDryContact5	= 41001:21:0;

vdvDryContact6	= 41001:22:0;

vdvDryContact7	= 41001:23:0;

vdvDryContact8	= 41001:24:0;

vdvDryContact9	= 41001:25:0;

vdvDryContact10	= 41001:26:0;

vdvDryContact11	= 41001:27:0;

vdvDryContact12	= 41001:28:0;

vdvDryContact13	= 41001:29:0;

vdvDryContact14	= 41001:30:0;

vdvDryContact15	= 41001:31:0;

vdvDryContact16	= 41001:32:0;

/////////////////////////////////////////////////////////////

// TOUCHPANELS

/////////////////////////////////////////////////////////////

dvPanel 		= 10001:1:0;

(***********************************************************)

(*               CONSTANT DEFINITIONS GO BELOW                              *)

(***********************************************************)

DEFINE_CONSTANT

/////////////////////////////////////////////////////////////

// DEVICE INFORMATION

/////////////////////////////////////////////////////////////

CHAR IP_ADDRESS		= '192.168.187.69';

CHAR USERNAME[] 	= 'admin';

CHAR PASSWORD[] 	= 'admin';

LOG_PAGE_SIZE		= 10;

(***********************************************************)

(*               VARIABLE DEFINITIONS GO BELOW            	         *)

(***********************************************************)

DEFINE_VARIABLE

VOLATILE DEV dvOutlets[] =

{

vdvOutlet1,

vdvOutlet2,

vdvOutlet3,

vdvOutlet4,

vdvOutlet5,

vdvOutlet6,

vdvOutlet7,

vdvOutlet8,

vdvOutlet9,

vdvOutlet10,

vdvOutlet11,

vdvOutlet12,

vdvOutlet13,

vdvOutlet14,

vdvOutlet15,

vdvOutlet16

}

VOLATILE DEV dvDryContacts[] =

{

vdvDryContact1,

vdvDryContact2,

vdvDryContact3,

vdvDryContact4,

vdvDryContact5,

vdvDryContact6,

vdvDryContact7,

vdvDryContact8,

vdvDryContact9,

vdvDryContact10,

vdvDryContact11,

vdvDryContact12,

vdvDryContact13,

vdvDryContact14,

vdvDryContact15,

vdvDryContact16

}

(***********************************************************)

(*                STARTUP CODE GOES BELOW                  	         *)

(***********************************************************)

DEFINE_START

/////////////////////////////////////////////////////////////

// DEFINE MODULES

/////////////////////////////////////////////////////////////

// COMM MODULE

DEFINE_MODULE 'Middle_Atlantic_RackLink_Premium_dr1_0_0' modCommRackLink(vdvRackLink, dvRackLink);

// UI MODULES

DEFINE_MODULE 'Middle Atlantic RackLink Premium UI v1_0' modUiRackLink(dvPanel, vdvRackLink, dvOutlets, dvDryContacts);

(***********************************************************)

(*                THE EVENTS GO BELOW                     		         *)

(***********************************************************)

DEFINE_EVENT

/////////////////////////////////////////////////////////////

// VIRTUAL DEVICE ONLINE

/////////////////////////////////////////////////////////////

DATA_EVENT[vdvRackLink]

{

ONLINE:

{

WAIT 50 'SETTLE DEVICE'

{

SEND_COMMAND vdvRackLink,"'DEBUG-1'";

SEND_COMMAND vdvRackLink,"'PROPERTY-IP_Address,', IP_ADDRESS";

SEND_COMMAND vdvRackLink,"'PROPERTY-Username,', USERNAME";

SEND_COMMAND vdvRackLink,"'PROPERTY-Password,', PASSWORD";

SEND_COMMAND vdvRackLink,"'PROPERTY-Log_Page_Size,', ITOA(LOG_PAGE_SIZE)";

SEND_COMMAND vdvRackLink,"'REINIT'";

}

}

}

Upon initialization the AMX Comm module will communicate with the RackLink Premium and information will be exchanged.

## Port Mapping

This module uses multiple virtual device ports in order distinguish events for one outlet/dry contact from another. The module supports a maximum of 16 outlets and 16 dry contacts, though your specific model may have or use less. Outlets are assigned ports 1 – 16 and dry contacts are assigned ports 17 – 32. All other functionality not specifically related to controlling outlets/dry contacts takes place on port 1.

| Virtual Device | Channels | Levels | Control | Feedback |
| --- | --- | --- | --- | --- |
| 41001:1:0 Main Functionality | 251, 252, 301 - 308, 311 -313, 321 -323, 331 - 346 | N/A | See Command Control List | See Command Feedback Lists |
| 41001:1:0 – 41001:16:0 Outlet Control | 9, 27, 28, 255 | N/A | See Command Control List | See Command Feedback Lists |
| 41001:17:0 – 41001:32:0 Dry Contacts Control | 9, 27, 28, 255 | N/A | See Command Control List | See Command Feedback Lists |

Table 1 - Port Mapping

## Channels

The UI module controls the RackLink Premium device via channel events (NetLinx commands pulse, on, and off) sent to the COMM module. The channels supported by the COMM module are listed below. These channels are associated with the virtual device(s) and are independent of the channels associated with the touch panel device. Not all channels will be available on all virtual ports.

Note: An ‘*’ indicates an extension to the standard API

| Channel | Description |
| --- | --- |
| 9 | PULSE: Cycle power for an outlet or dry contact (Ports 1 – 16 for outlets. Ports 17 – 32 for dry contacts) |
| 27 | PULSE: Turn on power for an outlet or dry contact (Ports 1 – 16 for outlets. Ports 17 – 32 for dry contacts) |
| 28 | PULSE: Turn off power for an outlet or dry contact (Ports 1 – 16 for outlets. Ports 17 – 32 for dry contacts) |
| 251 | ON: Device is Online – feedback only OFF: Device is not Online – feedback only (Port 1 only) |
| 252 | ON: Data is Initialized – feedback only OFF: Data is not Initialized – feedback only (Port 1 Only) |
| 255 | ON: Outlet or dry contact is on – feedback only OFF: Outlet or dry contact is off – feedback only (Ports 1 – 16 for outlets. Ports 17 – 32 for dry contacts) |
| *301 | PULSE: Select an outlet or dry contact (Ports 1 – 16 for outlets. Ports 17 – 32 for dry contacts) |
| *302 | PULSE: Deselect an outlet or dry contact (Ports 1 – 16 for outlets. Ports 17 – 32 for dry contacts) |
| *303 | PULSE: Cycle selection for an outlet or dry contact (Ports 1 – 16 for outlets. Ports 17 – 32 for dry contacts) |
| *304 | PULSE: Select all outlets (Port 1 Only) |
| *305 | PULSE: Deselect all outlets (Port 1 Only) |
| *306 | PULSE: Select all dry contacts (Port 1 Only) |
| *307 | PULSE: Deselect all dry contacts (Port 1 Only) |
| *308 | ON: Outlet or dry contacts is selected – feedback only OFF: Outlet or dry contacts is not selected – feedback only (Ports 1 – 16 for outlets. Ports 17 – 32 for dry contacts) |
| *311 | PULSE: Power cycle all currently selected outlets (Port 1 Only) |
| *312 | PULSE: Power on all currently selected outlets (Port 1 Only) |
| *313 | PULSE: Power off all currently selected outlets (Port 1 Only) |
| *321 | PULSE: Power cycle all currently selected dry contacts (Port 1 Only) |
| *322 | PULSE: Power on all currently selected dry contacts (Port 1 Only) |
| *323 | PULSE: Power off all currently selected dry contacts (Port 1 Only) |
| *331 | PULSE: Cycle polling for outlet state (Port 1 Only) |
| *332 | PULSE: Enable polling for outlet state (Port 1 Only) |
| *333 | PULSE: Disable polling for outlet state (Port 1 Only) |
| *334 | ON: Polling for outlet state is enabled – feedback only OFF: Polling for outlet state is disabled – feedback only (Port 1 only) |
| *335 | PULSE: Cycle polling for outlet information (Port 1 Only) |
| *336 | PULSE: Enable polling for outlet information (Port 1 Only) |
| *337 | PULSE: Disable polling for outlet information (Port 1 Only) |
| *338 | ON: Polling for outlet information is enabled – feedback only OFF: Polling for outlet information is disabled – feedback only (Port 1 only) |
| *339 | PULSE: Cycle polling for dry contact state (Port 1 Only) |
| *340 | PULSE: Enable polling for dry contact state (Port 1 Only) |
| *341 | PULSE: Disable polling for dry contact state (Port 1 Only) |
| *342 | ON: Polling for dry contact state is enabled – feedback only OFF: Polling for dry contact state is disabled – feedback only (Port 1 only) |
| *343 | PULSE: Cycle polling for PDU information (Port 1 Only) |
| *344 | PULSE: Enable polling for PDU information (Port 1 Only) |
| *345 | PULSE: Disable polling for PDU information (Port 1 Only) |
| *346 | ON: Polling for PDU information is enabled – feedback only OFF: Polling for PDU information is disabled – feedback only (Port 1 only) |

Table 2 - Virtual Device Channel Events

## Command Control

The UI module controls the RackLink Premium via command events (NetLinx command send_command) sent to the COMM module. The commands supported by the COMM module are listed below.

Note: An ‘*’ indicates an extension to the standard API.

| Command | Description |
| --- | --- |
| DEBUG-<value> | Set the state of debugging messages in the UI module and the Comm. module. Use the first virtual port when sending this command. Note: See Programming Notes section. <value> : 1 = set only error messages on 2 = set error and warning messages on 3 = set error, warning & debug messages on 4 = set all messages on DEBUG-1 |
| ?DEBUG | Request the state of the debug feature. Use the first virtual port when sending this command. ?DEBUG |
| ?DRY_CONTACT_LIST_INFO | Request the quantity of the dry contacts for the device. Use the first virtual port when sending this command. Example: ?DRY_CONTACT_LIST_INFO |
| ?DRY_CONTACT_LIST | Request the list of dry contacts for the device. Use the first virtual port when sending this command. Example: ?DRY_CONTACT_LIST |
| LOG_REFRESH | Request the latest event log from the device. Use the first virtual port when sending this command. Example: LOG_REFRESH |
| LOG_CLEAR | Clear the event log on the device. Use the first virtual port when sending this command. Example: LOG_CLEAR |
| LOG_PAGE_TOP | Request the first page of event log entries. Use the first virtual port when sending this command. Example: LOG_PAGE_TOP |
| LOG_PAGE_UP | Request the previous page of event log entries. Use the first virtual port when sending this command. Example: LOG_PAGE_UP |
| LOG_PAGE_DOWN | Request the next page of event log entries. Use the first virtual port when sending this command. Example: LOG_PAGE_DOWN |
| LOG_PAGE_BOTTOM | Request the last page of event log entries. Use the first virtual port when sending this command. Example: LOG_PAGE_BOTTOM |
| ?LOG_LIST-<page> | Request a particular page of event log entries. Use the first virtual port when sending this command. <page> : the desired page of event log entries. Example: ?LOG_LIST-5 // request page 5 from the event log. |
| ?OUTLET_LIST_INFO | Request the quantity of the outlets for the device. Use the first virtual port when sending this command. Example: ?OUTLET_LIST_INFO |
| ?OUTLET_LIST | Request the list of outlets for the device. Use the first virtual port when sending this command. Example: ?OUTLET_LIST |
| ?OUTLET_CURRENT_LOAD-<index> | Request the current load value for a particular outlet. Use the first virtual port when sending this command. <index>: the outlet number Example: ?OUTLET_CURRENT_LOAD |
| ?OUTLET_CURRENT_POWER_FACTOR-<index> | Request the current power factor value for a particular outlet. Use the first virtual port when sending this command. <index>: the outlet number Example: ?OUTLET_CURRENT_POWER_FACTOR |
| ?OUTLET_CURRENT_VOLTAGE-<index> | Request the current voltage value for a particular outlet. Use the first virtual port when sending this command. <index>: the outlet number Example: ?OUTLET_CURRENT_VOLTAGE |
| ?OUTLET_CURRENT_WATTAGE-<index> | Request the current wattage value for a particular outlet. Use the first virtual port when sending this command. <index>: the outlet number Example: ?OUTLET_CURRENT_WATTAGE |
| ?PDU_CURRENT_LOAD | Request the PDU current load value. Use the first virtual port when sending this command. Example: ?PDU_CURRENT_LOAD |
| ?PDU_CURRENT_POWER_FACTOR | Request the PDU current power factor value. Use the first virtual port when sending this command. Example: ?PDU_CURRENT_POWER_FACTOR |
| ?PDU_CURRENT_VOLTAGE | Request the PDU current voltage value. Use the first virtual port when sending this command. Example: ?PDU_CURRENT_VOLTAGE |
| ?PDU_CURRENT_WATTAGE | Request the PDU current wattage value. Use the first virtual port when sending this command. Example: ?PDU_CURRENT_WATTAGE |
| ?PDU_INFO_IP_ADDRESS | Request the IP Address of the PDU. Use the first virtual port when sending this command. Example: ?PDU_INFO_IP_ADDRESS |
| ?PDU_INFO_MAC_ADDRESS | Request the MAC Address of the PDU. Use the first virtual port when sending this command. Example: ?PDU_INFO_MAC_ADDRESS |
| ?PDU_INFO_MODEL | Request the Model of the PDU. Use the first virtual port when sending this command. Example: ?PDU_INFO_MODEL |
| ?PDU_INFO_RATING_CURRENT | Request the current rating of the PDU. Use the first virtual port when sending this command. Example: ?PDU_INFO_RATING_CURRENT |
| ?PDU_INFO_RATING_FREQUENCY | Request the frequency rating of the PDU. Use the first virtual port when sending this command. Example: ?PDU_INFO_RATING_FREQUENCY |
| ?PDU_INFO_RATING_POWER | Request the power rating of the PDU. Use the first virtual port when sending this command. Example: ?PDU_INFO_RATING_POWER |
| ?PDU_INFO_RATING_VOLTAGE | Request the voltage rating of the PDU. Use the first virtual port when sending this command. Example: ?PDU_INFO_RATING_VOLTAGE |
| ?PDU_TEMP_CURRENT | Request the current temperature of the PDU. Use the first virtual port when sending this command. Example: ?PDU_TEMP_CURRENT |
| ?PDU_TEMP_THRESHOLD_MAX | Request the maximum temperature threshold of the PDU. Use the first virtual port when sending this command. Example: ?PDU_TEMP_THRESHOLD_MAX |
| ?PDU_TEMP_THRESHOLD_MIN | Request the minimum temperature threshold of the PDU. Use the first virtual port when sending this command. Example: ?PDU_TEMP_THRESHOLD_MIN |
| ?PDU_THRESHOLD_LOAD_MAX | Request the maximum load threshold of the PDU. Use the first virtual port when sending this command. Example: ?PDU_THRESHOLD_LOAD_MAX |
| ?PDU_THRESHOLD_LOAD_MIN | Request the minimum load threshold of the PDU. Use the first virtual port when sending this command. Example: ?PDU_THRESHOLD_LOAD_MIN |
| ?PDU_THRESHOLD_VOLTAGE_MAX | Request the maximum voltage threshold of the PDU. Use the first virtual port when sending this command. Example: ?PDU_THRESHOLD_VOLTAGE_MAX |
| ?PDU_THRESHOLD_VOLTAGE_MIN | Request the minimum voltage threshold of the PDU. Use the first virtual port when sending this command. Example: ?PDU_THRESHOLD_VOLTAGE_MIN |
| PROPERTY-<key>,<value> | Set the value of property <key> to <value>. This must be followed by the REINIT command to take effect. These values are not initialized by default. Use the first virtual port when sending this command. <key> : IP_Address <value> : String = representing an IP address to use for connecting to the device <key> : Username <value> : String = representing the username to use for connecting to the device <key> : Password <value> : String = representing the password to use for connecting to the device <key> : Log_Page_Size <value> : String = representing the number of event log entries per page to return with each query (minimum size is 1. maximum size is 10) Example: PROPERTY-IP_Address,10.0.0.5 PROPERTY-Username,admin PROPERTY-Password,admin PROPERTY-Log_Page_Size,10 |
| ?PROPERTY-<key> | Get the value of a property <key>. If the value is not initialized, an empty string is returned. Use the first virtual port when sending this command. <key> : IP_Address Username Password Log_Page_Size Example: ?PROPERTY-IP_Address ?PROPERTY-Username ?PROPERTY-Password ?PROPERTY-Log_Page_Size |
| REINIT | Re-initializes the communication link and data. Note: This command deletes any messages waiting to go out to the device. Use the first virtual port when sending this command. REINIT |
| ?VERSION | Query for the current version number of the Duet module. Use the first virtual port when sending this command. ?VERSION |

Table 3 – Send Command Definitions

## Command Feedback

The COMM module provides feedback to the User Interface module for audio processors changes via command events. The commands supported are listed below.

PLEASE NOTE: Feedback is only provided when there is a state change. If no state change resulted from the command sent in, then no feedback will be returned.

| Command |  | Description |
| --- | --- | --- |
| DEBUG-<value> |  | Reports the state of debugging messages in the UI module and the Comm. module. This is reported on the first virtual port. <value> : 1 = set only error messages on 2 = set error and warning messages on 3 = set error, warning and debug messages on 4 = set all messages on DEBUG-1 |
| DRY_CONTACT_LIST_INFO-<listCount> |  | Reports the number of dry contacts in the list. This is reported on the first virtual port. listCount: the number of list entries. Example: DRY_CONTACT_LIST_INFO-5 // indicates there are 5 dry contacts connected to the device. |
| DRY_CONTACT_LIST-<listIndex>,itemName> |  | Returns information about a particular dry contact entry. This is reported on the first virtual port. listIndex: the index of the dry contact in the returned list. itemName: the name of the dry contact. Example: DRY_CONTACT_LIST-1,Dry Contact 1 // the first dry contact is being returned and it has a name of “Dry Contact 1” |
| LOG_LIST_INFO-<totalEntries>,<entriesPerPage>,<returnedEntries>,<currentPage>,<totalPages> |  | Returned before the list of log entries for the device in order to describe pertinent information about the entries that will be following this response. This is reported on the first virtual port. totalEntries: the total number of event log entries. entriesPerPage: the number of entries per page that will be returned returnedEntries: the total number of entries that will be returned from this query currentPage: the current page of the event log totalPages: the total pages in the event log Example: LOG_LIST_INFO-500,10,10,1,50 // indicates that there are 500 total entries in the log. You are about to receive 10 of these entries which fall on page 1 of 50 total pages. |
| LOG_LIST-<listIndex>,<absoluteIndex>,<id>,<date>,<time>,<category>,<message> |  | Returns information about a particular event log entry. This is reported on the first virtual port. listIndex: the index of the entry in the returned list. absoluteIndex: the absolute index of the entry in the entire list of entries. id: the id of the entry date: the date of the entry time: the time of the entry category: the category of the entry message: the entry message Example: LOG_LIST-1,50,50,10-10-2018,12:00 PM,Sensor,Sensor 1 was turned on // you are receiving the first entry in the returned list. It is entry number 50 out of all the entries in the entire list. The remaining information describes the details of the entry itself. |
| OUTLET_LIST_INFO-<listCount> | Reports the number of outlets in the list. This is reported on the first virtual port. listCount: the number of list entries. Example: OUTLET_LIST_INFO-8 // indicates there are 8 outlets on the device. |  |
| OUTLET_LIST-<listIndex>,itemName> | Returns information about a particular outlet entry. This is reported on the first virtual port. listIndex: the index of the outlet in the returned list. itemName: the name of the outlet. Example: OUTLET_LIST-1,Outlet 1 // the first outlet is being returned and it has a name of “Outlet 1” |  |
| OUTLET_CURRENT_LOAD-<index> | Returns the current load value for a particular outlet. This is reported on the first virtual port. <index> : the outlet number Example: OUTLET_CURRENT_LOAD-1,0.00 // outlet 1 |  |
| OUTLET_CURRENT_POWER_FACTOR-<index> | Returns the current power factor value for a particular outlet. This is reported on the first virtual port. <index> : the outlet number Example: OUTLET_CURRENT_POWER_FACTOR-1,1.00 // outlet 1 |  |
| OUTLET_CURRENT_VOLTAGE-<index> | Returns the current voltage value for a particular outlet. This is reported on the first virtual port. <index> : the outlet number Example: OUTLET_CURRENT_VOLTAGE-1,119.70 // outlet 1 |  |
| OUTLET_CURRENT_WATTAGE-<index> | Returns the current wattage value for a particular outlet. This is reported on the first virtual port. <index> : the outlet number Example: OUTLET_CURRENT_WATTAGE-1,0.00 // outlet 1 |  |
| PDU_CURRENT_LOAD | Returns the PDU current load value. This is reported on the first virtual port. Example: PDU_CURRENT_LOAD-0.00 |  |
| PDU_CURRENT_POWER_FACTOR | Returns the PDU current power factor value. This is reported on the first virtual port. Example: PDU_CURRENT_POWER_FACTOR-1.00 |  |
| PDU_CURRENT_VOLTAGE | Returns the PDU current voltage value. This is reported on the first virtual port. Example: PDU_CURRENT_VOLTAGE-119.70 |  |
| PDU_CURRENT_WATTAGE | Returns the PDU current wattage value. This is reported on the first virtual port. Example: PDU_CURRENT_WATTAGE-0.00 |  |
| PDU_INFO_IP_ADDRESS | Returns the IP Address of the PDU. This is reported on the first virtual port. Example: PDU_INFO_IP_ADDRESS-192.168.1.100 |  |
| PDU_INFO_MAC_ADDRESS | Returns the MAC Address of the PDU. This is reported on the first virtual port. Example: PDU_INFO_MAC_ADDRESS-02:9b:a4:a4:5c:45 |  |
| PDU_INFO_MODEL | Returns the Model of the PDU. This is reported on the first virtual port. Example: PDU_INFO_MODEL-RLNK-P915R |  |
| PDU_INFO_RATING_CURRENT | Returns the current rating of the PDU. This is reported on the first virtual port. Example: PDU_INFO_RATING_CURRENT-12A |  |
| PDU_INFO_RATING_FREQUENCY | Returns the frequency rating of the PDU. This is reported on the first virtual port. Example: PDU_INFO_RATING_FREQUENCY-60Hz |  |
| PDU_INFO_RATING_POWER | Returns the power rating of the PDU. This is reported on the first virtual port. Example: PDU_INFO_RATING_POWER-1.2-1.4kVA |  |
| PDU_INFO_RATING_VOLTAGE | Returns the voltage rating of the PDU. This is reported on the first virtual port. Example: PDU_INFO_RATING_VOLTAGE-100-150V |  |
| PDU_TEMP_CURRENT | Returns the current temperature of the PDU. This is reported on the first virtual port. Example: PDU_TEMP_CURRENT-26.68 |  |
| PDU_TEMP_THRESHOLD_MAX | Returns the maximum temperature threshold of the PDU. This is reported on the first virtual port. Example: PDU_TEMP_THRESHOLD_MAX-35.00 |  |
| PDU_TEMP_THRESHOLD_MIN | Returns the minimum temperature threshold of the PDU. This is reported on the first virtual port. Example: PDU_TEMP_THRESHOLD_MIN-10.00 |  |
| PDU_THRESHOLD_LOAD_MAX | Returns the maximum load threshold of the PDU. This is reported on the first virtual port. Example: PDU_THRESHOLD_LOAD_MAX-9.60 |  |
| PDU_THRESHOLD_LOAD_MIN | Returns the minimum load threshold of the PDU. This is reported on the first virtual port. Example: PDU_THRESHOLD_LOAD_MIN-0.00 |  |
| PDU_THRESHOLD_VOLTAGE_MAX | Returns the maximum voltage threshold of the PDU. This is reported on the first virtual port. Example: PDU_THRESHOLD_VOLTAGE_MAX-159.00 |  |
| PDU_THRESHOLD_VOLTAGE_MIN | Returns the minimum voltage threshold of the PDU. This is reported on the first virtual port. Example: PDU_THRESHOLD_VOLTAGE_MIN-112.00 |  |
| PROPERTY-<key>,<value> |  | Reports the value of property <key>. This is reported on the first virtual port. <key> : IP_Address <value> : String = representing an IP address to use for connecting to the device <key> : Username <value> : String = representing the username to use for connecting to the device <key> : Password <value> : String = representing the password to use for connecting to the device <key> : Log_Page_Size <value> : String = representing the number of event log entries per page to return with each query (minimum size is 1. maximum size is 10) Example: PROPERTY-IP_Address,10.0.0.5 PROPERTY-Username,admin PROPERTY-Password,admin PROPERTY-Log_Page_Size,10 |
| VERSION-<value> |  | Reports the current version number of the Duet module. This is reported on the first virtual port. <value> : current version number in xx.yy.zz format VERSION-1.0.0 |

Table 4 - Command Feedback Definitions

## Important Notes

- Different models exist within the RackLink Premium family. This module will control any of them provided they adhere to the REST API. The module supports a model with a maximum of 16 outlets and 16 dry contacts. Dry contacts are available via external/add-on sensors.

- Temperature reading is done via an external sensor. Multiple sensors can potentially be attached to the PDU, however the module will only read from the first temperature sensor it finds.

- Multiple polling channels have been included in the module to allow for querying different aspects of the PDU.  Some polls, such as outlet information, may contain multiple different queries for each outlet. Depending on the number of outlets, a single poll cycle may contain dozens to over a hundred individual queries to the device and, as such, can be very data and time intensive.  It is HIGHLY recommended to only enable the polls that are required for your particular program and to note that, depending on which polls are enabled, feedback from the poll may take some time to query everything and update all signals. As an example, a PDU with 8 outlets and 2 dry contacts was tested during development. With all polls enabled, each cycle took over a minute to complete and refresh. If you are working with a device that has the maximum 16 outlets and 16 dry contacts and have all polls enabled, expect this time to be significantly higher.

- The polling rate (i.e. the time between the last query in the poll cycle and the first query in the next poll cycle) is always 15 seconds.

- The module will automatically initialize when the program starts up (or manually when the Reinit command is sent). Initialization consists of querying the device for all status information required for proper module operation. Depending on the number of outlets, dry contacts and log entries on the device, this process can be quite time consuming so it is normal for the initialization process to take several minutes.

## Programming Notes

PLEASE NOTE: In order to establish a communication connection between the module and the device, the module must be informed of the IP address of the device it is to connect to, the username and password credentials for the device and the page size of the event log (if used) on the touchpanel. Use the PROPERTY- command to set this information and then use the REINIT command to force it to take affect. Here is an example of how these commands are used:

Send_Command 41001:1:0, ‘PROPERTY-IP_Address,192.168.1.100’

Send_Command 41001:1:0, ‘PROPERTY-Username,admin’

Send_Command 41001:1:0, ‘PROPERTY-Password,admin’

Send_Command 41001:1:0, ‘PROPERTY-Log_Page_Size,10’

Send_Command 41001:1:0, ‘REINIT’

Note that your virtual device (41001:1:0) and the IP address, credentials and log size may differ from this example. Substitute the appropriate values where necessary.