CHAPTER 5 CASE STUDY:

Walls below ground and basement construction

- Case study page 146

Here we see examples of contiguous and secant piling used to form basement walls. Note the concrete capping/ring beam which connects the piles and provides a sound base upon which to form the superstructure walls.

In this case we also see pre-cast, reinforced concrete sections used in conjunction with the secant piles.

The photos above show a masonry diaphragm wall being used as a retaining wall between differing ground levels. Note the connecting fins between the inner and outer skin of the wall. The resulting voids are filled with concrete to ensure sufficient strength.