CHAPTER 5	Walls Below Ground And Basement Construction

- 5.1 Requirements of walls below ground

- Review Task 1, p126

With reference to the grades of basement with which you are now familiar, identify a range of basement uses appropriate to each grade.

The following table may be useful in reflecting on your own answers to this:

| Grade | Possible use |
| --- | --- |
| Grade 1: Basic Utility | Car parking Mechanical plant rooms Non-specific void space Transit routes for delivery vehicles, etc. |
| Grade 2: Better Utility | Retail storage Electrical plant rooms Archive of non-delicate materials |
| Grade 3: Habitable | Offices Residential use Kitchens, restaurants, etc. |
| Grade 4: Special | Archive storage of books, documents, art, etc. |

5.2 Options for waterproofing of basements

- Review Task 2, p132

List the advantages and disadvantages of the various types of basement described in this section and attempt to identify where each may be most appropriate.

The various types of basement lend themselves to different uses and each has advantages and disadvantages. Taking each in turn:

| Option | Advantages | Disadvantages | Appropriate use |
| --- | --- | --- | --- |
| Type A: Tanked basements | Familiar technology; access to internally tanked walls allows remedial treatments | Externally tanked structures are difficult to repair; internally tanked structures are subject to hydrostatic pressure forcing membranes off walls | Perhaps the most common option of small scale basements with traditional wall construction; used in small commercial and residential projects |
| Type B: Waterproof structures | Single element waterproofing and structure; extent of waterproofing detailing is reduced | Need to incorporate water-bars, etc., in structural elements | Very common for larger scale basement structures and deep basement forms; probably the most common form of basement in larger commercial projects |
| Type C: Drained cavities | Simplicity of construction | Need to cater for removal of water; possible problems with soil fines and silting of pumps, etc. | Used where basement conditions do not need to be totally waterproof such as car parks and non-sensitive basement areas |

- 5.3	Methods of basement construction

- Review Task 3, p147

List the advantages and disadvantages of each of the options available for water-proofing basements.

| Category | Description | Details | Advantages | Disadvantages |
| --- | --- | --- | --- | --- |
| 1 | Bonded sheet membranes | Bonded, hot or cold, to structural walls externally or internally | Simple familiar construction; relatively cheap | Need to form joints between sheets; detailing can be complex |
| 2 | Cavity drain membranes | High density ridged or pimpled plastic sheeting, generally placed internally to allow draining of moisture passing through walls | Simple in form with many available products; cheap and simple to install | Need to drain moisture away from points of accumulation |
| 3 | Bentonite clay active membranes | Sandwich of bentonite clay between layers of cardboard; used externally; the clay swells to fill gaps when in contact with water | Detailing and installation; simple in detailing | Detailing and installation can be laborious; really intended for large scale projects |
| 4 | Liquid applied membranes | Epoxy, polymer or bitumen solutions applied cold to walls internally or externally; these cope well with unusual profiles and variable shapes | Cheap and simple to apply; deal easily with penetrations through basement walls | Questions about uniformity of application; require clean, dry sub-base for effective application |
| 5 | Mastic asphalt membranes | Applied hot as multi-layer application internally or externally | Simple to apply with correct skills; deal easily with penetrations through basement walls | Increasing scarcity of skills; hot process has health and safety issues |
| 6 | Cementitious renders and toppings | Generally used internally and applied as multi-layer coatings incorporating waterproof add-mixtures; these cope well with variable surfaces and are sometimes used as remedial treatments for existing basements | Simple to apply with correct skills; deal easily with penetrations through basement walls; some can be applied to damp walls effectively | Increasing scarcity of skills; questions about uniformity of application |