Understanding Structures, 5th edition

Derek Seward

Solutions to end-of-chapter exercises

Contents

Chapter 1 - Design	2

Chapter 2 - Basics	9

Chapter 3 - Materials	12

Chapter 4 - Loads	16

Chapter 5 – Pin-jointed trusses	18

Chapter 6 - Tension	20

Chapter 7 - Beams	24

Chapter 8 - Compression	30

Chapter 9 – Combined axial and bending stresses	32

Chapter 10 - Torsion	36

Chapter 11 - Connections	38

Chapter 12 – Arches and portal frames	41

Chapter 13 – Foundations and retaining walls	45

Chapter 14 – Deflection	50

Chapter 15 – Indeterminate structures and computers	53

# Chapter 1 – Design

E1.1

- A tree trunk acts as a vertical cantilever beam to resist wind force. Roots transfer the load into the ground

- An apple stalk is a tension member transferring the weight of the apple back to the tree. Vines and trailing plants are also tension structures

- Creatures such as crabs have exo-skeletons. This is a “shell” structure which provides the strength and shape on the outside of the flesh.

Step ladders are an example of a structure to transfer the weight of a person down to the ground. The feet are prevented from moving apart partly by the tensile rope and partly by friction on the ground.

A bed is a similar structure to a bridge and consists of a beam spanning between compressive columns.

The shelf bracket transfers the load back to the wall with the help of screws. The diagonal member is a compressive ‘strut’ and reduces bending in the bracket.

E1.2

a1. Steel lattice tower with tension guys.

a2. Concrete shell with restaurant.

b1. Shell dome with flying buttresses.

b2. Cable-supported ‘tent’ roof (cross-section)

E1.3

Stage 1 – site survey

Determine the most suitable point to cross the stream taking into account:

- Stream width

- Height of banks

Foundation material (any mid-stream rocks for extra support?)

- Location of access roads.

Carry out a detailed survey and produce a cross-section of the chosen point – drawn to scale:

Stage 2 – alternative concepts

Comments:

- Best appearance and long life, but expensive and difficult,

- Reasonably good appearance, fairly cheap and easy but possibly short life,

- Robust and easy but requires maintenance for long life,

- Least attractive but cheap and easy.

Choose say b.

Stage 3 – detailed development

Stage 4 - Assessment of loads

This can be tricky and very much depends upon who has access to the bridge. Is it accessible by the public and emergency vehicles? If this is the case the bridge will need to be designed for loads specified in Eurocode 1-2. If the bridge merely provides a link between two fields and will only be used by the farmer for movement of animals and a tractor and trailer the load requirements may be more relaxed.  The local building regulation authority should be contacted for advice.

Assuming use by only the farmer:

Animals: Design for say the number of cows that can fit on the bridge x weight of one cow. (about 1000 kg each?). Side load, P, on the handrail might be 20% of cow weight?

Vehicles: The highest axle loads should be placed at various points in the span.

Stage 5 Analysis

Structural model:

Stage 6 – detail design

possible failure modes:

E1.4

- Permissible stress design

Total load in rod	=	12 + 16		=	28 kN

Area required	=	force/stress	=	28 x 103/150

=	187 mm2

- Limit state design

Design permanent load	=	12 x 1.35 	=	16.2 kN

Design variable load	=	16 x 1.5		=	24.0 kN

Total design load					=	40.2 kN

Design strength		=	fy/M		=	275/1.0		=	275 N/mm2

Area required		=	40.2 x 103/275	=	146 mm2

# Chapter 2 - Basics

E2.1

- Force from 2.0 kg weight	=	2.0 x 9.81	=	19.6 N

Force from 3.5 kg weight	=	3.5 x 9.81	=	34.3 N

34.3 N

19.6 N

30°

Structural model:

- Horizontal component of force	=	34.3 cos 30°	=	29.7 N

Vertical component of force	=     19.6 + 34.3 sin 30°	=	36.8 N

- Magnitude of resultant force	=      (29.72 + 36.82)	=	47.3 N

Direction of resultant force	=      tan-1(36.8/29.7)	=	51.1°

47.3 N

51.1°

- Stress in cable			=	Force/Area	=	34.3/ (π x 1.52/4)

=	19.4 N/mm2

E2.2

C

FCH

30°

20 kN

FCV

- FCH	=	20 Cos 30°	=	17.32 kN

FCH	=	20 Sin 30°	=	10.0 kN

17.32

RAH

10.0

RBH

RAV

RBV

Support B is a roller  RBH = 0

M about A

(17.32 x 0.75) + (10 x 3.5)	=	RBV x 2.0

RBV	=	24.0 kN

H = 0 					RAH	=	17.32 kN

V = 0 				10.0 + RAV	=	24.0 kN

RAV	=	14.0 kN

E2.3

No horizontal forces or reactions

- M about A

10 x 3	=	5 x RA

RA	=	6 kN

V = 0

10	=	6 + RC

RC	=	4 kN   i.e. tension in rod

10.0

C

RC

B

A

structural model:

RA

- Area of rod	=	π x 102/4	=	78.5 mm2

Stress		=	force/area	=	4 x 103/78.5	=	51 N/mm2

- Extension, 	=	FL/EA		where E  =  205 kN/mm2

=	4 x 103 x 3000/205 000 x 78.5		=	0.75 mm

# Chapter 3 - Materials

E3.1

- Underground water storage reservoir

Choice: Reinforced concrete

Reasons:

- Will resist bending from water pressure

- Can be made waterproof with additives

- Durable and requires no maintenance if cover to reinforcement adequate

- Relatively cheap

- Can be formed on-site from locally available materials

- Footbridge in chemical plant

Choice: Fibre composite

Reasons:

- Resistant to chemical attack

- No maintenance required

- Lightweight and easily transported

- Portable grandstand

Choice: Aluminium alloy

Reasons:

- Lightweight and easily transported

- Good range of extrusions available

- Can be welded

- Can be painted or anodised for good corrosion protection

- Stiffer than fibre composite

E 3.2

- Yield stress, fy, and Modulus of elasticity, E

- Gauge length is the initial length of the specimen for the purposes of measuring extension and hence strain and modulus of elasticity.

- Plastic behaviour means that materials are ductile. ( i.e. they can be deformed by large amounts without losing their strength. The opposite of brittle.

- Because by the time mild steel has reached its ultimate strength it will have deformed excessively. Keeping below the yield strength means that permanent deformations do not take place.

- Yield point (i.e. start of plastic behaviour), limit of proportionality (i.e. end of linear behaviour where strain is proportional to stress) and elastic limit (i.e . end of reversible deformation).

E3.3

Area of specimen	=	π/4 x 52		=	19.63 mm2

Stress			=	load/area

% Strain		=	100 x extension/gauge length

| Load (kN) | Extension (mm) | Stress (N/mm2) | % Strain |
| --- | --- | --- | --- |
| 0.00 | 0.00 | 0 | 0.000 |
| 1.00 | 0.01 | 51 | 0.025 |
| 2.00 | 0.02 | 102 | 0.050 |
| 3.00 | 0.03 | 153 | 0.075 |
| 4.00 | 0.04 | 204 | 0.100 |
| 5.00 | 0.05 | 255 | 0.125 |
| 5.32 | 0.06 | 271 | 0.150 |
| 5.38 | 0.07 | 274 | 0.175 |
| 5.34 | 0.08 | 272 | 0.200 |

2733

204

From graph:

- Yield stress, fy		=	274 N/mm2

- Modulus of elasticity, E	=	204/0.001	=	204 000 N/mm2

E3.4

| Result (x) | (x-xm) | (x-xm)2 |
| --- | --- | --- |
| 455 | -4 | 16 |
| 467 | 8 | 64 |
| 449 | -10 | 100 |
| 452 | -7 | 49 |
| 471 | 12 | 144 |
| 462 | 3 | 9 |
| 2756 |  | 382 |

Mean x m	=	2756/6		=	459

Standard deviation	=	[(x-xm)2/(n-1)]	=	382/5

=	8.74

Characteristic strength	=	459 – 1.59 x 8.74

=	445 N/mm2

Design strength		=	Characteristic strength/M

=	445/1.15	=	387 N/mm2

# Chapter 4 - Loads

E4.1

From table 4.1 unit weight of steel	=	70 kN/m3

Weight of of 1 square meter of plate 6 mm thick	=	70 x 0.006	=	0.42 kN/m2

- Characteristic permanent load		=	0.42 kN/m2

- From table 4.3 variable floor load for factory	=	5.0 kN/m2

Total design load	=	(1.35 x 0.42) + (1.5 x 5.0)

=	8.07 kN/m2

E4.2

Resultant occurs at centroid of triangle i.e. 1/3rd of height.

- Force		=	38.25 x 4.5/2	=	86.1 kNm

- Bending moment	=	86.1 x 1.5	=	129.2 kNm

E4.3

Area of each floor	=	60 x 60		=	3600 m2

From table 4.1      Wt. of floor/m2	=	25 x 0.2		=	5 kN/m2

Wt. of one entire floor		=	3600 x 5	=	18 000 kN

Wall area per storey		=	4 x 4  x 60	=	960 m2

Weight of wall per storey	=	960 x 1.0	=	960 kN

Total permanent load/storey	=	18 000 + 960	=	18 960 kN

From table 4.3	Variable load per floor	=	3600 x 2.5	=	9000 kN

Total design load	=	110 x [(1.35 x 18 960) + ( 1.5 x 9000)]

=	4.30 x 106 kN

E4.4

The following is for a 1 m wide strip of the floor (into the page):

From table 4.1  	Permanent load of wall	=	22 x 0.215 x 1.0		=	4.73 kN/m

Design permanent wall load	=	4.73 x 1.35		=	5.81 kN/m

Permanent load of slab	=	25 x 0.3 		=	7.5 kN/m

Design permanent slab load	=	1.35 x 7.5		=	10.1 kN/m

From table 4.3	Variable load on slab	=	2.5 kN/m

Design variable load	=	1.5 x 2.5		=	3.75 kN/m

Total slab design load	=	10.1 + 3.75		=	13.85 kN/m

5.81 kN

13.85 kN/m kN

Structural model

1.8 m

8.0 m

B

A

Load on beam is reaction at A - take moments about B:

RA x 8.0		=	(5.81 x 9.8) + (13.85 x 9.8 x 9.8/2)

RA	=	90.25 kN/m

Design self-weight of beam	=	140 x 9.81/103 x 1.35	=	1.9 kN/m

Total design load on beam	=	90.25 + 1.9		=	92.15 kN/m

# Chapter 5 – Pin-jointed trusses

E5.1

E5.2

cut

E5.3

Consider only the left-hand-side of the cut:

M about E for FBC:

FBC x yE	=	RA x 8

FBC x 4.85	=	260 x 8

FBC	=	429 kN compression

M about x for FCE:

(180 x 20)	=	(260 x 12) + (FCE x 16.64)

FCE	=	28.8 kN tension

E5.4

- m = 13, j  = 6   i(1)

- m = 11, j  = 5 + reduced roller support, but formula doesn’t work – actually m + i(1)

- m = 16, j  = 8  extra roller  i(1)

- m = 8, j  = 4   s

- m = 11, j  = 5   i(1)

# Chapter 6 - Tension

E6.1

From text:	fy   =   275 N/mm2, fu   =   410 N/mm2

M0   =   1.0, M2   =   1.1

222

5

Option a)

w

Min. Agross	=	150×1031.0	=	545 mm2

Anet	=	150×1030.9×4101.1	=	447 mm2

W	=	447/5  +  22	=	111.4   say   115 mm

Agross	=	115 x 5		=	575 mm2 > 545 mm2

Option b)

Agross	=		545 mm2 as above

From appendix		use 60 x 60 x 5 Angle     (A = 582 mm2)

E6.2

From symmetry consider only half of the structure:

F2

3

2

F1

2 kN

					=	tan-1(2/3)	=	33.7°

From vertical equilibrium:

vertical component of force F2  = 2 kN

F2 Sin33.7°		=	2.0

Force in cable at support, F2	=	3.60 kN

From horizontal equilibrium:

Force in cable at midspan, F1	=	3.60Cos 33.7°	=	3.00 kN

Area of cable required	=	3600/(1200/5)	=	15 mm2

Diameter	=	(15x4/π)	=	4.4 mm

Use 5mm diameter cable

E6.3

Total design load	=	(1.35 x 6) + (1.5 x 5 x 2)         [where 2 is the width)

RV

=	23.1 kN/m

Diagram identical for both main span and side-spans

RH

15.0

RH

23.1 kN/m

32.0

[6.4]		RV	=	wL/2	=	23.1 x 64/2	=	739 kN

[6.5]		RH	=        wL2/8D	=	(23.1 x 642)/(8 x 15)	=	788 kN

a.   	Max cable force at support	=	0.5 x (7392 + 7882)	=	540 kN

[0.5 above is because of two cables]

b.	Compressive force in tower	=	vert. component in cables at each side

=	2 x 739			=	1478 kN

c.	Area of cable required		=	Force/(strength/M)	=	540 x 103/(1590/3)					=	1019 mm2

Diameter	=	[(4 x 1019)/π]		=	36.02 mm

say 40 mm

d.Horizontal force on cable anchor	=	RH/2	=	788/2	=	394 kN

E6.4

Design variable load from one light	=	2.5 x 1.5 x 9.81	=	368 N

M about A

(368 x 1.3) + (368 x 4.7)	=	(368 x 0.7) + (RB x 4.4)

RB	=	443 N

V  =  0

RA	=	(3 x 368) – 443

=	661 N

Area of wire required		=	661/(300/5)	=	11.01 mm2

Diameter of wirte		=       [(4 x 11.01)/π]	=	3.75  say 4.0 mm

# Chapter 7 - Beams

E7.1

E7.2

Bending moment	=	wl2/8	=	22 x 8.02 /8	=	176 kNm

fy	=	275,			M	=	1.0

required Wpl		=	(176 x 106)/(275 x 103)		=	640 cm3

Use 406 x 140 x 39 kg/m Universal Beam

E7.3

Total area	=	9 + 6 + 6

=	21cm2

Half area	=	21/2

=	10.5 cm2

For the plastic modulus the neutral axis divides the total area into two equal parts.

Y	=	1.0 + (10.5 – 6)/0.8	=	6.625 cm

| Item | A (cm2) | Y (cm) | Ay (cm3) |
| --- | --- | --- | --- |
| 1 | 9 | 3.375 | 30.3 |
| 2 | 1.5 | 0.938 | 1.4 |
| 3 | 4.5 | 2.813 | 12.7 |
| 4 | 6 | 6.125 | 36.8 |
|  |  |  | 81.3 |

Plastic modulus, Wpl  =   81.3 cm3

Design load	=	(1.35 x 0.9) + (1.5 x 3.5)	=	6.47 kN/m2

Design load/m, w	=	6.47 x 0.35		=	2.26 kN/m

Design bending moment	=	wl2/8		=	2.26l2/8

Design resistance moment	=       	Wpl x fy		=	81.3 x 275/103

=	22.36

Equating  and

2.26l2/8      	=	22.36

lmax		=	8.9 m

E7.4

- Design load	=	10 x 1.5		=	15 kN

15

Lever arm, l	=	0.8 cos 45°

=	0.566 m

M about A:

15 x 1.7		=	FBD x 0.566

Force in ram, FBD 	=	45.1 kN

M about E:

15

15 x 1.7		=	RF x x2.2

RF	=	11.59 kN

V = 0:

15	=	11.59 + RE

RE	=	3.41 kN

c.

Mmax	=	15 x 0.9		=	13.5 kNm

Wpl  required	=	(13.5 x 106)/(275 x 103)

=	49.1 cm3

From appendix:

Use 100 x 100 x 4.0 square hollow section  (Wpl = 54.9 cm3)

E7.5

| Part | Area (mm2 ) | yn | A yn | y | Ay2 | Iself |
| --- | --- | --- | --- | --- | --- | --- |
| Rectangle | 600 | 20 | 12000 | 1.51 | 1368 | 80 000 |
| Hole | -78.5 | 30 | -2355 | 11.51 | -10400 | -491 |
|  | 521.5 |  | 9645 |  | -9032 | 79 509 |

Y	=	9645/521.5	=	18.49

INA	=	79 509 – 9032	=	70 477 mm4

Wel,top	=     70 477/ (40-18.49)	=	3276 mm3

Wel,bottom=     70 477/ 18.49	=	3812 mm3

top	     =	0.5 x 106/3276	   =	153 N/mm2 compression

bottom	     =	0.5 x 106/3812	   =	131 N/mm2 tension

E7.5

a. Effective depth, d	=	450 – 30 12.5	=	407.5 mm

200

Based on concrete:

Mult	=	0.214 fckbd2	=     0.214 x 30 x 200 x 407.52 x 10-6

=	213.2 kNm

Based on reinforcement:

Table 3.1	Area of steel	=	2 x 491

=	982 mm2

Mult	=	0.696 Asfykd	=     0.696 x 982 x 500 x 407.5 x 10-6

=	139.3 kNm

407.5

But  MEd	=	wl2/8		=	40l2/8

l 	=       8 x 139.3/40)	=	5.28 m span

b. 	Shear force on beam, VEd	=	5.28 x 40/2	=	105.6 kN

Design shear stress, vEd		=	105.6 x 103/(200 x 407.5)	=	1.30 N/mm2

% reinforcing steel	=	100 As/bd

=	100 x 982/(200 x 407.5)		=	1.2

Interpolating from table 7.2  		vRd,c	=	0.71 N/mm2 < 1.30 – shear links required

New , vED		=	105.6 x 103/(200 x 407.5 x 0.9)

=	1.44 N/mm2

From table 7.3 (cot  = 2.5)		vRd,max	=	5.28 > 1.30

Asw		=	vEdsb /1087	=	1.44sb/1087

Max link spacing, s	=	0.75 x 407.5	=	305.6

Say	=	300 mm

Asw		=	1.44 x 300 x 200/1087

=	79.5 mm2

From table 3.1			Use 8 mm dia. Links at 300 mm spacing (Asw   =   100 mm2)

# Chapter 8 - Compression

E8.1

From equation 8.1 		NRd	=	fck/c  ×  b  ×  hw  ×  (1 – 2e/hw)

Where:		Eccentricity, e	=	lw/400	=	3750/400

=	9.4 mm

NRd	=	40/1.5  ×  400  ×  400  ×  (1- 2 × 9.4/400) x 10-3

=	4066 kN

E8.2

For 10mm x 10mm strut, I	=	bd3 /12	=	10 x 103 /12	=	833 mm4

`For steel , E	=	210 000 N/mm2

Euler buckling load , Pcrit	=	π2EI/L2	=	π2x 210 000 x 833/9002

=	2130 N

E8.3

Design load	=	250/2 x 1.5		=	187.5 kN

Approximate area	=	187.5 x 103 /100 x 102	=	18.7 cm2

From appendix		Try 114.3 x 6.3 circular hollow section (A = 21.4 cm2 , i = 3.82 cm)

Effective length		=	3500 x 2		=	7000 mm

Non-dimensional slenderness, 		=	7000/(86 x 38.2)	=	2.13

From fig 7.38				=	0.18

Buckling stress		=	275 x  0.18		=	49.5 N/mm2

Actual stress		=	187.5 x 103 /21.4 x 102	=	87.6 N/mm2

– no good

From appendix		Try 168.3 x 5.0 circular hollow section (A = 5.7 cm2 , i = 5.78 cm)

Non-dimensional slenderness, 		=	7000/(86 x 57.8)	=	1.4

From fig 7.38				=	0.38

Buckling stress		=	275 x  0.38		=	104.5 N/mm2

Actual stress		=	187.5 x 103 /25.7 x 102	=	73 N/mm2

Use 168.3 x 5.0 circular hollow section

# Chapter 9 – Combined axial and bending stresses

E9.1

Axial load, NEd	=	675 + 450 + 590

=	1715 kN

Approx. area required	=	1715 x 103/100 x 102

=	172 cm2

Try 305 x 305 x 137 Universal Column

(A = 175 cm2, Wpl,y = 2300 cm3, Wpl,z = 1052 cm3, iz= 7.82 cm,

h = 320.5, tf = 13.8)

Dimensions as shown

MyEd	=	(675 – 590) x (0.160 + 0.1)

=	22.1 kNm

MzEd	=	450 x (0.007 + 0.1)

=	48.2 kNm

Non-dimensional slenderness ratio, 	=	LE/(86 x iz)	=	6000/(86 x 78.2)

=	0.89

From [9.2]	 	NEd              My,Ed                               Mz,Ed

×      <  1.0

Afy         LTfyWpl,y                          fyWpl,z

From fig 7.38 				=	0.55

Buckling stress		=	0.55 x 275	=	151 N/mm2

Bending stress 		=	0.55 x 275	=	151 N/mm2

Axial resistance, Afy	=     175 x 151 x 10-1	=	2642 kN

y-y bending resistance,	LTfyWpl,y=      151 x 2300 x 10-3	=	347 kNm

z-z bending resistance, fyWpl,z	=     275 x 1052 x 10-3	=	289 kNm

substitute into equation [9.2]

1715         22.1                         48.2

×    =   0.963  <  1.0

2642         347                          

Use 305 x 305 x 137 Universal Column

E9.2

a.

| Item | Area (cm2) | yn (cm) | A yn | y | Ay2 | Iself |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 4.0 | 9.5 | 38 | 5.48 | 120 | - |
| 2 | 6.4 | 5.0 | 32 | 0.98 | 6 | 34 |
| 3 | 8.0 | 0.5 | 4 | 3.52 | 99 | 1 |
|  | 18.4 |  | 74 |  | 225 | 35 |

Y	=	74/18.4		=	4.02 cm

INA	=	225 + 44	=	260 cm4

Wel,inner	=	260/4.02	=	64.7 cm3

Welouter	=	260/(10-4.02)	=	43.5 cm3

b.			 Total load	=	(30 x 70) + 2000		=            4100 kg

Force		=	4100 x 9.81/103		=	40.2 kN

Eccentricity,e		=	0.26 +0.042		=	0.30 m

Moment		=	40.2 x 0.30		=	12.06 kNm

=	F/A     M/Wel

inner	=	40.2 x 103/18.4 x 102  +  12.06 x 106 /64.7 x 103

=	21.8 + 186.4		=	208.2 N/mm2 tens.

inner	=	40.2 x 103/18.4 x 102  -  12.06 x 106 /43.5 x 103

=	21.8 – 276.0		=           254.2 N/mm2 comp.

E9.3

Elastic moduli:

Wel,top		=	18 000/10	=	1800 cm3

Wel,bottom	=	18 000/15	=	1200 cm3

Sagging BM:

bottom	=	24 x 106/1200 x 103	=	20 N/mm2 tension

Hogging BM:

top	=	9 x 106/1800 x 103	=	5 N/mm2 tension

Required pre-stress	-

Stress at N.A. level	=	5 + (15 x 10/25)		=	11 N/mm2

pre-stressing force	=	11 x 200 x 102/103	=	220 kN

Consider top stresses,	top	=	P/A  -  Pe/ Wel,top	=	5

5	=	11 -220 x 103 x e/1800 x 103

5	=	11 – 0.122e

e	=	49.2 mm

Check bottom stresses,	bottom	=	11 + 220 x 103 x 49.2/1200 x 103

=	11 + 9.02	=	20.02 N/mm2  - check

E9.4

INA	=	BD3/12 – bd3/12

=	3 x 33/12 – 1.8 x 1.83/12

=	6.75 – 0.875

=	5.875 m4

Elastic modulus, Wel	=	5.875/1.5

=	3.917 m3

Area of section		=	32 – 1.82

=	5.76 m2

Total weight		=	30 x 5.76 x 20

=	3456 kN

Total wind force	=	30/2 x 1.3 x 3.0

=	54 kN

Resultant acts 2/3 of height up chimney:

=	2/3 x 30

=	20 m

Overturning moment	=	54 x 20

=	1080 kNm

Stresses at base		=	F/A  M/Wel

=	3456/5.76   1080/3.917

=	600   276 kN/m2

=	0.876 N/mm2  Compressive

And

=	0.324 N/mm2  Compression

# Chapter 10 - Torsion

E10.1

Tult	=	2yR3/3

=	2 x 159 x  x 153/3

=	1 124 000 Nmm

=	1.12 kNm

=	L/GR

=	(3000 x 90)/(77 000 x 15)

=	0.234 rad

=	13.4°

E10.2

a.

Area of sign				=	1.8 x 0.9		=	1.62 m2

Design load				=	1.62 x 1.1		=	1.782 kN

Bending moment				=	1.782 x 2.8		=	4.98 kNm

Torque				=	1.782 x (0.4 + 0.9/2)	=	1.51 kNm

b.

Plastic modulus for bending, Wpl	=	4.98 x 106/275 x 103	=	18.1 cm3

Try 60 x 60 x 4.0 square hollow section (Wpl   =   18.6 cm3)

bending	 shear	=	1782/(2 x 60 x 4)	=	3.7 N/mm2

Am			=	56 x 56 			=	3136 mm2

Equation [10.6]		torsion	shear		=	T/2Amt			=     1.51 x 106/(2 x 3136 x 4)

=	60.2 N/mm2

total		=	3.7 + 60.2		=          63.9 N/mm2 < 159

Use 60 x 60 x 4.0 square hollow section

E10.3

a.

Equation [10.7]			max	=	3T/at2

Where			a		=	145 + 70

=	215 mm

max	=	3 x 1.2 x 106/(215 x 102)

=	167 N/mm2

b.

=	3TL/at3G	=	3×1.2×106×4000215×103 ×77000

=	0.870 rad	=	49.8°

# Chapter 11 - Connections

E11.1

a.

based on gross area of tie – equation [6.2]

NRd 		=	70 x 10 x 275/103			=	192.5 kN

based on net area of tie – equation [6.3]

NRd 		=	(70-24)  x 10 x 0.9 x 410/(1.1 x 103)	=	154.3 kN

b.

based on shear capacity - equation [11.3]

Fv		=	2 x 0.6  x 400 x π x 112 x 10-3/1.25	=	146.0 kN

c.

based on bearing capacity - equation [11.4]

Fb		=	2 x 400 x 22 x 10 x 10-3/1.25		=	140.8 kN

Design resistance, NRd	=	140.8 kN

E11.2

a.

based on gross area of tie – equation [6.2]- as above

NRd 		=	70 x 10 x 275/103			=	192.5 kN

based on net area of tie – equation [6.3]

NRd 		=	(70-22)  x 10 x 0.9 x 410/(1.1 x 103)	=	161.0 kN

b.

based on slip resistance - equation [11.5]

Fv		=	2 x 0.7  x 1 x 0.5 x 800 x 245 x 10-3/1.25	=	109.80 kN

c.

based on bearing capacity - equation [11.4]

Fb		=	2 x 400 x 20 x 10 x 10-3/1.25		=	128 kN

Design resistance, NRd	=	109.8 kN

E11.3

| Item | A(cm2) | yn (cm) | Ayn(cm3) | Y (cm) | Ay2 (cm4) | Iself (cm4) |
| --- | --- | --- | --- | --- | --- | --- |
| Plate | 60 | 53.8 | 3228 | 17.4 | 18166 | 20 |
| Beam | 104 | 26.6 | 2746 | 10.0 | 10400 | 47500 |
|  | 164 |  | 5974 |  | 28566 | 47520 |

Ŷ	=	5974/164	=	36.4 cm

INA	=	28566 + 47520	=	76086 cm4

Equation [11.9]

shear flow/weld, q	 =	VQ/I		=	2500 x 60 x 17.4/76086

=	17.15 kN/cm

=	1.72 kN/mm

From table 10.1			Use 12 mm fillet weld (strength = 1.87 N/mm)

E11.4

| Item | A(cm2) | yn (cm) | Ayn(cm3) | Y (cm) | Ay2 (cm4) | Iself (cm4) |
| --- | --- | --- | --- | --- | --- | --- |
| Plate | 300 | 0.5 | 150 | 8.45 | 21421 | - |
| Beams | 205.2 | 21.3 | 4371 | 12.35 | 31298 | 55800 |
|  | 505.2 |  | 4521 |  | 52719 | 55800 |

Ŷ	=	4521/505.2		=	8.95 cm

INA	=	52 719 + 55 800		=	108 519 cm4

a.

Wpl,top		=	108 519/8.95		=	 12 125 cm3

Wpl,bottom	=	108 519/(41.6-8.95)	=	3 324 cm3

For critical BM put load at midspan:

MRd		=	PL/4			=	150 x 8/4

=	300 kNm

comp		=	300 x 106/12 125 x 103	=	25.0 N/mm2

tens		=	300 x 106/3 324 x 103	=	90.3 N/mm2

b.

For critical shear put load adjacent to support:

VRd		 =	150 kN

shear flow, q			 =	VQ/I		=	150 x 103 x 300 x 8.45/108 519

=	3504 N/cm

But this is resisted by 6 welds:

Shear flow/weld			=	3504/(6 x 10)	=	58.4 N/mm

# Chapter 12 – Arches and portal frames

E12.1

M about A:

(600 x 8) +(600 x 16) + (2400 x 24)		=	RE x 32

RE	=	2250 kN

V =0:

600 + 600 + 2400	=	2250 + RA

RA	=	1350 kN

M about C:

(2400 x 8) + (HE x 6)		=	2250 x 16

HE	=	2800 kN

H =0:

HA	=	-2800 kN

a.

MB	=	(1350 x 8 ) – (2800 x 6)

=	-6000 kNm – tension in top

b.

MD	=	(2800 x 6 ) – (2250 x 8)

=	-1200 kNm – tension in bottom

LDE		=	5 m

For thrust resolve forces parallel to DE:

Thrust		=	2250 x 3/5 + 2800 x 4/5	=   3590 kN

For shear resolve forces perpendicular to DE:

Shear		=	2250 x 4/5 - 2800 x 3/5	=   120 kN

E12.2

M about A:

(20 x 4) +(40 x 4) + (80 x 8) + (40 x12)	=	(RBV x 12) + (RBH x 1)

1360	=	12RBV + RBH

M about D:

(40 x 4) + (RBH x 3)		=	RBV x 4

160	=	4RBV - 3RBH

x 3					480	=	12RBV - 9RBH

- 					880	=	10RBH

RBH	=	88 kN

Sub in 				1360	=	12RBV  +  88

RBV	=	106 kN

V =0:

40 + 80 + 40	=	106 + RAV

RAV	=	54 kN

H =0:

RAH + 20		=	88

RAH	= 	68 kN

MC		=	(63 x 4) – (68 x 4)	=	-56 kNm (tension in top)

ME		=	(88 x 3)			=	264 kNm (tension in top)

Thrust AC	=	54 cos 45° + 68 cos 45°	=	86.3 kN (compression)

Shear AC	=	54 cos 45° - 68 cos 45°	=	-9.9 kN

E12.3

M about A:

(2.5 x 6 x 3) +(6.5 x 16 x 8) + (1.2 x 6 x3)		=	RBV x 16

RBV	=	56.2 kN

M about D:

(6.5 x 8 x 4) +( RBH x 7)	=	(56.2 x 8) +(1.2 x 6 x 4)								RBH	=	38.6 kN

V =0:

6.5 x 16 	=	56.2 + RAV

RAV	=	47.8 kN

H =0:

(2.5 x 6) +(1.2 x 6 ) + RAH	=	38.6										RAH	=	16.4 kN

Design values:

MRd	=	210 kNm

NRd	=	56.2 kN

E12.4

Using equations 6.1 to 6.3

Reactions RAV	=	RBV		=	wL/2	=	40 x 20/2	=	400 kN

Reactions RAH	=	-RBH	=	wL2/8D	=	40 x 202/(8x4)	=	500 kN

Maximum force in arch	=	(4002 + 5002)		=	640.3 kN

Approximate area required			=	640.3 x 103/150 x 102		=	42.7 cm2

Try 152 x 152 x 37 Universal Column (iz= 3.87 cm, A = 47.4 cm2)

Non-dimensional slenderness ratio,  	=	2000/(86 x 38.7)	=	0.60

From figure 7.38			=	0.76

NRd	=	0.76 x 47.4 x 275	=	991 kN>640.3

Try a smaller size:

Try 152 x 152 x 30 Universal Column (iz= 3.82 cm, A 38.2 cm2)

Non-dimensional slenderness ratio,  	=	2000/(86 x 38.2)	=	0.61

From figure 7.38			=	0.75

NRd	=	0.75 x 38.2 x 275	=	788 kN>640.3

Use 152 x 152 x 30 Universal Column

# Chapter 13 – Foundations and retaining walls

E13.1

Total unfactored load 	=	350 + 275	=	625 kN

table 13.1	 -  stiff clay , bearing pressure, gd	=	200 kN/m2

Area of foundation	=	625/200	=	3.125 m2

For a square, width	=	3.125		=	1.77 m say 1.8 m

Projection		=	0.9 -0.2		=	0.7 m

From section 13.5.2				Depth, say	=	0.75 m

E13.2

For reinforced concrete pad use same plan dimensions as above but only 300 mm deep.

Design column load, NRd			=	(1.35 x 350) + (1.5 x 275)	=	885 kN

Ground pressure, q			=	885/1.82			=	273 kN/m2

Design moment, MRd			=	0.5 x 273 x 1.8 (1.8/2 -0.4/2)2	=	120 kNm

Estimate 12 mm dia. Bars

Effective depth, d			=	300 – 40 -12 -6			=	242 mm

K	=	M/bd2fck	=	120 x 106/(1800 x 2422 x 30)

=	0.04

From figure 7.47		z/d		=	0.95

z		=	0.95 x 242	=	 230 mm

As		=	M/fyz		=	120 x 106/(500/1.15 x 230)

=	1200 mm2

Number of 12 mm bars			=	1200/113	=	11

1.5(c + 3d)			=	1.5(0.4 + 3 x 0.242)		=	1.69 m < 1.8 m

put 2/3 of bars in middle band

Width of band	=	C  + 3d	=	0.4 + (3 x 0.242)		=	1.13 m

Number of bars in band	=	2/3 x 11	=	7

Spacing within band 	=	1130/6	=	188 mm

But spacing if evenly spaced 		=	(1800-80)/10		=	172 mm

Use even spacing

Shear across pad

Shear force, VEd	=	q x L x  L/ 2 – C/2 – d)	=	273 x 1.8 x (0.9 – 0.2 – 0.242)

=	225 kN

Shear area	=	L x d	=	1800 x 242		=	435 600 mm2

Shear stress, vEd	=	VEd/(L x d)	=	225 x 103/435 600	=	0.52 N/mm2

100 As/bd	=	(100 x 1200)/(1800 x 242)	=	0.28

From table 7.2	vRd,c	=	0.54 > 0.52 – no shear reinforcement required.

Punching shear

Check at the face of the column:

Shear stress, vEd	=	qL2 -C24Cd        =	    2731.82 -0.42×10-34×0.4×0.242

=	2.17 N/mm2

From table 7.3 			vRd,max		=	5.28 N/mm2  >  2.17 N/mm2

Check at a = d:

Shear stress, vEd	=	qL2 -4Cd-πd24C+2πdd

=      2731.82 - 4×0.4×0.242- π×0.2422×10-3(4×0.4+2×π×0.242)×0.242

=      0.961 N/mm2

From shear across pad		vRd,c		=	0.54 N/mm2

vRd,		=	0.54 × 2d/d

=	1.08 N/mm2 > 0.961 N/mm2

Check at a = 2d:

Shear stress, vEd	=	qL2 -8Cd-4πd24C+πdd

=     2731.82 - 8×0.4×0.242- 4×π×0.2422×10-34(0.4+π×0.242)×0.242

=	0.413 N/mm2

From above 			vRd,c		=	0.54 N/mm2

vRd 		=	0.54 × 2d/2d

=	0.54 N/mm2> 0.413 N/mm2

Proposed dimensions and reinforcement are satisfactory

E13.3

M about A:

| Item | Weight (kN) | Distance (m) | Moment (kNm) |
| --- | --- | --- | --- |
| 1 | 1.0 x 2.5 x 24 = 60.0 | 0.750 | 45.0 |
| 2 | 0.25 x 0.75 x 24 = 4.5 | 0.125 | 0.5 |
|  | 64.5 |  | 45.5 |

Distance from A to centre of mass	      =	45.5/64.5					      =	0.705 m

Distance from base centre			      =	0.705 – 0.625

=	0.080 m

Soil pressure at base	=	Kasz			=	0.33 x 20 x 2.5	    =	16.5 kN/m2

Active force	=	16.5/2 x 2.5		=	20.63 kN

Overturning moment	=	20.63 x 2.5/3		=	17.2 kNm

Eccentiicity, er	=	17.2 – (64.5 x 0.08)/64.5	=	0.187 m

Distance to edge of mid. 1/3rd	=	1.25/6

=	0.208 m>0.187 m

i.e. resultant is within middle third

max		=	F/A  + Fer/Wel		=	64.5/1.25+64.5 x 0.187/(1 x 1.252/6)

=	51.6 + 37.1			=	88.7 kN/m2

Sliding:

Active force	=	20.63 kN

Friction	=	64.5 x 0.6			=	38.7 kN

Passive resistance force	=	Kpsz2/2			= 	3 x 20 x 0.52/2	    =	7.5 kN

Factor of safety	=	(38.7 + 7.5)/20.63		=	2.24

E13.4

Pressure behind wall becomes:

Ka’z          =        0.33 x (20 – 9.81) x 2.5   =   3.36 kN/m2

wz             =       9.81 x 2.5				=   24.53 kN/m2

Total pressure   =   3.36 + 24.53		=	27.89 kN/m2

Force         =        27.89/2 x 2.5		=	34.9 kN

Moment   =        34.9 x 2.5/3			=	29.1 kNm

Er                         =       29.1 – (64.5 0.08)/64.5	=         0.37 m > 0.208 m

i.e outside middle third

x	=	d/2 - er		=	0.625 – 0.37	=	0.255

max		=	2 x 64.5/(3 x 0.255)	=	169 kN/m2

Sliding:

Factor of safety		=	(38.7 + 7.5)/34.9	=	1.32

# Chapter 14 – Deflection

E14.1

E   =   7000

| Member | L (mm) | A (mm2) | FL (N) | = FLLEA (mm) | Fu | Fu |
| --- | --- | --- | --- | --- | --- | --- |
| AD | 3606 | 5000 | 14 400 | -1.48 | -0.60 | 0.89 |
| DC | 3606 | 5000 | 10 800 | -1.11 | -0.60 | 0.67 |
| CE | 3606 | 5000 | 10 800 | -1.11 | -1.20 | 1.33 |
| EB | 3606 | 5000 | 14 400 | -1.48 | -1.20 | 1.78 |
| BG | 4000 | 5000 | 12 000 | 1.37 | 1.00 | 1.37 |
| GF | 4000 | 5000 | 6 000 | 0.69 | 0.50 | 0.35 |
| FA | 4000 | 5000 | 12 000 | 1.37 | 0.50 | 0.69 |
| DF | 2236 | 5000 | 6 700 | -0.43 | 0 | 0.00 |
| FC | 4472 | 5000 | 6 700 | 0.86 | 0 | 0.00 |
| CG | 4472 | 5000 | 6 700 | 0.86 | 1.12 | 0.96 |
| GE | 2236 | 5000 | 6 700 | -0.43 | 0 | 0.00 |
|  |  |  |  |  | = | 8.04 |

Deflection	=	8.04 mm

E14.2

from appendix:

I   =   18 600 cm4

E  =  205 000 N/mm2

Use superposition:

Deflection, 	=	5wL4384EI+PL348EI

=	5×16×75004384×205000×18600×104+32×7500348×205000×18600×103

=	17.3 + 7.4

=	24.7 mm

E14.3

Aluminium, 	E 	=  70 000 N/mm2   -   table 3.3

2nd mom. Area, I		=  R4/4   -   r4/4   - figure 7.31

=   /4 (604 -  584)   =   1.29 x 106 mm4

=	PL3/3EI

=	150 x 60003/(3 x 70 000 x 1.29 x 106)

=	120 mm

E14.4

Table 3.1	As	=	3 x 491	=	1473 mm2

Effective depth, d	=	500 – 40 - 10 – 12.5   =   437.5 mm

% reinforcement	=	100 x 1473/(300 x 437.5)	   =   1.12%

Table 14.1 by interpolation:

Basic span/depth ratio	=	6.76

Maximum span	=	6.76 x 437.5	=	2960 mm   say 3.0 m

# Chapter 15 – Indeterminate structures and computers

E15.1

a. Each crossed member (2) adds a redundancy plus additional internal roller supports (2) – answer – 4.

b. Simple cantilever is statically determinate so internal roller supports add two redundancies -  answer – 2

c. A Three-pin portal is statically determinate so making two pins into rigid joints adds two redundancies  - answer – 2

E15.2

M about A:

-267 + (300 x 2) + 133 – 6RB	=	0

RB	=	78 kN

`              V = 0

RA 	=	300 – 78

=	222 kN

MC	=	267 –(222 x 2)

=	177 kNm

Design moment, MEd	=	267 kNm	(i.e. biggest moment on beam)

Wpl		=	267 x 106/275 x 103 		  (M  =  1.0)

=	971 cm3

Use 406 x 178 x 54 kg/m universal beam (Wpl   =   1050 cm3)

E15.3

MP(2 + 3 + )	=		300 x 4

6MP		=	1200

MP		=	200 kNm

Wpl		=	200 x 106/275 x 103 	(M  =  1.0)

=	727 cm3

Use 406 x 140 x 39 kg/m universal beam (Wpl   =   724 cm3)

% reduction		=	(54-39)/54 x 100    =   27.8%