Microchip TCP/IP Stack Release Notes:

BETA v[6.00.04]       [14 June 2013]

# Introduction

This document contains the release notes for the Beta revision v6.00.04 of the TCPIP stack.

Unless otherwise mentioned, the release notes for the previous version (v6.00, v6.00.02) still apply.

The following release notes are relevant to the new features added to the version v6.00.04 of the TCP/IP stack.

For the vast majority of the TCP/IP related issues the regular help file distributed with MCHP MLA is still valid and should be consulted as a source of information.

- This release of the v6 stack does not support 16 or 8 bit processor platforms (PIC24, dsPIC, PIC18, etc.). PIC24, dsPIC and PIC18 support is currently not planned for TCPIP Stack v6. Please consider using PIC32 or continue using TCPIP Stack v5 for 8/16 bit platforms at this time.

- Please note that this release does contain the Wi-Fi related projects. The updated Wi-Fi G module driver and files are contained in this distribution. Please see the “TCPIP Stack Release_Beta v6.00.04_MRF24WG_Notes.docx”  document included in this release.

## Demos Provided

The release contains demos for the following PIC32 platforms:

- Explorer16 + ENC624J600

- Explorer16 + MRF24WG

- Ethernet Starter Kit (ESK)

- ESK + MRF24WG

  - Please note that in order to use the MRF24WG module with the PIC32ESK you need to use an I/O Expansion board (DM320002) and insert the MRF24WG module in one of the PICtail Plus slots (SPI1 position is the default).

  - You need to connect the J10/PMPA4/56 and J10/INT1/37 pins on the I/O Expansion board so that the MRF24WG interrupt signal reaches the PIC32 processor. For this configuration the MRF24WG driver uses a CN (change notice) interrupt.

- USB SK + ENC28J60

- USB SK + ENC624J600

## SW License Agreement

MICROCHIP IS WILLING TO LICENSE THE ACCOMPANYING SOFTWARE AND DOCUMENTATION TO YOU ONLY ON THE CONDITION THAT YOU ACCEPT ALL OF THE FOLLOWING TERMS.  TO ACCEPT THE TERMS OF THIS LICENSE, CLICK "I ACCEPT" AND PROCEED WITH THE DOWNLOAD OR INSTALL.  IF YOU DO NOT ACCEPT THESE LICENSE TERMS, CLICK "I DO NOT ACCEPT," AND DO NOT DOWNLOAD OR INSTALL THIS SOFTWARE.

NON-EXCLUSIVE SOFTWARE LICENSE AGREEMENT

- This Nonexclusive Software License Agreement ("Agreement") is a contract between you, your heirs, successors and assigns ("Licensee") and Microchip Technology Incorporated, a Delaware corporation, with a principal place of business at 2355 W. Chandler Blvd., Chandler, AZ 85224-6199, and its subsidiary, Microchip Technology (Barbados) II Incorporated (collectively, "Microchip") for the accompanying Microchip software including, but not limited to, Graphics Library Software, IrDA Stack Software, MCHPFSUSB Stack Software, Memory Disk Drive File System Software, mTouch(TM) Capacitive Library Software, Smart Card Library Software, TCP/IP Stack Software, MiWi(TM) DE Software, Security Package Software, and/or any PC programs and any updates thereto (collectively, the "Software"), and accompanying documentation, including images and any other graphic resources provided by Microchip ("Documentation").

- Definitions.  As used in this Agreement, the following capitalized terms will have the meanings defined below:

  - "Microchip Products" means Microchip microcontrollers and Microchip digital signal controllers.

  - "Licensee Products" means Licensee products that use or incorporate Microchip Products.

  - "Object Code" means the Software computer programming code that is in binary form (including related documentation, if any), and error corrections, improvements, modifications, and updates.

  - "Source Code" means the Software computer programming code that may be printed out or displayed in human readable form (including related programmer comments and documentation, if any), and error corrections, improvements, modifications, and updates.

  - "Third Party" means Licensee’s agents, representatives, consultants, clients, customers, or contract manufacturers.

  - "Third Party Products" means Third Party products that use or incorporate Microchip Products.

- Software License Grant.  Microchip grants strictly to Licensee a non-exclusive, non-transferable, worldwide license to:

  - use the Software in connection with Licensee Products and/or Third Party Products;

  - if Source Code is provided, modify the Software; provided that Licensee clearly notifies Third Parties regarding the source of such modifications;

  - distribute the Software to Third Parties for use in Third Party Products, so long as such Third Party agrees to be bound by this Agreement (in writing or by "click to accept") and this Agreement accompanies such distribution;

  - sublicense to a Third Party to use the Software, so long as such Third Party agrees to be bound by this Agreement (in writing or by "click to accept");

  - with respect to the TCP/IP Stack Software, Licensee may port the ENC28J60.c,  ENC28J60.h, ENCX24J600.c, and ENCX24J600.h driver source files to a non-Microchip Product used in conjunction with a Microchip ethernet controller;

  - with respect to the MiWi (TM) DE Software, Licensee may only exercise its rights when the Software is embedded on a Microchip Product and used with a Microchip radio frequency transceiver or UBEC UZ2400 radio frequency transceiver which are integrated into Licensee Products or Third Party Products.

For purposes of clarity, Licensee may NOT embed the Software on a non-Microchip Product, except as described in this Section.

- Documentation License Grant.  Microchip grants strictly to Licensee a non-exclusive, non-transferable, worldwide license to use the Documentation in support of Licensee's authorized use of the Software

- Third Party Requirements.  Licensee acknowledges that it is Licensee’s responsibility to comply with any third party license terms or requirements applicable to the use of such third party software, specifications, systems, or tools.  This includes, by way of example but not as a limitation, any standards setting organizations requirements and, particularly with respect to the Security Package Software, local encryption laws and requirements.  Microchip is not responsible and will not be held responsible in any manner for Licensee’s failure to comply with such applicable terms or requirements.

- Open Source Components.  Notwithstanding the license grant in Section 1 above, Licensee further acknowledges that certain components of the Software may be covered by so-called "open source" software licenses ("Open Source Components").  Open Source Components means any software licenses approved as open source licenses by the Open Source Initiative or any substantially similar licenses, including without limitation any license that, as a condition of distribution of the software licensed under such license, requires that the distributor make the software available in source code format.  To the extent required by the licenses covering Open Source Components, the terms of such license will apply in lieu of the terms of this Agreement.  To the extent the terms of the licenses applicable to Open Source Components prohibit any of the restrictions in this Agreement with respect to such Open Source Components, such restrictions will not apply to such Open Source Component.

- Licensee Obligations.  Licensee will not: (a) engage in unauthorized use, modification, disclosure or distribution of Software or Documentation, or its derivatives; (b) use all or any portion of the Software, Documentation, or its derivatives except in conjunction with Microchip Products, Licensee Products or Third Party Products; or (c) reverse engineer (by disassembly, decompilation or otherwise) Software or any portion thereof.  Licensee may not remove or alter any Microchip copyright or other proprietary rights notice posted in any portion of the Software or Documentation.  Licensee will defend, indemnify and hold Microchip and its subsidiaries harmless from and against any and all claims, costs, damages, expenses (including reasonable attorney's fees), liabilities, and losses, including without limitation: (x) any claims directly or indirectly arising from or related to the use, modification, disclosure or distribution of the Software, Documentation, or any intellectual property rights related thereto; (y) the use, sale and distribution of Licensee Products or Third Party Products; and (z) breach of this Agreement.

- Confidentiality.  Licensee agrees that the Software (including but not limited to the Source Code, Object Code and library files) and its derivatives, Documentation and underlying inventions, algorithms, know-how and ideas relating to the Software and the Documentation are proprietary information belonging to Microchip and its licensors ("Proprietary Information").  Except as expressly and unambiguously allowed herein, Licensee will hold in confidence and not use or disclose any Proprietary Information and will similarly bind its employees and Third Party(ies) in writing.  Proprietary Information will not include information that: (i) is in or enters the public domain without breach of this Agreement and through no fault of the receiving party; (ii) the receiving party was legally in possession of prior to receiving it; (iii) the receiving party can demonstrate was developed by the receiving party independently and without use of or reference to the disclosing party's Proprietary Information; or (iv) the receiving party receives from a third party without restriction on disclosure.  If Licensee is required to disclose Proprietary Information by law, court order, or government agency, License will give Microchip prompt notice of such requirement in order to allow Microchip to object or limit such disclosure.  Licensee agrees that the provisions of this Agreement regarding unauthorized use and nondisclosure of the Software, Documentation and related Proprietary Rights are necessary to protect the legitimate business interests of Microchip and its licensors and that monetary damage alone cannot adequately compensate Microchip or its licensors if such provisions are violated.  Licensee, therefore, agrees that if Microchip alleges that Licensee or Third Party has breached or violated such provision then Microchip will have the right to injunctive relief, without the requirement for the posting of a bond, in addition to all other remedies at law or in equity.

- Ownership of Proprietary Rights.  Microchip and its licensors retain all right, title and interest in and to the Software and Documentation including, but not limited to all patent, copyright, trade secret and other intellectual property rights in the Software, Documentation, and underlying technology and all copies and derivative works thereof (by whomever produced).  Licensee and Third Party use of such modifications and derivatives is limited to the license rights described in this Agreement.

- Termination of Agreement.  Without prejudice to any other rights, this Agreement terminates immediately, without notice by Microchip, upon a failure by Licensee or Third Party to comply with any provision of this Agreement.  Upon termination, Licensee and Third Party will immediately stop using the Software, Documentation, and derivatives thereof, and immediately destroy all such copies.

- Warranty Disclaimers.  THE SOFTWARE AND DOCUMENTATION ARE PROVIDED "AS IS" WITHOUT WARRANTY OF ANY KIND, EITHER EXPRESS OR IMPLIED, INCLUDING WITHOUT LIMITATION, ANY WARRANTY OF MERCHANTABILITY, TITLE, NON-INFRINGEMENT AND FITNESS FOR A PARTICULAR PURPOSE.  MICROCHIP AND ITS LICENSORS ASSUME NO RESPONSIBILITY FOR THE ACCURACY, RELIABILITY OR APPLICATION OF THE SOFTWARE OR DOCUMENTATION.  MICROCHIP AND ITS LICENSORS DO NOT WARRANT THAT THE SOFTWARE WILL MEET REQUIREMENTS OF LICENSEE OR THIRD PARTY, BE UNINTERRUPTED OR ERROR-FREE.  MICROCHIP AND ITS LICENSORS HAVE NO OBLIGATION TO CORRECT ANY DEFECTS IN THE SOFTWARE.

- Limited Liability.  IN NO EVENT WILL MICROCHIP OR ITS LICENSORS BE LIABLE OR OBLIGATED UNDER ANY LEGAL OR EQUITABLE THEORY FOR ANY DIRECT OR INDIRECT DAMAGES OR EXPENSES INCLUDING BUT NOT LIMITED TO INCIDENTAL, SPECIAL, INDIRECT, PUNITIVE OR CONSEQUENTIAL DAMAGES, LOST PROFITS OR LOST DATA, COST OF PROCUREMENT OF SUBSTITUTE GOODS, TECHNOLOGY, SERVICES, OR ANY CLAIMS BY THIRD PARTIES (INCLUDING BUT NOT LIMITED TO ANY DEFENSE THEREOF), OR OTHER SIMILAR COSTS.  The aggregate and cumulative liability of Microchip and its licensors for damages hereunder will in no event exceed $1000 or the amount Licensee paid Microchip for the Software and Documentation, whichever is greater.  Licensee acknowledges that the foregoing limitations are reasonable and an essential part of this Agreement.

- General.  THIS AGREEMENT WILL BE GOVERNED BY AND CONSTRUED UNDER THE LAWS OF THE STATE OF ARIZONA AND THE UNITED STATES WITHOUT REGARD TO CONFLICTS OF LAWS PROVISIONS.  Licensee agrees that any disputes arising out of or related to this Agreement, Software or Documentation will be brought exclusively in either the U.S. District Court for the District of Arizona, Phoenix Division, or the Superior Court of Arizona located in Maricopa County, Arizona.  This Agreement will constitute the entire agreement between the parties with respect to the subject matter hereof.  It will not be modified except by a written agreement signed by an authorized representative of Microchip.  If any provision of this Agreement will be held by a court of competent jurisdiction to be illegal, invalid or unenforceable, that provision will be limited or eliminated to the minimum extent necessary so that this Agreement will otherwise remain in full force and effect and enforceable.  No waiver of any breach of any provision of this Agreement will constitute a waiver of any prior, concurrent or subsequent breach of the same or any other provisions hereof, and no waiver will be effective unless made in writing and signed by an authorized representative of the waiving party.  Licensee agrees to comply with all import and export laws and restrictions and regulations of the Department of Commerce or other United States or foreign agency or authority. The indemnities, obligations of confidentiality, and limitations on liability described herein, and any right of action for breach of this Agreement prior to termination, will survive any termination of this Agreement. Any prohibited assignment will be null and void.  Use, duplication or disclosure by the United States Government is subject to restrictions set forth in subparagraphs (a) through (d) of the Commercial Computer-Restricted Rights clause of FAR 52.227-19 when applicable, or in subparagraph (c)(1)(ii) of the Rights in Technical Data and Computer Software clause at DFARS 252.227-7013, and in similar clauses in the NASA FAR Supplement.  Contractor/manufacturer is Microchip Technology Inc., 2355 W. Chandler Blvd., Chandler, AZ 85224-6199.

If Licensee has any questions about this Agreement, please write to Microchip Technology Inc., 2355 W. Chandler Blvd., Chandler, AZ 85224-6199 USA. ATTN: Marketing.

Copyright (c) 2012 Microchip Technology Inc.  All rights reserved.

License Rev. No. 05-012412

# Release Notes

## TCP/IP stack

Updates:

- The stack is now distributed with the following encryption modules included:

  - rsa.c and  arcfour.c for SSL operation

  - AES_PIC32MX.a for SNMPv3 operation

Known issues:

- There is an ongoing development effort on some of the stack modules in order to reduce the code size of the stack. Part of the planned updates includes changes in the MAC driver allowing a full buffered API and removal of the direct control and manipulation of the on MAC memory resources. These changes will have impact both the stack footprint and the increased transfer performance.

## TCP

Updates:

- TCP state machine has been updated so that higher throughput numbers are now possible (see the iperf benchmark results below).

## IPv6

Updates:

- The IPv6 API has been updated and a function for listing the network interface associated IPv6 addresses was added:

  - tcpip_manager.h:: TCPIP_STACK_NetIPv6AddressGet()

Fixes:

- A potential security issue that could lead to memory corruption because of malformed IPv6 packets was identified and solved. The issue occurs because the function TCPIP_IPV6_ProcessFragmentationHeader() did not validate the "fragment offset" field in the IPv6 fragmentation header.

- Thanks to Andrew Zonenberg of RPISEC for identifying and signaling this issue to us.

## HTTP:

Updates:

- The HTTP API has been updated to contain a HTTP connection handle argument. See the “TCPIP_Stack_v5tov6_upgrade.docx” document for details.

Known issues:

- The HTTP server is still using the MPFS file system although the SYS_FS system service is available. Some issues with the handling of the dynamic variables are still investigated.

## ZeroConf:

Updates:

- A priority has been established between stack services that try to provide an IP address to an interface: DHCP client, ZeroConf, Default IP Address, DHCP Server. Multiple address services running over the same interface are considered to be in conflict and only one service is allowed at a time. The active service can be selected at initialization in

network_config.h:: TCPIP_HOSTS_CONFIGURATION[]::startFlags field.

Alternatively the address services can be dynamically started/stopped at run time.

For example, if both DHCP client and ZeroConf are enabled for a specific interface, i.e.

TCPIP_NETWORK_CONFIG_DHCP_CLIENT_ON

TCPIP_NETWORK_CONFIG_ZCLL_ON

flags are set in the network_config.h::startFlags then when disabling the DHCP client on that interface the ZeroConf will start automatically.

Fixes:

- A bug that was clearing the network IP address and mask to zero has been solved.

Known issues:

- The mDNS module may occasionally crash in a network environment with many Bonjour hosts having long TXT resource records. The exact cause of the bug is under investigation. An update will be provided when all the implications are understood.

## BSD API

Fixes:

- Improper usage of function TCPOpenClient() (trying to connect to IP address 0) in the berkeley_api.c::connect function was fixed.

Known issues:

- This module is currently being updated and improved. It will be available in the next stack release.

## DNS Server

Known issues:

- This module is currently being updated. It should not be enabled in the current release of the stack.

## SMTP:

Updates:

- The SMTP API has been updated to eliminate the usage of global variables. See the “TCPIP_Stack_v5tov6_upgrade.docx” document for details.

## SSL

Updates:

- There is 3rd party SSL support for the Microchip TCP/IP stack from WolfSSL offering advanced encryption algorithms, TLS and certificate validation and manipulation. Visit their website for more details:

  - http://www.wolfssl.com/yaSSL/cyassl-pic32.html

Known issues:

- This module needs a complete rewrite  to solve well known issues:

  - Use the external Crypto Library API and not internal copies of the RSA and RC4.

  - Remove the SSL code from the TCP module. SSL has to be an independent module, capable of using a stack supplied transport mechanism.

  - New, more powerful symmetric encryption algorithms have to be supported.

- Functionality issues:

  - Connecting using RSA 2048 keys to google.com fails intermittently. Since the connection seems to always work when the module/stack is brought up, it seems to indicate an initialization issue. The fixes suggested in the forum posts do not completely solve the issue. Further investigation is needed.

## tcpip_command

Updates:

- The IPv6 interface address listing has been added to the “netinfo” command.

- New commands to turn on/off address services (DHCP, ZeroConf and DHCP server) have been added. Use “help” command for details.

  - Note that if the DHCP client is enabled, the ZeroConf address service is not started. You have to disable the DHCP client and start the ZeroConf either using the tcpip console commands or from within the application. For example, the following sequence of tcpip console commands :

    - >dhcp PIC32INT off

    - >zcll PIC32INT on

will disable the DHCP client and start the ZeroConf address service.

## TCPIP storage

  - The TCP/IP storage service is now obsolete and is no longer maintained.

    - The symbol tcpip_config.h::TCPIP_STACK_USE_STORAGE should not be enabled.

## System Services:

Known issues:

- The MDD support has been added to the SYS-FS service. However, because of issues with running the HTTP server, MDD has not been added to the current release.

## Documentation updates:

The TCPIP_Stack_v5tov6_upgrade.docx migration document has been updated with the latest changes affecting the TCP/IP modules.

## iperf benchmark updates:

Following are the numbers obtained with the current release of the TCP/IP stack over an Ethernet interface using the internal PIC32 100 Mbps Ethernet controller:

TCP (4096 Bytes TX and RX buffers):

| Host | Command | Result |
| --- | --- | --- |
| PIC32 | iperf -c 192.168.1.11 -t 10 -i 5 -l 1024 -x 100M -M 1460 | 26.85 Mbps |
| PC | iperf -s -i 5 -m -w 10K -N | 26.8 Mbps |
| PIC32 | iperf -s -i 5 -x 100M | 30.99 Mbps |
| PC | iperf -c mchpboard_e -t 10 -i 5 -l 1024 | 31.0 Mbps |

UDP:

| Host | Command | Result |
| --- | --- | --- |
| PIC32 | iperf -s -u -i 5 | 37.24 Mbps |
| PC | iperf -c mchpboard_e -u -b 100M -i 5 -t 10 -l 1024 | 37.2 Mbps |
| PIC32 | iperf -c 192.168.1.11 -u -b 100M -i 5 -t 10 -l 1024 | 22.58 Mbps |
| PC | iperf -s -u -i 5 | 24.3 Mbps |