LAGOS CLEANER ENERGY SCHOOLS INNOVATION CHALLENGE, LCESIC 2026

TECHNICAL STANDARDS & SPECIFICATIONS FOR TOOLKIT COMPONENTS

1. GENERAL PROCUREMENT STANDARDS

All materials must meet the following baseline requirements:

✅ Quality Standards

- Durable under repeated student use (minimum 6-week project cycle)

- Suitable for beginner to intermediate learning levels

- Resistant to mild environmental exposure (heat, dust)

- Compatible with low-voltage systems (≤ 24V)

✅ Safety Requirements

- No exposed live wiring

- Insulated components where applicable

- Compliant with international safety practices (IEC/ISO recommended)

- Non-toxic materials

✅ Educational Suitability

- Easy to assemble/disassemble

- Supports experimentation and prototyping

- Clearly labeled components

2. RENEWABLE ENERGY COMPONENT SPECIFICATIONS

🔆 Solar Photovoltaic Panels

- Power Rating: 3W – 10W

- Voltage Output: 5V – 18V DC

- Type: Polycrystalline or Monocrystalline

- Frame: Reinforced plastic or aluminum edge

- Protection: Scratch-resistant laminated surface

🔋 Rechargeable Batteries

- Type: Li-ion or LiFePO4 recommended

- Voltage Range: 3.7V – 12V

- Capacity: ≥ 2000mAh

- Protection: Built-in Battery Management System (BMS)

- Cycle Life: Minimum 300 cycles

⚡ Charge Controllers

- Type: PWM (basic) or MPPT (optional advanced)

- Voltage Range: 6V–12V systems

- Protection:

  - Overcharge protection

  - Short-circuit protection

  - Reverse polarity protection

🔄 Small DC Motors

- Voltage: 3V – 12V DC

- RPM Range: 1000–10,000 RPM

- Housing: Enclosed casing for safety

- Mounting: Compatible with simple fixtures

💡 LED Lighting Systems

- Power: 0.5W – 5W

- Voltage: 3V – 12V

- Type: Low-energy white LEDs

- Lifespan: Minimum 20,000 hours

🔌 Solar Switches

- Type: Manual and light-sensitive switches

- Voltage Rating: ≤ 24V DC

- Housing: Insulated plastic casing

🔗 Wiring and Connectors

- Wire Gauge: 18–22 AWG

- Insulation: PVC coating

- Color Coding: Red (positive), Black (negative)

- Connectors: Plug-and-play terminals preferred

🔁 Inverters (Demonstration Models)

- Capacity: 50W – 150W

- Input: 12V DC

- Output: 220–240V AC (demonstration only)

- Safety: Overload and thermal protection

📊 Multimeters

- Type: Digital

- Functions:

  - Voltage (DC/AC)

  - Current

  - Resistance

- Safety: CAT II rated minimum

🌡 Temperature Sensors

- Type: Analog or digital (e.g., LM35, DS18B20)

- Range: -10°C to 100°C

- Accuracy: ±1°C

🌞 Light Intensity Sensors

- Type: LDR or photodiode

- Sensitivity: Suitable for visible light detection

- Output: Analog voltage signal

🔌 Circuit Boards & Breadboards

- Breadboards:

  - Minimum 400 tie-points

- Circuit boards:

  - Pre-drilled PCB boards

- Reusability: High

3. TOOLS AND HARDWARE STANDARDS

🛠 Hand Tools

- Screwdrivers:

  - Insulated handles

  - Multiple head types (flat, Phillips)

- Wire Cutters:

  - Hardened steel

  - Non-slip grips

- Measuring Tapes:

  - Minimum 3 meters

  - Metric system

🌡 Digital Thermometers

- Range: -10°C to 110°C

- Accuracy: ±1°C

- Display: LCD

4. PROTOTYPING MATERIALS SPECIFICATIONS

📦 Structural Materials

Cardboard

- Thickness: 3–7 mm corrugated

- Strength: Double-layer recommended

Acrylic Sheets

- Thickness: 2–5 mm

- Type: Clear or colored

- Finish: Smooth edges

PVC Pipes

- Diameter: 20–50 mm

- Length: Modular, easy-to-cut

🔩 Fabrication Materials

Adhesives

- Type:

  - Glue (non-toxic)

  - Epoxy (for strong bonding)

- Dry time: ≤ 30 minutes (basic)

Plastic Containers

- Type: Reusable, non-brittle

- Capacity: 0.5L – 5L

🪵 Wood Materials

- Type: Lightweight plywood or softwood

- Thickness: 5–10 mm

- Surface: Smooth (splinter-free)

🔧 Metal Scraps

- Type: Aluminum or mild steel

- Safety: No sharp edges (must be filed smooth)

♻️ Recyclable Materials

- Must be:

  - Clean

  - Safe to handle

  - Free of hazardous residues

5. SAFETY AND PROTECTION STANDARDS

🧤 Protective Gloves

- Material: Rubber or synthetic

- Fit: Various sizes (student-friendly)

- Function: Electrical and mechanical protection

🥽 Safety Goggles

- Type: Transparent polycarbonate

- Features:

  - Anti-fog

  - Scratch-resistant