980H WHEEL LOADER SPECIFICATIONS

- Equipped with reliable XIAN CUMMINS engine

- KD switch allows better efficiency & easy operation

- Reliable ZF transmission case and Torque Converter

- Famous original ZF wet-type axles with multi-disc limited-slip differentials deliver maximum traction in changing traction conditions

- Bucket automatic leveling ability

- Combined seal cylinder obtains high seal performance and more reliability

- Dual brake valve  & dual pipe full hydraulic brake and central lubrication

| Overall Dimension外形尺寸 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Length(with bucket on ground)整机长度（铲斗放置在地面上） |  |  |  |  |  |  |  |  |  |  |  |  | 8696 (mm) |  |
| 2 | Width(to outside of wheels)整机宽度（轮胎外沿） |  |  |  |  |  |  |  |  |  |  |  |  | 3099(mm) |  |
| 3 | Bucket width铲斗宽度 |  |  |  |  |  |  |  |  |  |  |  |  | 3250(mm) |  |
| 4 | Height(To the top of the cab)整机高度（到驾驶室顶部） |  |  |  |  |  |  |  |  |  |  |  |  | 3710(mm) |  |
| 5 | Wheel base轴距 |  |  |  |  |  |  |  |  |  |  |  |  | 3400(mm) |  |
| 6 | Tread轮距 |  |  |  |  |  |  |  |  |  |  |  |  | 2350(mm) |  |
| 7 | Min. ground clearance最小离地间隙 |  |  |  |  |  |  |  |  |  |  |  |  | 504(mm) |  |
| Main Technical Specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Rated load额定载荷 |  |  |  |  |  |  |  |  |  |  | 7500(Kg) |  |  |  |
| 2 | Operating weight操作重量 |  |  |  |  |  |  |  |  |  |  | 23500kg |  |  |  |
| 3 | Rated bucket capacity额定斗容 |  |  |  |  |  |  |  |  |  |  | 4.2(m3)，可选3.2-4.8 (m3) |  |  |  |
| 4 | Max. breakout force最大掘起力 |  |  |  |  |  |  |  |  |  |  | 218KN |  |  |  |
| 5 | Max. Drawing force最大牵引力 |  |  |  |  |  |  |  |  |  |  | 222 KN |  |  |  |
| 6 | Max. dump clearance最大卸载高度 |  |  |  |  |  |  |  |  |  |  | 3220(mm) |  |  |  |
| 7 | Dump reach卸载距离 |  |  |  |  |  |  |  |  |  |  | 1275(mm) |  |  |  |
| 8 | Dump angle at any position 卸载角（在任意位置） |  |  |  |  |  |  |  |  |  |  | ≥450 |  |  |  |
| 9 | Digging depth(with bucket bottom horizontal)挖掘深度（铲斗水平放置） |  |  |  |  |  |  |  |  |  |  |  |  |  | 35(mm) |
| 10 | Min. turning radius最小转弯半径 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (1)Outside of bucket 铲斗外侧 |  |  |  |  |  |  |  |  |  |  | 7042(mm) |  |  |  |
|  | (2)Outside of rear wheel后轮外侧 |  |  |  |  |  |  |  |  |  |  | 6486(mm) |  |  |  |
| 11 | Pivot angle frame 转向角 |  |  |  |  |  |  |  |  |  |  | 38+10 |  |  |  |
| 12 | Oscillating angle of rear axle后桥摆动角 |  |  |  |  |  |  |  |  |  |  | +12o |  |  |  |
| 13 | Lifting time of bucket铲斗提升时间 |  |  |  |  |  |  |  |  |  |  | 6.2(sec.) |  |  |  |
| 14 | Lowering time of bucket铲斗下降时间 |  |  |  |  |  |  |  |  |  |  | 3.9(sec.) |  |  |  |
| 15 | Dumping time倾翻时间 |  |  |  |  |  |  |  |  |  |  | 1.9(sec.) |  |  |  |
| 16 | Traveling speed(Km/h)行驶速度 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (1)1st gear (Forward & reverse)1档速度（前/后） |  |  |  |  |  |  |  |  |  |  | 7.4 |  |  |  |
|  | (2)2nd gear (Forward & reverse) 2档速度（前/后） |  |  |  |  |  |  |  |  |  |  | 12.4 |  |  |  |
|  | (3)3rd gear (Forward & reverse) 3档速度（前/后） |  |  |  |  |  |  |  |  |  |  | 25.9 |  |  |  |
|  | (4)4th gear (Forward) 4档速度（前） |  |  |  |  |  |  |  |  |  |  | 37 |  |  |  |
| Diesel engine柴油机 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Model型号 |  |  | XIAN Cummins QSM11-C290 西安康明斯QSM11-C290 |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Type型式 |  |  | Direct injection. Turbocharged. Air to air cooling 直喷、增压、空空中冷 |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Rated output额定功率 |  |  |  |  |  |  |  |  |  | 216(Kw) |  |  |  |  |
| 4 | Inter-bore of cylinder/stroke缸径/行程 |  |  |  |  |  |  |  |  |  | 125/147(mm) |  |  |  |  |
| 5 | Total exhaust of cylinder排气量 |  |  |  |  |  |  |  |  |  | 10.8(L) |  |  |  |  |
| 6 | Model of starting motor启动马达型号 |  |  |  |  |  |  |  |  |  | 42MT400 |  |  |  |  |
| 7 | Power of starting motor启动马达功率 |  |  |  |  |  |  |  |  |  | 7.5(KW) |  |  |  |  |
| 8 | Voltage of starting motor启动马达电压 |  |  |  |  |  |  |  |  |  | 24(V) |  |  |  |  |
| 9 | Rated speed额定转数 |  |  |  |  |  |  |  |  |  | 2100(r/min.) |  |  |  |  |
| 10 | Max. Torque最大扭矩 |  |  |  |  |  |  |  |  |  | 1478(N.m) |  |  |  |  |
| 11 | Starting type起动方式 |  |  |  |  |  |  |  |  |  | Electric |  |  |  |  |
| 12 | Min. specific fuel consumption最小燃油消耗率 |  |  |  |  |  |  |  |  |  | 212(g/Kw.h) |  |  |  |  |
| 13 | Engine specific oil consumption发动机机油消耗 |  |  |  |  |  |  |  |  |  | 1.0 (g/Kw.h) |  |  |  |  |
| 14 | Net weight发动机重量 |  |  |  |  |  |  |  |  |  | 980(Kg) |  |  |  |  |
| Transmission System传动系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. Torque Converter变矩器 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Type型式 |  |  |  |  |  | 3-element.single stage单级三元件 |  |  |  |  |  |  |  |  |
| 2 | Torque ratio变矩系数 |  |  |  |  |  |  |  |  |  | 2.985 |  |  |  |  |
| 3 | Max. Dia. of flow path循环圆直径 |  |  |  |  |  |  |  |  |  | 370(mm) |  |  |  |  |
| 4 | Cooling type冷却方式 |  |  |  |  |  | Pressure oil circulating压力油循环 |  |  |  |  |  |  |  |  |
| 2. Gear Box变速箱 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Model型号 |  |  |  |  | ZF 4WG260 |  |  |  |  |  |  |  |  |  |
| 2 | Type型式 |  |  |  |  | Power shift, spur gear constant-mesh动力换档，直齿常啮合 |  |  |  |  |  |  |  |  |  |
| 3 | Gear shift position档位数 |  |  |  |  | 4 Forward and 3 reverse gears前进4档后退3档 |  |  |  |  |  |  |  |  |  |
| 3. Axle and Tire桥和轮胎 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Type of main reducer主传动型式 |  |  |  |  |  |  | Spiral gear single stage螺旋伞齿轮单级 |  |  |  |  |  |  |
| 2 |  | Type of final reducer轮边减速型式 |  |  |  |  |  |  | single stage planetary行星轮式单级 |  |  |  |  |  |  |
| 3 |  | Total ratios总速比 |  |  |  |  |  |  | 24.66 |  |  |  |  |  |  |
| 4 |  | Size of tyre轮胎尺寸 |  |  |  |  |  |  | 26.5-25 -20PR (L-3) |  |  |  |  |  |  |
| Working Device Hydraulic System工作液压系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Model of oil pump油泵型号(含制动泵) |  |  |  |  |  |  | P360A367(DRPAA)ZA15-6/P124A082DXZA12-85 |  |  |  |  |  |
| 2 |  |  | System pressure系统压力 |  |  |  |  |  |  | 21(MPa) |  |  |  |  |  |
| 3 |  |  | Model of Muti-way directional valve多路阀型号 |  |  |  |  |  |  | GD40-00 |  |  |  |  |  |
| 4 |  |  | Model of pilot valve先导阀型号 |  |  |  |  |  |  | CRV-00（W2） |  |  |  |  |  |
| 5 |  |  | (d*l)Dimension of lifting cylinder 提升油缸尺寸 |  |  |  |  |  |  | Ф180*100*767(mm) |  |  |  |  |  |
| 6 |  |  | (D*l)Dimension of tilting cylinder翻斗油缸尺寸 |  |  |  |  |  |  | Ф210*110*550(mm) |  |  |  |  |  |
| Steering System转向系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Type型式 |  | Middle articulated frame. full hydraulic power中间铰接，全液压转向 |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  | Model of steering pump转向泵型号 |  |  |  |  | PB360A367DRZA17-6/P124ANSZA20 |  |  |  |  |  |  |  |
| 3 |  |  | Model of redirector转向器型号 |  |  |  |  | 150-1084/101-5TX-315-E |  |  |  |  |  |  |  |
| 4 |  |  | System pressure系统压力 |  |  |  |  | 17.5MPa |  |  |  |  |  |  |  |
| 5 |  |  | Dimension of steering cylinder 转向油缸尺寸 |  |  |  |  | Ф100*50*441(mm) |  |  |  |  |  |  |  |
| Brake System制动系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Type of traveling brake 行程制动系统 |  |  |  |  | Wet type multi-disc brake湿式多片制动器 Dual brake valve& dual pipe, full hydraulic brake 双制动阀和双管路，全液压制动 |  |  |  |  |  |  |  |
| 2 |  |  | System pressure系统压力 |  |  |  |  | 16 (MPa) |  |  |  |  |  |  |  |
| 3 |  |  | Type of emergency brake紧急制动型号 |  |  |  |  | Lower pressure alarm brake低压报警制动 |  |  |  |  |  |  |  |
| 4 |  |  | Type of parking brake停车制动型式 |  |  |  |  | Electric control brake电控制动 |  |  |  |  |  |  |  |
| Oil Capacity油量 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Fuel(diesel)柴油 |  |  |  |  |  |  |  |  |  | 423(L) |  |  |
| 2 |  |  | Engine oil发动机机油 |  |  |  |  |  |  |  |  |  | 32(L) |  |  |
| 3 |  |  | Oil for converter and gear box双变用油 |  |  |  |  |  |  |  |  |  | 38(L) |  |  |
| 4 |  |  | Oil for hydraulic system液压系统用油 |  |  |  |  |  |  |  |  |  | 300(L) |  |  |
| 5 |  |  | Oil for driving axles（F/R）驱动桥用油（前桥/后桥） |  |  |  |  |  |  |  |  |  | 51(L) for each |  |  |

980H装载机配置表

| 序号 | 名 称 | 型号 | 数量 | 厂 家 | 产地 |
| --- | --- | --- | --- | --- | --- |
| 1 | 发动机 | QSM11-C290 | 1 | 西安康明斯 | 中国 |
| 2 | 变速器 | 4WG2604646054110 | 1 | ZF | 进口 |
| 3 | 驱动桥 | 前桥4474 054 250 | 1 | ZF | 进口 |
|  |  | 后桥4474 054 251 | 1 | ZF | 进口 |
| 4 | 轮胎 | 26.5-25-20PR L-3 | 4 | 三角轮胎 | 中国 |
| 5 | 转向器 | 150-1084/101-5TX-315-E | 1 | DANFOSS | 进口 |
| 6 | 限位阀 | XF-B6 | 1 | 浙江临海 | 中国 |
| 7 | 转向油泵: | PB360A367DRZA17-6/P124ANSZA20 | 1 | 泊姆克 | 进口 |
| 8 | 转向油缸 | Z75H.10.5 | 1 | 常林 | 中国 |
|  |  | Z75H.10.14 | 1 | 常林 |  |
| 9 | 制动系统 | 980H.9C | 1 | MICO | 进口 |
| 10 | 工作液压系统 | 980H.14C | 1 | 常林 | 中国 |
| 11 | 工作液压油泵: | P360A367(DRPAA)ZA15-6/P124A082DXZA12-85 | 1 | 泊姆克 | 进口 |
| 12 | 多路换向阀 | GD40-00 | 1 | 高宇 | 中国 |
| 13 | 先导阀 | CRV-00（W2） | 1 | 高宇 | 中国 |
| 14 | 动臂油缸: | Z75H.14.20 | 2 | 常林 | 中国 |
| 15 | 转斗油缸: | Z75H.14.21 | 1 | 常林 | 中国 |
| 16 | 空调 | GQT-314 | 1 | 河南豫新 | 中国 |