# 918 WHEEL LOADER SPECIFICATIONS

- Equipped with reliable  Cummins or Chinese Luotuo engine

- 2 forward & 2 reverse power shift transmission

- Combined seal cylinder obtains high seal performance and more reliability

| - Overall Dimension |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Length(with bucket on ground) 整机长度（铲斗放置地面） |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5880 (mm) |
| 2 | Width(to outside of wheels) 整机宽度（轮胎外沿） |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2060 (mm) |
| 3 | Bucket width铲斗宽度 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2110(mm) |
| 4 | Height(To the top of the cab) 高度（到驾驶室顶） |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2820 (mm) |
| 5 | Wheel base轴距 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2350 (mm) |
| 6 | Tread轮距 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1500 (mm) |
| 7 | Min. ground clearance最小离地间隙 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 320(mm) |
| Main Technical Specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Rated load额定载重 |  |  |  |  |  |  |  |  |  |  |  |  | 2000(Kg) |  |  |
| 2 | Operating weight操作重量 |  |  |  |  |  |  |  |  |  |  |  |  | 6000KG |  |  |
| 3 | Rated bucket capacity额定斗容 |  |  |  |  |  |  |  |  |  |  |  |  | 1.1(m3) |  |  |
| 4 | Max. breakout force最大掘起力 |  |  |  |  |  |  |  |  |  |  |  |  | 60kN |  |  |
| 5 | Max. dump clearance最大卸载高度 |  |  |  |  |  |  |  |  |  |  |  |  | 2750(mm) |  |  |
| 6 | Dump reach卸载距离 |  |  |  |  |  |  |  |  |  |  |  |  | 920(mm) |  |  |
| 7 | Dump angle at any position任意位置卸载角 |  |  |  |  |  |  |  |  |  |  |  |  | ≥450 |  |  |
| 8 | 最大牵引力 |  |  |  |  |  |  |  |  |  |  |  |  |  | 48KN |  |
| 9 | 最大爬坡能力 |  |  |  |  |  |  |  |  |  |  |  |  |  | 25° |  |
| 10 | Min. turning radius最小转弯半径 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (1) Outside of bucket铲斗外沿 |  |  |  |  |  |  |  |  |  |  |  |  | 4650(mm) |  |  |
|  | (2)Center of rear wheel后轮胎中心 |  |  |  |  |  |  |  |  |  |  |  |  | 4420(mm) |  |  |
| 11 | Pivot angle frame铰接角 |  |  |  |  |  |  |  |  |  |  |  |  | ±380 |  |  |
| 12 | Lifting time of bucket铲斗举升时间 |  |  |  |  |  |  |  |  |  |  |  |  | ≤4.5(sec.) |  |  |
| 13 | Lowering time of bucket铲斗下降时间 |  |  |  |  |  |  |  |  |  |  |  |  | ≤3. (sec.) |  |  |
| 14 | Dumping time翻斗时间 |  |  |  |  |  |  |  |  |  |  |  |  | ≤1(sec.) |  |  |
| 15 | Traveling speed(Km/h)，2 forward and 2 reverse行驶速度，前2后2档位 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (1) 1st gear forward前进一档 |  |  |  |  |  |  |  |  |  |  |  |  | 10 |  |  |
|  | (2) 2nd gear forward前进二档 |  |  |  |  |  |  |  |  |  |  |  |  | 27 |  |  |
|  | (3) 1st gear reverse后退一档 |  |  |  |  |  |  |  |  |  |  |  |  | 10 |  |  |
|  | (3) 2nd gear reverse后退二档 |  |  |  |  |  |  |  |  |  |  |  |  | 27 |  |  |
| Diesel engine发动机 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Model型号 |  |  |  |  |  | Chinese LUOTUO LR4B3-G92X2A engine |  |  |  |  |  |  |  |  |  |  |
| 2 | Type形式 |  |  |  |  |  | Direct injection. Natural Aspirate. Water cooling 直喷，自然吸气，水冷 |  |  |  |  |  |  |  |  |  |  |
| 3 | Rated output额定功率 |  |  |  |  | 60 (kW) |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Rated speed额定转速 |  |  |  |  | 2400(r/min) |  |  |  |  |  |  |  |  |  |  |  |
| 5 | max torque 最大扭矩 |  |  |  |  | 286N.m/1600-1800 |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Starting type启动形式 |  |  |  |  | Electric电启动 |  |  |  |  |  |  |  |  |  |  |  |
| 7 | Voltage of starting motor 启动电压 |  |  |  |  | 24V |  |  |  |  |  |  |  |  |  |  |  |
| 8 | Inter-bore of cylinder/stroke 缸径×行程 |  |  |  |  | 108×120 |  |  |  |  |  |  |  |  |  |  |  |
| 9 | Total exhaust of cylinder 总排气量 |  |  |  |  | 4.58L |  |  |  |  |  |  |  |  |  |  |  |
| 10 | Compression ratio压缩比 |  |  |  |  | 17∶1 |  |  |  |  |  |  |  |  |  |  |  |
| 11 | method of lubrication 润滑方式 |  |  |  |  | Pressure and splash compound 压力与飞溅复合式 |  |  |  |  |  |  |  |  |  |  |  |
| 12 | Min. specific fuel consumption燃油消耗率 |  |  |  |  | ≤242g/kw.h |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 机油消耗率 |  |  |  |  | ≤1.6g/kw.h |  |  |  |  |  |  |  |  |  |  |  |
| Transmission System变速系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (一)Torque Converter变矩器 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Model型号 |  |  |  |  |  |  | Jingyi brand |  |  |  |  |  |  |  |  |
| 2 | Type形式 |  |  |  |  |  |  | 3-elements.single stage三相单级 |  |  |  |  |  |  |  |  |
| 3 | Torque ratio变矩比 |  |  |  |  |  |  | 3.06 |  |  |  |  |  |  |  |  |
| 4 | Cooling type冷却形式 |  |  |  |  |  |  | Pressure oil circulating压力油循环 |  |  |  |  |  |  |  |  |
| 5 | Oil pressure出口油压 |  |  |  |  |  |  | 0.245~0.343MPa |  |  |  |  |  |  |  |  |
| (二)、Transmission case变速箱 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Model型号 |  |  |  | Jingyi |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Type形式 |  |  |  | Mechanical control Fixed shaft-power shift 机械操纵定轴式动力换挡 |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Gearbox pump变速油泵 |  |  |  | CBGj1025齿轮泵（左） |  |  |  |  |  |  |  |  |  |  |  |
| 4 | clutch oil pressure 离合器油压 |  |  |  | 1.46~1.86MPa |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Transmission gear and ratio 变速 档位及速比 |  |  | 1st gear forward前进一档 | 2.9862 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2nd gear forward前进二档 | 0.6829 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1st gear reverse后退一速 | 2.508 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2nd gear reverse后退二速 | 0.599 |  |  |  |  |  |  |  |  |  |  |  |
| (三)、Axle and Tire桥和轮胎 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Type of main reducer 主传动形式 |  |  |  |  |  |  | spiral bevel gear, single stage 单级螺伞锥齿轮 |  |  |  |  |  |  |  |
| 2 |  | Gear ratio of main reducer 主传动速比 |  |  |  |  |  |  | 5.83 |  |  |  |  |  |  |  |
| 3 |  | Type of final reducer终传动形式 |  |  |  |  |  |  | Single stage planetary单级行星轮 |  |  |  |  |  |  |  |
| 4 |  | Gear of ratio of final reducer终传动速比 |  |  |  |  |  |  | 4.4 |  |  |  |  |  |  |  |
| 5 |  | Gear ratio总速比 |  |  |  |  |  |  | 32.66 |  |  |  |  |  |  |  |
| 6 |  | Size of tire轮胎型号 |  |  |  |  |  |  | 16/70-20 |  |  |  |  |  |  |  |
| 7 |  | Tire pressure轮胎压力 |  |  |  |  |  |  | 2.45kpa |  |  |  |  |  |  |  |
| Working Device Hydraulic System工作液压系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Model of oil pump工作油泵型号 |  |  |  |  |  |  |  |  |  | CBGJ2063 |  |  |  |  |
| 2 |  | System pressure系统压力 |  |  |  |  |  |  |  |  |  | 160(kgf/cm2) |  |  |  |  |
| 3 |  | Model of distribution valve分配阀型号 |  |  |  |  |  |  |  |  |  | ZS1-L20E-T-20 |  |  |  |  |
| 4 |  | Priority-valve优先阀型号 |  |  |  |  |  |  |  |  |  | YX2-F160L-N7 |  |  |  |  |
| 5 |  | (D*L)Dimension of lifting cylinder 举升油缸尺寸 |  |  |  |  |  |  |  |  |  | 2-Ф80*45*530(mm) |  |  |  |  |
| 6 |  | (D*L)Dimension of tilting cylinder翻斗油缸尺寸 |  |  |  |  |  |  |  |  |  | Ф100*60*380(mm) |  |  |  |  |
| Steering System转向系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Type 形式 |  | Middle articulated frame. Full-hydraulic Steering 车架铰接，全液压转向 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Model of steering gear转向器型号 |  |  |  |  |  |  |  |  |  | BZZ-320 |  |  |  |  |  |
| 3 | System pressure系统压力 |  |  |  |  |  |  |  |  |  | 180(kgf/cm2) |  |  |  |  |  |
| 4 | Dimension of steering cylinder转向油缸规格 |  |  |  |  |  |  |  |  |  | Ф63*35*300(mm) |  |  |  |  |  |
| Brake System制动系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Type of traveling brake 行车制动形式 |  |  |  |  |  |  |  |  | Assisted oil.4 wheel brake气顶液四轮制动 |  |  |  |  |  |  |
| 2 | Air pressure制动气压 |  |  |  |  |  |  |  |  | 5.88-7.85(kgf/cm2) |  |  |  |  |  |  |
| 3 | Type of parking brake停车制动形式 |  |  |  |  |  |  |  |  | Flexible shaft control Clamp brake 软轴操纵夹钳式制动器 |  |  |  |  |  |  |
| Oil Capacity油量 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Fuel(diesel)燃油 |  |  |  |  |  |  |  |  |  |  |  | 100(L) |  |  |  |
| 2 | Engine lubricating oil机油 |  |  |  |  |  |  |  |  |  |  |  | 16(L) |  |  |  |
| 3 | Oil for converter and gear box变速箱油 |  |  |  |  |  |  |  |  |  |  |  | 28(L) |  |  |  |
| 4 | Oil for hydraulic system液压油 |  |  |  |  |  |  |  |  |  |  |  | 100(L) |  |  |  |
| 5 | Oil for driving axles（F/R）前后驱动桥油 |  |  |  |  |  |  |  |  |  |  |  | 2x10(L) |  |  |  |
| 6 | Oil for brake制动液 |  |  |  |  |  |  |  |  |  |  |  | 3(L) |  |  |  |