# vehicle parameter table

| CHL5240JSQJ5 Type Truck Crane Transport Vehicle |  |  |  |
| --- | --- | --- | --- |
| Vehicle name: | truck with loading crane |  |  |
| class of vehicle ： | truck with loading crane |  |  |
| Vehicle model: | CHL5240JSQJ5 |  |  |
| exemption ： | deny |  |  |
| fuel oil ： | yes |  |  |
| exemption from conscription ： | deny |  |  |
| environmental protection ： | deny |  |  |
| Chinese brand: | Sinomach Heavy Industry brand | manufacturer ： | Sinomach Heavy Industry Group Changlin Co., Ltd. |
| engine type ： | CA6DK1-220 | engine power ： | 209/284 |
| engine displacement ： | 7146 | Engine manufacturer: | China First Automobile Group Corporation |
| Chassis model: | CA1250P62K1L3T1A1E | Fuel type: | diesel oil |
| Dimensions (length/width/height): | 12000×2521×3870 | Trunk (length/wide/height): | 8510×2405×550 |
| gross mass （Kg）： | 25000 | load utilization factor: | 1.21 |
| Preparation weight (Kg): | 16295 | Rated mass (Kg): | 8575 |
| ABS ： | have | Front passenger(s): | 2 |
| Approach angle/Departure angle: | 22/7 | Front/rear suspension (mm): | 1400/3450 |
| wheel base （mm）： | 5800+1350 | axle weight ： | 7000/18000 (two-axis set) |
| number of axles ： | 3 | Maximum speed (km/h): | 89 |
| oil wear ： | 30.74 | Number of spring tabs: | 12/10 |
| Number of wheels: | 10 | tyre size ： | 11.00R20 |
| front gauge ： | 1928,1950,1988 | rear track ： | 1878/1878 |
| Chassis standard: National V |  |  |  |

Note: The vehicle specifications are subject to the actual published parameters.

# crane technical parameters

| order number | project |  |  |  | unit |  | numeric value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | maximum lifting moment |  |  |  | kN·m |  | 300 |
| 2 | maximum lifting load |  |  |  | kg |  | 10000 |
| 3 | maximum working range |  |  |  | mm |  | 13500 |
| 4 | fully retracted boom length |  |  |  | mm |  | 5600 |
| 5 | Maximum lifting height (above ground) |  |  |  | mm |  | About 16,000 |
| 6 | lift ratio |  |  |  |  |  | 6/4 |
| 7 | maximum boom angle |  |  |  | (°) |  | 75 |
| 8 | minimum boom angle |  |  |  | (°) |  | 0 |
| 9 | transverse span of front leg |  |  |  | mm |  | 2284~5730mm |
| 10 | transverse span of rear outrigger |  |  |  | mm |  | 2200 |
| 11 | maximum rate of ascent | main hook Single rope | Lift | m/min |  | 55 |  |
|  |  |  | descend |  |  | 55 |  |
| 12 | swiveling speed |  |  |  | r/min |  | ≤2 |
| 13 | rotation angle |  |  |  | ° |  | 360° full rotation |
| 14 | crane self-weight |  |  |  | kg |  | 5020 |
| 15 | oil pump form |  |  |  | ------------- |  | gear pump + plunger pump |
| 16 | fuel tank capacity |  |  |  | L |  | 240 |
| 17 | working pressure of hydraulic system |  |  |  | MPa |  | 26 |
| 18 | flow rate of hydraulic system |  |  |  | L/min |  | 100 |

# 外形图 of crane

# Main features of crane

(1) Rotary device

The planetary reducer is used to reduce the speed, the rotation is more stable, the rotation angle is 360 degrees, and there is no dead point.

(2) Support leg system

horizontal support leg hydraulic cylinder

The front leg of the crane adopts an extended span to enhance stability, with a full extension span of 5.7 meters.

oil cylinder of front vertical support leg

The standard hydraulic support leg can adapt to various chassis heights.

posterior leg

The standard configuration features fixed rear outriggers for normal operation. An optional hydraulic rear outrigger can be installed to enhance overall stability, making it suitable for more complex and challenging conditions.

(3) Lifting Arm System

The hexagonal cross-section is adopted, and the lateral stability and arm cylinder stiffness are greatly improved compared with the traditional quadrilateral. One basic arm, three telescopic arms, double-stage hydraulic telescopic cylinder, three and four arms extend synchronously.

(4) Control System

crane control system

The crane is controlled by the main valve, which is located at the seat.

support leg control system

The support leg is independently controlled by the leg valve to prevent misoperation and enables bilateral manipulation.

(5) Safety devices

The standard configuration includes a height limit switch and over-rolling alarm device to provide maximum protection for the working equipment.

Optional high-power LED working lights are available for low-light conditions or nighttime operations.

Optional torque limiter for enhanced safety and reliability in the working environment.