# vehicle parameter table

| Sinomach Changlin truck-mounted crane and transport vehicle |  |  |  |
| --- | --- | --- | --- |
| Vehicle name: | truck with loading crane |  |  |
| class of vehicle ： | truck with loading crane |  |  |
| Vehicle model: | CA5180JSQP62K1L2A1 |  |  |
| exemption ： | deny |  |  |
| fuel oil ： | yes |  |  |
| exemption from conscription ： | deny |  |  |
| environmental protection ： | yes |  |  |
| Chinese brand: | Jiefang Plaque | manufacturer ： | China First Automobile Group Corporation |
| engine type ： | CA4DK1-22E6 | engine power ： | 165/224 |
| engine displacement ： | 4764 | Engine manufacturer: | China First Automobile Group Corporation |
| Fuel type: | diesel oil | oil wear ： | 26.25 |
| Dimensions (length/width/height): | 9000×2550×3650 | Trunk (length/wide/height): | 6300×2460×600 |
| Total mass (Kg): | 18000 | load utilization factor: | 1.14,1.07 |
| Preparation mass (Kg): | 11705,12000 | Rated load capacity (Kg): | 6100,5805,6165,5870 |
| ABS ： | have | Front passenger(s): | 3,2 |
| Approach Angle/Departure Angle: | 17/12 | Front/rear suspension (mm): | 1400/2600 |
| wheel base （mm）： | 5000 | axle weight ： | 6500/11500 |
| number of axles ： | 2 | Maximum speed (km/h): | 89 |
| Number of wheels: | 6 | tyre size ： | 10.00R20 |
| Conversion form: | steering wheel | Based on the criteria: | GB3847-2005, GB17691-2018 National V standards |
| Chassis base standard: | National VI |  |  |
| else ： | The vehicle is equipped with a straight-arm crane model SQS200, with a weight of 3315kg and a maximum lifting capacity of 8000kg. |  |  |

Note: The vehicle specifications are subject to the actual published parameters.

# crane technical parameters

| order number | project |  |  |  | unit |  | numeric value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | maximum lifting moment |  |  |  | kN·m |  | 200 |
| 2 | maximum lifting load |  |  |  | kg |  | 8000 |
| 3 | maximum working range |  |  |  | mm |  | 13300 |
| 4 | fully retracted boom length |  |  |  | mm |  | 4360 |
| 5 | Maximum lifting height (above ground) |  |  |  | mm |  | Approximately 15,300 |
| 6 | lift ratio |  |  |  |  |  | 6/4 |
| 7 | maximum boom angle |  |  |  | (°) |  | 77 |
| 8 | minimum boom angle |  |  |  | (°) |  | 0 |
| 9 | transverse span of front leg |  |  |  | mm |  | 2114~5740 |
| 10 | transverse span of rear outrigger |  |  |  | mm |  | 2200 |
| 11 | maximum rate of ascent and descent | Main hook (single rope) | Lift | m/min |  | 87 |  |
|  |  |  | descend |  |  | 87 |  |
| 12 | swiveling speed |  |  |  | r/min |  | ≤2.5 |
| 13 | rotation angle |  |  |  | ° |  | 360° full rotation |
| 14 | crane self-weight |  |  |  | kg |  | 3300 (including rear legs) |
| 15 | oil pump form |  |  |  | ------------- |  | gear pump |
| 16 | fuel tank capacity |  |  |  | L |  | 150 |
| 17 | working pressure of hydraulic system |  |  |  | MPa |  | 23 |
| 18 | flow rate of hydraulic system |  |  |  | L/min |  | 60 |

# 外形图 of crane

# crane characteristic curve

performance curve

H Hoist arm lifting height (M)

R hoist arm working radius (m)

# Main features of crane

(1) Rotary device

The worm gear reducer has a full rotation of 360 degrees and no dead point.

(2) Support leg system: Front horizontal support leg cylinder

The front outrigger of the crane is of the extended type to increase its stability, and the full extension span is 5.56 m.

Standard hydraulic support leg, adaptable to various chassis heights. Rear support leg

The standard configuration is fixed rear outriggers, which can meet the normal working conditions. The optional hydraulic rear outriggers can increase the stability of the vehicle and be suitable for more complex and harsh working conditions.

(3) Lifting Arm System

The hexagonal section is adopted, the lateral stability and arm cylinder stiffness are greatly improved compared with the traditional quadrilateral. One basic arm, three telescopic arms, a single hydraulic cylinder, the third and fourth arms are synchronized by rope arrangement.

(4) Control System of the Crane

The crane is controlled by the main valve, and the control system of the support leg can be realized on both sides.

The support leg is operated by the leg valve to avoid misoperation, and the same can be achieved on both sides.

(5) Safety devices

The standard configuration includes a height limit switch and over-rolling alarm device to maximize protection for the working apparatus. An optional high-power LED working light is available for low-light conditions or nighttime operations.

Optional torque limiter to provide customers with a safer and more reliable working environment.