GYS10J型单钢轮振动压路机

GYS10J SINGLE DRUMVIBRATORY ROLLER

| 规格型号：GYS10J型单钢轮振动压路机 |  |  |
| --- | --- | --- |
| Model: GYS10J Single Drum Vibratory Roller |  |  |
| 主要性能特点Performance characteristics： - 1、机械驱动，三档行驶速度。 Mechanical drive, multi-step traveling speed. - 2、液压振动，单频双幅，电控延时起振，激振力大，压实效率高； Hydraulic vibration, single-frequency double amplitude, electronically controlled delayed vibration, large excitation force, high compaction efficiency. - 3、液压转向，铰接式车架，操作轻便灵活； Hydraulic steering, articulated frame, light and flexible operation. - 4、配置带同步器的压路机专用变速箱，传动扭矩大，换挡轻便； Equipped with special gearbox for road rollers with synchronizer, high transmission torque and easy to shift. 5、气助力液压钳盘行车制动，内涨鼓式机械停车制动，安全可靠； Air-assisted hydraulic caliper disk running brake, internal drum mechanical parking brake, safe and reliable. 6、后机罩可大角度翻起，维修保养方便； Rear hood can be flipped up at a wide angle for easy maintenance. - 7、液压元件采用知名品牌，性能可靠； Hydraulic components adopt famous brand, reliable performance. - 8、标配驾驶室，可选装空调。 Standard cab, optional A/C. |  |  |
| 主要技术参数 Main Parameters： | GYS10J |  |
| 1、整机重量 Operating weight | 10000 kg |  |
| 2、振动轮直径 Vibratory drum diameter | 1430 mm |  |
| 3、振动轮宽度 Vibratory drum width | 1980 mm |  |
| 4、振动频率 Vibratory frequency | 30 Hz（theoretical） |  |
| 5、理论振幅 Vibratory amplitude | 1.6/0.8 mm |  |
| 6、激振力 Centrifugal force | 225/105 kN |  |
| 7、行驶速度 Traveling speed |  |  |
| 前进 Forward | 2.0/4.2/8.7 km/h |  |
| 后退 Reverse | 2.0/4.2 km/h |  |
| 8、最小离地间隙 Min. ground clearance | 380 mm |  |
| 9、轴距 Wheel base | 2900 mm |  |
| 10、静线载荷 Static line pressure | 240 N/cm |  |
| 11、最小转弯半径 Min. turning radius | 5800 mm |  |
| 14、最大爬坡能力 Gradeability | 30% |  |
| 15、外形尺寸（长×宽×高）Dimensions | 5570×2180×3010 mm |  |
| 16、发动机型号 Engine model | WP4G110E221 | WP4.1G102E301 |
| 型式 Type | 直列、四缸、四冲程、水冷 4 cylinder inline，4 stroke, water cooled |  |
| 功率 Power | 82 kW | 74.9 kW |
| 转速 Rated speed | 2000 r/min | 2400 r/min |
| 17、燃油箱容积 Fuel tank capacity | 240 L |  |
| 18、液压油箱 Hydraulic oil tank | 240 L |  |
| 19、轮胎 Tire type | 17.5-25 |  |

主要配置说明 Main configuration

| 序号 S/N | 部件名称 Part name | 型号规格 MODEL | 数量 No. | 制造商 Manufacturer |
| --- | --- | --- | --- | --- |
| 1 | 柴油机 Engine | WP4G110E221 WP4.1G102E301 | 1 | 潍柴 WEICHAI |
| 2 | 振动泵 Vibratory pump | P5100-F63 | 1 | 泊姆克 PERMCO |
| 3 | 振动马达 Vibratory motor | M5100（15-6） | 1 | 泊姆克 PERMCO |