液压单驱单钢轮振动压路机系列

HYDRAULIC REAR WHEEL DRIVE SINGLE DRUM VIBRATORY ROLLER SERIES

适用范围 Application

适用于高等级公路、大坝、铁路、机场等大型工程基础的压实，可碾压砂土、砾石、碎石、岩石填方等非粘性材料及半粘性材料

Suitable for compaction of large-scale engineering foundations such as high-grade roads, dams, railways, airports, etc., non-stick materials and semi-viscous materials such as sand, gravel, gravel, rock fill, etc.

性能特点 Characteristic

- 液压后轮驱动，两档行驶速度；

Hydraulic rear wheel drive, two speeds

- 液压开式振动，单频双幅电控起振，激振力大，压实性能好；

Open type hydraulic vibration, single frequency and double amplitude electric control, large excitation force, good compaction performance

- 液压转向，铰接式车架，转向轻便灵活，操作轻便灵活；

Hydraulic steering, articulated frame, light and flexible steering, light and flexible operation

- 配置压路机专用驱动桥，传动扭矩大，换挡轻便；

Equipped with the special drive axle of the roller, the transmission torque is large, and the shifting is light.

- 液压行车制动，盘式停车制动，安全可靠；

Hydraulic service brake, disc parking brake, safe and reliable

- 后机罩可大角度翻起，维修保养方便；

The rear hood can be turned up at a large angle for easy maintenance

- 液压元件采用国际知名品牌，性能可靠；

Hydraulic components adopt internationally renowned brands with reliable performance

- 标配驾驶室，可选装冷暖空调；

Standard cab, optional air conditioning

技术参数 Parameter

| 产品型号 MODEL |  | GYS161J | GYS181J | GYS201J |
| --- | --- | --- | --- | --- |
| 工作质量 Operating weight | kg | 16000 | 18000 | 20000 |
| 钢轮直径 Drum diameter | mm | 1550 | 1550 | 1600 |
| 压实宽度 Compaction width | mm | 2130 | 2130 | 2130 |
| 静线压力 Static line Pressure | N/cm | 370 | 415 | 460 |
| 振动频率 Vibratory frequency | Hz | 32 | 32 | 32 |
| 名义振幅 Vibratory amplitude | mm | 1.8/0.9 | 1.9/1.0 | 2.0/1.0 |
| 激 振 力 Centrifugal force | kN | 325/165 | 365/190 | 405/220 |
| 行驶速度 Travelling speed | km/h | 0-6/0-10 | 0-6/0-10 | 0-6/0-10 |
| 爬坡能力 Gradeability | % | 30 | 30 | 30 |
| 发动机型号 Engine model |  | WP6G160E22 | WP6G175E22 | WP6G175E22 |
| 发动机额定功率/转速 Rated power/speed | kW/ rpm | 118/2200 | 129/2200 | 129/2200 |
| 轮胎规格 Tyre |  | 20.5-25 | 20.5-25 | 20.5-25 |
| 外形尺寸（长×宽×高） Dimension （L×W×H） | mm | 6240×2280×3080 | 6240×2280×3080 | 6240×2300×3080 |