GYD06J型振动压路机

GYD06J DOUBLE DRUM VIBRATORY ROLLER

性能特点 Main features:

- 1.机械驱动，两档速度；

- Mechanical drive, two-speed gears.

- 2.液压后轮振动，单频单幅，电控起振，激振力大，压实效率高；

- Hydraulic rear wheel vibration, single-frequency single amplitude, electronically controlled starting vibration, large excitation force, high compaction efficiency.

- 3.铰接车架，转向灵活；

- Articulated frame, flexible steering.

- 4.电控压力喷洒水，防腐洒水箱和系统；

- Electronically controlled pressure spray water, anti-corrosion sprinkler tank and system.

- 5.液压元件采用知名品牌，性能可靠；

- Hydraulic components adopt well-known brands, reliable performance.

- 6.可选购平板拖车，便于施工转移；

- Optional flatbed trailer for easy construction transfer.

| 主要技术参数 Main parameters |  |
| --- | --- |
| - 整机重量：Operating weight | 6000 kg |
| - 振动轮直径：Vibratory drum diameter | 950 mm |
| - 压实宽度：Vibratory drum width | 1320 mm |
| - 振动频率：Vibratory frequency | 48 Hz |
| - 振幅：Vibratory amplitude | 0.62 mm |
| - 静线载荷：Static line pressure | 260 N/cm |
| - 行驶速度：Travelling speed | Forward Ⅰ 2.7km/h Ⅱ 8km/h |
|  | Reverse Ⅰ 2.7km/h Ⅱ 8km/h |
| - 激振力：Centrifugal force | 60 kN |
| - 爬坡能力：Gradeability | 20% |
| - 转弯半径：Min. turning radius | 5600 mm |
| - 轴距：Wheel base | 2570 mm |
| 最小离地间隙： Ground clearance | 240 mm |
| 燃油箱 Fuel tank | 60 L |
| 洒水箱 Sprinkler tank | 480 L |
| 液压油箱 Hydraulic oil tank | 80 L |
| 外形尺寸(长×宽×高)：Dimension | 3660×1550×2630 mm |
| 发动机 Engine |  |
| 发动机型号： Model | 4L68 |
| 制 造 厂：Manufacture | 常柴 CHANGCHAI |
| 功 率： Rated power | 40 kw |
| 转 速：Rated speed | 2600 r/min |

主要配置说明 Main configuration

| 序号 S/N | 部件名称 ITEM | 型号规格 MODEL | 数量 NO. | 制造商 MANUFACTURE |
| --- | --- | --- | --- | --- |
| 1 | 柴油机 Engine | 4L68 | 1 | 常柴股份有限公司 CHANGCHAI |
| 2 | 转向泵 Steering pump | CBW-F310 | 1 | 合肥长源 CHANGYUAN |
| 3 | 振动马达 Vibratory motor | GM5-16 | 1 | 派克(天津)特精液压有限公司 PAIKER |
| 4 | 振动泵 Vibration pump | G5-20 | 1 | 派克(天津)特精液压有限公司 PAIKER |