GYS202J全液压单钢轮振动压路机

GYS202J FULL HYDRAULIC SINGLE DRUM VIBRATORY ROLLER

主要性能特点：Main characteristic

- 1.液压前后轮驱动，两档无级变速，爬坡能力强。

- Fully hydraulic drive, two speeds, continuously variable.

- 2.液压振动，双频双幅，电控延时起振。

Hydraulic vibration, dual-frequency double amplitude, electronically controlled delayed vibration.

- 3.液压转向，铰接式车架，转向灵活

- Hydraulic steering, articulated frame, flexible steering.

- 4.液压行走制动，电控钳盘停车制动

Hydraulic service brake, electronically controlled caliper disk parking brake.

- 5.后机罩可大角度翻起，方便维修保养

Engine hood can be flipped up at a wide angle for easy maintenance.

- 6.主要液压元件采用国外知名品牌

The main hydraulic components adopt foreign famous brands.

- 7.标配驾驶室，可选装空调

Standard cabin with optional A/C.

| 规格型号：GYS202J全液压单钢轮振动压路机 |  |
| --- | --- |
| MODEL：GYS202J FULL HYDRAULIC SINGLE DRUM VIBRATORY ROLLER |  |
| 主要技术参数 Main parameter |  |
| 1、工作质量 Operating mass | 20000kg |
| 2、外形尺寸 Overall dimensions |  |
| 总长 Length | 6360 mm |
| 总宽 Width | 2300 mm |
| 总高 Height | 3100 mm |
| 最小离地间隙 Min. ground clearance | 450 mm |
| 轴距 Wheel base | 3150 mm |
| 3、发动机 Engine |  |
| 生产商 Manufacturer | WEICHAI |
| 型号 Model | WP6G190E22（Ⅱ） |
| 额定功率 Rated power | 140 kW |
| 额定转速 Rated speed | 2200 r/min |
| 冷却型式 Cooling type | Water cooled |
| 4、工作装置 Working equipment |  |
| 振动轮直径 Diameter of drum | 1600 mm |
| 压实宽度 Width of compaction | 2130 mm |
| 静线载荷 Static line pressure | 575 N.m |
| 名义振幅 Vibratory amplitude | 1.9/0.9 mm |
| 振动频率 Vibratory frequency | 28/32 Hz |
| 激振力 Centrifugal force | 385/260 kN |
| 5、行走系统 Walking system |  |
| 前进 Forward speed | 0-4/0-5.5/0-6.5/0-11 km/h |
| 后退 Reverse speed | 0-4/0-5.5/0-6.5/0-11 km/h |
| 最大爬坡能力 Gradeability | 40% |
| 最小转弯半径 Min. turning radius | 6500 mm |
| 轮胎规格 Tyre model | 20.5-25 |

主要配置说明 Main configuration

| Item | Model | Q’ty | Manufacturer |
| --- | --- | --- | --- |
| Engine | WP6G190E22 Optional MEE Ⅲ | 1 | WEICHAI |
| Travel pump | HPV105 | 1 | LINDE |
| Front travel motor | HMV075 | 1 | LINDE |
| Rear travel motor | HMV105 | 1 | LINDE |
| Vibration pump | HPV055 | 1 | LINDE |
| Vibration motor | HMF055 | 1 | LINDE |