# 724H MOTOR GRADER SPECIFICATIONS

- Equipped withCummins6CTA8.3-C240 engine

- Sing-handle electro-hydraulic controlled power-shift T/M, 6forward &3reverse shifts makes operation easy and convenient

- Hydraulic lock, auto lock/unlock NO-SPIN differential ensures stability and powerful traction

- Ideal axel load allocation provides excellent stability while cutting hard surface

- Serviceable, efficient hydraulic system enables every part to be fully used

- Box-typed frame and advanced T/M enable it to complete heavy-duty work

- A wide working range is accomplished through flexible blade suspension system and articulated frame

| - Overall Dimension整机尺寸 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Length总长 |  |  |  |  |  |  |  |  |  |  |  |  |  | 9980mm |
| 2 | Width总宽 |  |  |  |  |  |  |  |  |  |  |  |  |  | 2830mm |
| 3 | Height(To the top of the cab)总高（到驾驶室顶部） |  |  |  |  |  |  |  |  |  |  |  |  |  | 3630mm |
| 4 | Wheel base轴距 |  |  |  |  |  |  |  |  |  |  |  |  |  | 6900mm |
| 5 | Tread轮距 |  |  |  |  |  |  |  |  |  |  |  |  |  | 2580(mm) |
| Main Technical Specification主要技术参数 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Operating weight操作重量 |  |  |  |  |  |  |  |  |  |  |  |  | 18000kg |  |
| 2 | Blade length 刀板长度 |  |  |  |  |  |  |  |  |  |  |  |  | 4420mm |  |
| 3 | Blade height刀板高度 |  |  |  |  |  |  |  |  |  |  |  |  | 580mm |  |
| 4 | Shovel Blade thickness 铲刃板厚度 |  |  |  |  |  |  |  |  |  |  |  |  | 20mm |  |
| 5 | Traveling speed(Km/h)，6 forward and 3 reverse行驶速度，前六后三 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (1)forward前进 |  |  |  |  |  |  |  |  | 4.96/8.55/11.3/19.32/24.25/40 |  |  |  |  |  |
|  | (2) reverse后退 |  |  |  |  |  |  |  |  | 4.96/11.3/23 |  |  |  |  |  |
| Diesel engine发动机 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Model型号 |  |  |  |  | Cummins6CTA8.3-C240 engine |  |  |  |  |  |  |  |  |  |
| 2 | Type型式 |  |  |  |  | Direct injection. Turbocharged. Water cooling 直喷、增压、水冷 |  |  |  |  |  |  |  |  |  |
| 3 | Rated output额定功率 |  |  |  |  |  | 179 kW |  |  |  |  |  |  |  |  |
| 4 | Rated speed额定转速 |  |  |  |  |  | 2200(r/min) |  |  |  |  |  |  |  |  |
| 5 | Max. Torque最大扭矩 |  |  |  |  |  | 1028(N.m)/(1500r/min) |  |  |  |  |  |  |  |  |
| Transmission case变速箱 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Model型号 |  |  |  | 6WG200（ZF） |  |  |  |  |  |  |  |  |  |  |
| 2 | Type形式 |  |  |  | power shift, single-shift lever, netural-shift lock device 动力换挡、单手柄、中位自锁 |  |  |  |  |  |  |  |  |  |  |
| 3 | Gear shift position档位 |  |  |  | 6 Forward and 3 reverse gears 前六后三 |  |  |  |  |  |  |  |  |  |  |
| Axle and Tire桥和轮胎 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Max. drawing force 最大牵引力 |  |  |  |  | 103KN |  |  |  |  |  |  |  |
| 2 |  |  | Size of tire轮胎型号 |  |  |  |  | 20.5-25-16PR L-2 (standard) 标配 |  |  |  |  |  |  |  |
| Working Device Hydraulic System工作液压系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Model of oil pump(include steering pump) 油泵型号(双联泵) |  |  |  |  |  |  |  |  |  | CBGj1032/1032 |  |  |  |
| 2 |  | System pressure系统压力 |  |  |  |  |  |  |  |  |  | 17.5MPa |  |  |  |
| 3 |  | Model of distribution valve多路阀型号 |  |  |  |  |  |  |  |  |  | HUSCO 5000 |  |  |  |
| 4 |  | Model of oil motor回转马达型号 |  |  |  |  |  |  |  |  |  | 604-0663 |  |  |  |
| Steering System转向系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Type型号 |  |  | Fore Tiredeflexion前轮偏摆式转向 |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Model of steering gear转向器型号 |  |  |  |  |  |  |  |  |  | BZZ5-E160BG-1+FKB-3020C |  |  |  |  |
| 3 | System pressure系统压力 |  |  |  |  |  |  |  |  |  | 16MPa |  |  |  |  |
| Brake System制动系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Type of traveling brake 行车制动系统型式 |  |  |  |  |  |  |  | Hydraulic and inner-expanding brake 全液压内涨蹄式制动 |  |  |  |  |  |  |
| 2 | Model of brake pump制动泵型号 |  |  |  |  |  |  |  | CBGj1032/1032 |  |  |  |  |  |  |
| 3 | System pressure系统压力 |  |  |  |  |  |  |  | 10MPa |  |  |  |  |  |  |
| 4 | Type of parking brake停车制动型式 |  |  |  |  |  |  |  | Mechanical and inner-expanding brake 机械式内涨蹄式制动器 |  |  |  |  |  |  |
| Oil Capacity用油量 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fuel(diesel)燃油箱 |  |  |  |  |  |  |  |  |  |  |  |  | 515L |  |  |
| Engine lubricating oil发动机机油 |  |  |  |  |  |  |  |  |  |  |  |  | 28L |  |  |
| Oil for converter and gear box变速箱 |  |  |  |  |  |  |  |  |  |  |  |  | 28L |  |  |
| Oil for hydraulic system液压系统 |  |  |  |  |  |  |  |  |  |  |  |  | 90L |  |  |
| Oil for driving rear axles后桥 |  |  |  |  |  |  |  |  |  |  |  |  | 80L |  |  |
| Tandem box 平衡箱 |  |  |  |  |  |  |  |  |  |  |  |  | 165L×2 |  |  |

3、724H平地机标准配置表

Standard configuration table for Motor Grader 724H

| 序号 NO. | 名称 Item | 型号 Model | 厂家 Manufacturer | 产地 Origin |
| --- | --- | --- | --- | --- |
| 1 | 发动机 Engine | 6CTA8.3-C240 | 东风康明斯 Cummins | 中国 China |
| 2 | 变速箱 Transmission | 6WG200 | ZF | 中国 China |
| 3 | 驱动桥 Axle | 18MRD-X243JC | 美驰 Meritor | 中国 China |
| 4 | 轮胎 Tire | 20.5-25-16PR | 徐轮或前进 Triangle | 中国 China |
| 5 | 工作液压油泵: Working Pump | CBGj1032/1032 | 济南 JiNan | 中国 China |
| 6 | 转向装置 Steering System | BZZ5-E160BG-1+FKB-3020C | 镇江液压件厂 ZhenJiang | 中国 China |
| 7 | 回转马达 Rotary motor | 604-0663 | 伊顿 EATON | 中国 CHINA |
| 8 | 多路阀 Valve | 5000 | HUSCO | 美国 USA |