735T MOTOR GRADER SPECIFICATIONS

- Equipped with reliable Cummins QSM11-C360 engine

- Sing-handle electro-hydraulic controlled power-shift T/M, 6 forward & 3reverse shifts makes operation easy and convenient

- Hydraulic lock, auto lock/unlock NO-SPIN differential ensures stability and powerful traction

- Ideal axel load allocation provides excellent stability while cutting hard surface

- Serviceable, efficient hydraulic system enables every part to be fully used

- Box-typed frame and advanced T/M enable it to complete heavy-duty work

- A wide working range is accomplished through flexible blade suspension system and articulated frame

| - Overall Dimension整机尺寸 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Length总长 |  |  |  |  |  |  | 12500mm |
| 2 | Width总宽 |  |  |  |  |  |  | 3300mm |
| 3 | Height(To the top of the cab)总高（到驾驶室顶部） |  |  |  |  |  |  | 3700mm |
| 4 | Wheel base轴距 |  |  |  |  |  |  | 7760mm |
| 5 | Tread轮距 |  |  |  |  |  |  | 2650mm |
| 6 | Min. turning radius 最小转弯半径 |  |  |  |  |  |  | 9000mm |
| Main Technical Specification主要技术参数 |  |  |  |  |  |  |  |  |
| 1 | Operating weight操作重量 |  |  |  |  |  | 28000kg |  |
| 2 | Blade length 刀板长度 |  |  |  |  |  | 4920mm |  |
| 3 | Blade height刀板高度 |  |  |  |  |  | 695mm |  |
| 4 | Blade width 刀板厚度 |  |  |  |  |  | 25mm |  |
| 5 | Max. lift above ground 最大提升高度 |  |  |  |  |  | 515mm |  |
| 6 | Max. grading depth 最大切削深度 |  |  |  |  |  | 735mm |  |
| 7 | Max. shoulder reach outside rear tire 铲刀最大侧伸距离 |  |  |  |  |  | 2260mm（L）/1950mm（R） |  |
| 8 | Traveling speed(Km/h)，6 forward and 3 reverse行驶速度，前六后三 |  |  |  |  |  |  |  |
|  | (1) forward前进 |  |  |  |  | 5.4/8.2/12.7/19.3/27.5/41 km/h |  |  |
|  | (2) reverse后退 |  |  |  |  | 5.4/12.7/27.5 km/h |  |  |
| Diesel engine发动机 |  |  |  |  |  |  |  |  |
| 1 | Model型号 | Cummins QSM11-C360 康明斯QSM11-C360发动机 |  |  |  |  |  |  |
| 2 | Type型式 | EFI,turbocharger,air-to-aircooling,temperature control independent cooling system 电喷、涡轮增压、空空中冷；温控独立散热系统 |  |  |  |  |  |  |
| 3 | Rated output额定功率 | 268(kW) |  |  |  |  |  |  |
| 4 | Rated speed额定转速 | 2100(r/min) |  |  |  |  |  |  |
| 5 | Max. Torque最大扭矩 | 1830(N.m)/(1400r/min) |  |  |  |  |  |  |
| Transmission case变速箱 |  |  |  |  |  |  |  |  |
| 1 | Model型号 |  | 6WG310 （ZF technology） |  |  |  |  |  |
| 2 | Type型式 |  | electro-hydraulic controlled power shift single handle operation，automatic transmission 电液控制动力换档、自动变速箱，单手柄操作 |  |  |  |  |  |
| 3 | Gear shift position 档位数 |  | 6 Forward and 3 reverse gears 前六后三 |  |  |  |  |  |
| Axle and Tire桥和轮胎 |  |  |  |  |  |  |  |  |
| 1 | Max.drawing force 最大牵引力 |  |  | 144kN |  |  |  |  |
| 2 | Size of tire轮胎型号 |  |  | 18.00-25-28PR |  |  |  |  |
| Working Device Hydraulic System工作液压系统 |  |  |  |  |  |  |  |  |
| 1 | Model of oil pump油泵型号 |  |  |  |  | 丹佛斯 FRR074BLS2520-3S1N2A1 |  |  |
| 2 | System pressure系统压力 |  |  |  |  | 25MPa |  |  |
| 3 | Model of distribution valve多路阀型号 |  |  |  |  | Walvoil DRFC22132 DPX100/5 Walvoil DRFC22133 DPX100/5 |  |  |
| 4 | Model of oil motor回转马达型号 |  |  |  |  | Eaton 604-1113 |  |  |
| Steering System转向系统 |  |  |  |  |  |  |  |  |
| 1 | Type型式 |  |  |  |  | ForeTiredeflexion前轮偏摆式转向 |  |  |
| 2 | Model of steering gear转向器型号 |  |  |  |  | Eaton Xcel45-320 |  |  |
| 3 | System pressure系统压力 |  |  |  |  | 16MPa |  |  |
| Brake System制动系统 |  |  |  |  |  |  |  |  |
| 1 | Type of traveling brake 行车制动系统型式 |  |  |  | Full Hydraulic Wet Brakes全液压湿式制动 |  |  |  |
| 2 | Model of brake pump制动泵型号 |  |  |  | 泊姆克HPT2-35/HPT1-20/9 |  |  |  |
| 3 | System pressure系统压力 |  |  |  | 6.9MPa |  |  |  |
| 4 | Type of parking brake停车制动型式 |  |  |  | Hydraulically Operated Disc Brake 液压操纵盘式制动 |  |  |  |
| Oil Capacity用油量 |  |  |  |  |  |  |  |  |
| 1 | Fuel(diesel)燃油箱 |  |  |  |  | 540L |  |  |
| 2 | Engine lubricating oil发动机机油 |  |  |  |  | 28L |  |  |
| 3 | Oil for converter and gear box变速箱 |  |  |  |  | 35-40L |  |  |
| 4 | Oil for hydraulic system液压系统 |  | 200L |  |  |  |  |  |
| 5 | Oil for driving rear axles后桥 |  | 35L |  |  |  |  |  |
| 6 | Tandem box 平衡箱 |  | 2*60L |  |  |  |  |  |

3、735T平地机标准配置表

Standard configuration table for Motor Grader 735T

| 序号 NO. | 名称 Item | 型号 Model | 厂家Manufacturer | 产地 Origin |
| --- | --- | --- | --- | --- |
| 1 | 发动机 Engine | QSM11-C360 | 康明斯 Cummins | 中国 China |
| 2 | 变速箱 Transmission | 6WG310 | 采埃孚 ZF | 德国 GER |
| 3 | 驱动桥 Axle | 19MRW-X245E | 美驰 XZmeritor | 中国 China |
| 4 | 轮胎 Tire | 18.00-25-28PR | 轮胎 | 中国 China |
| 5 | 工作液压油泵: Working Pump | FRR074BLS2520-3S1N2A1 | 丹佛斯 Danfoss | 丹麦 Denmark |
| 6 | 转向器 Steering Gear | Xcel45-320 | 伊顿 Eaton | 美国 USA |
| 7 | 回转马达 Rotary motor | 604-1113 | 伊顿 Eaton | 美国 USA |
| 8 | 差速器: Differential | NO-SPIN | 美驰 XZmeritor | 中国 China |
| 9 | 多路阀 Valve | DRFC22132 DPX100/5 DRFC22133 DPX100/5 | 沃尔福 Walvoil | 意大利 Italy |
| 10 | 空调 Air conditioner | TY50D02 | 徐州天一 Tianyi | 中国 China |