# 713H MOTOR GRADER SPECIFICATIONS

- Sing-handle electro-hydraulic controlled power-shift T/M, 6forward & 3reverse shifts makes operation easy and convenient

- Hydraulic lock, auto lock/unlock NO-SPIN（OPTIONAL） differential ensures stability and powerful traction

- Ideal axle load allocation provides excellent stability while cutting hard surface

- Serviceable, efficient hydraulic system enables every part to be fully used

- Reliable full hydraulic brake system ensures safety

- Box-typed frame and advanced T/M enable it to complete heavy-duty work

- A wide working range is accomplished through flexible blade suspension system and articulated frame

| Overall Dimension整机尺寸 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Length整机长度 |  |  |  |  |  |  |  |  |  |  |  |  | 8290mm |
| 2 | Width整机宽度 |  |  |  |  |  |  |  |  |  |  |  |  | 2600mm |
| 3 | Height(To the top of the cab)总高（到驾驶室顶部） |  |  |  |  |  |  |  |  |  |  |  |  | 3465mm |
| 4 | Wheel base轴距 |  |  |  |  |  |  |  |  |  |  |  |  | 5780mm |
| 5 | Tread轮距 |  |  |  |  |  |  |  |  |  |  |  |  | 2120(mm) |
| 6 | Min. turning radius 最小转弯半径 |  |  |  |  |  |  |  |  |  |  |  |  | 6.6（m） |
| Main Technical Specification主要技术参数 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Operating weight操作重量 |  |  |  |  |  |  |  |  |  |  |  | 12000kg |  |
| 2 | Blade length 刀板长度 |  |  |  |  |  |  |  |  |  |  |  | 3658mm |  |
| 3 | Blade height刀板高度 |  |  |  |  |  |  |  |  |  |  |  | 545mm |  |
| 4 | Shovel Blade thickness 铲刃板厚度 |  |  |  |  |  |  |  |  |  |  |  | 20mm |  |
| 5 | Max. lift above ground 最大提升高度 |  |  |  |  |  |  |  |  |  |  |  | 410mm |  |
| 6 | Max. grading depth 最大切削深度 |  |  |  |  |  |  |  |  |  |  |  | 560mm |  |
| 7 | Max. shoulder reach outside rear tire 铲刀最大侧伸距离 |  |  |  |  |  |  |  |  |  |  |  | 2000mm |  |
| 8 | Traveling speed(Km/h)，6 forward and 3 reverse行驶速度，前六后三 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (1) forward前进 |  |  |  |  |  |  |  | 6.9/10.7/17.3/26/37.8/53.4 |  |  |  |  |  |
|  | (2) reverse后退 |  |  |  |  |  |  |  | 6.9/17.3/37.8 |  |  |  |  |  |
| Diesel engine发动机 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Model型号 |  |  |  |  | DongFeng Cummins 6BT5.9-C130 东康6BT5.9-C130 |  |  |  |  |  |  |  |  |
| 2 | Type型式 |  |  |  |  | Direct injection. Turbocharged. Water cooling 直喷、增压、水冷 |  |  |  |  |  |  |  |  |
| 3 | Rated output额定功率 |  |  |  |  | 97kW |  |  |  |  |  |  |  |  |
| 4 | Rated speed额定转速 |  |  |  |  | 2200(r/min) |  |  |  |  |  |  |  |  |
| 5 | Max. Torque最大扭矩 |  |  |  |  | 560(N.M)/1500rpm |  |  |  |  |  |  |  |  |
| Transmission case变速箱 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Model型号 |  |  |  | YD13 （CHINESE HANGCHI） |  |  |  |  |  |  |  |  |  |
| 2 | Type形式 |  |  |  | power shift, single-shift lever, natural-shift lock device 动力换挡、单手柄、中位自锁 |  |  |  |  |  |  |  |  |  |
| 3 | Gear shift position档位 |  |  |  | 6 Forward and 3 reverse gears前6后3 |  |  |  |  |  |  |  |  |  |
| Axle and Tire桥和轮胎 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Max. drawing force 最大牵引力 |  |  |  | 61.7KN（f=0.75） |  |  |  |  |  |  |  |
| 2 |  |  | Size of tire轮胎型号 |  |  |  | 17.5-25-12PR(standard) 标配 13.00-24-12PR(optional)选配 |  |  |  |  |  |  |  |
| Working Device Hydraulic System工作液压系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Model of tandempump 油泵型号 |  |  |  |  |  |  |  |  | HP51B678-07-07-7N |  |  |  |
| 2 |  | System pressure系统压力 |  |  |  |  |  |  |  |  | 17.5MPa |  |  |  |
| 3 |  | Model of distribution valve多路阀型号 |  |  |  |  |  |  |  |  | PD15.5CLR-00；PD15.5CLL-00 |  |  |  |
| 4 |  | Model of oil motor回转马达型号 |  |  |  |  |  |  |  |  | EATON, 604-0663 |  |  |  |
| Steering System转向系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Type形式 |  |  | Fore Tires deflexion前轮偏移式转向 |  |  |  |  |  |  |  |  |  |  |
| 2 | Model of redirector转向器型号 |  |  |  |  |  |  |  |  | BZZ5-E160B+FKB-3022 |  |  |  |  |
| 3 | System pressure系统压力 |  |  |  |  |  |  |  |  | 16MPa |  |  |  |  |
| Brake System制动系统 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Type of service brake 行车制动系统型式 |  |  |  |  |  |  | Full hydraulic and inner-expanding brake 全液压内涨蹄式制动 |  |  |  |  |  |  |
| 2 | Oil pressure系统压力 |  |  |  |  |  |  | 10Mpa |  |  |  |  |  |  |
| 3 | Type of parking brake停车制动形式 |  |  |  |  |  |  | Mechanical and inner expand hoof brake 机械式内涨蹄式制动器 |  |  |  |  |  |  |
| Oil Capacity油量 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Fuel(diesel)燃油箱 |  |  |  |  |  |  |  |  |  |  | 370L |  |  |
| 2 | Engine lubricating oil发动机机油 |  |  |  |  |  |  |  |  |  |  | 14L |  |  |
| 3 | Oil for converter and gear box变速箱 |  |  |  |  |  |  |  |  |  |  | 32L |  |  |
| 4 | Oil for hydraulic system液压系统 |  |  |  |  |  |  |  |  |  |  | 65L |  |  |
| 5 | Oil for driving rear axles后桥 |  |  |  |  |  |  |  |  |  |  | 17L |  |  |
| 6 | Tandem box 平衡箱 |  |  |  |  |  |  |  |  |  |  | 90L |  |  |

3、713H平地机标准配置表

| 序号 | 名称 | 型号 | 厂家 | 产地 |
| --- | --- | --- | --- | --- |
| 1 | 发动机 | 6BT5.9-C130 | 康明斯 | 中国 |
| 2 | 变速箱 | YD13 | 杭齿 | 中国 |
| 3 | 驱动桥 |  | 常林 | 中国 |
| 4 | 轮胎 | 17.5-25-12PR | 徐轮/贵轮 | 中国 |
| 5 | 工作液压油泵: | HP51B678-07-07-7N | 海克力斯 | 中国 |
| 6 | 转向装置 | BZZ5-E160B+FKB-3022 | 镇江液压件厂 | 中国 |
| 7 | 回转马达 | 604-0663 | 伊顿 | 中国 |
| 8 | 链条: |  | 东华/椿本（选） | 中国/日本 |
| 9 | 差速器: | NO-SPIN（选） | 常林/伊顿（选） | 中国/美国 |
| 10 | 多路阀 | PD15.5CL | 海宏 | 中国 |
| 11 | 液压锁 | WBCDELU-03-B等 | 奇威/EATON | 中国/美国 |
| 12 | 空调（选配） |  | 豫新 | 中国 |