732T-5 MOTOR GRADER

Front wheel auxiliary drive configuration and parameters

- Equipped with reliable Cummins QSM11-C330 engine

- Sing-handle electro-hydraulic controlled power-shift T/M, 6 forward & 3reverse shifts makes operation easy and convenient

- Hydraulic lock, auto lock/unlock NO-SPIN differential ensures stability and powerful traction

- Hydraulic system: dual handle electric control load sensitive system

- Steering system: fully hydraulic front wheel steering, articulated steering

- Service braking: fully hydraulic wet braking

- Parking brake: hydraulic operated disc brake

| - Overall Dimension整机尺寸 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Length总长 |  |  |  |  |  |  |  |  |  | 12750mm |  |
| 2 | Width总宽 |  |  |  |  |  |  |  |  |  | 3300mm |  |
| 3 | Height(To the top of the cab)总高（到驾驶室顶部） |  |  |  |  |  |  |  |  |  | 3700mm |  |
| 4 | Wheel base轴距 |  |  |  |  |  |  |  |  |  | 8010mm |  |
| 5 | Tread轮距 |  |  |  |  |  |  |  |  |  | 2650mm |  |
| 6 | Min. turning radius 最小转弯半径 |  |  |  |  |  |  |  |  |  | 9000mm |  |
| Main Technical Specification主要技术参数 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Operating weight操作重量 |  |  |  |  |  |  |  |  | 30000kg |  |  |
| 2 | Blade length 刀板长度 |  |  |  |  |  |  |  |  | 4620mm |  |  |
| 3 | Blade height刀板高度 |  |  |  |  |  |  |  |  | 695mm |  |  |
| 4 | Blade width 刀板厚度 |  |  |  |  |  |  |  |  | 25mm |  |  |
| 5 | Max. lift above ground 最大提升高度 |  |  |  |  |  |  |  |  | 515mm |  |  |
| 6 | Max. grading depth 最大切削深度 |  |  |  |  |  |  |  |  | 735mm |  |  |
| 7 | Max. shoulder reach outside rear tire 铲刀最大侧伸距离 |  |  |  |  |  |  |  |  | 2260mm(R)/1950mm(L) |  |  |
| 8 | Maximum inclination angle 最大倾斜角 |  |  |  |  |  |  |  |  | 65 ° |  |  |
| 9 | Cutting angle adjustment range切削角调整范围 |  |  |  |  |  |  |  |  | 23-73 ° |  |  |
| 10 | Max. swing angle of the front axle: 前桥最大摆动角 |  |  |  |  |  |  |  |  | 15 ° |  |  |
| 11 | Max. inclination angle of front wheel 前轮最大倾斜角 |  |  |  |  |  |  |  |  | 17 ° |  |  |
| 12 | Max. front wheel steering angle 前轮最大转向角 |  |  |  |  |  |  |  |  | 49 ° |  |  |
| 13 | Max. turning angle of articulation 铰接最大转向角 |  |  |  |  |  |  |  |  | 25 ° |  |  |
| 14 | Traveling speed(km/h)，6 forward and 3 reverse行驶速度，前六后三 |  |  |  |  |  |  |  |  |  |  |  |
|  | (1) forward前进 |  |  |  |  |  |  | 5.5、8.6、13.3、20.3、29、43 |  |  |  |  |
|  | (2) reverse后退 |  |  |  |  |  |  | 5.5、13.3、29 |  |  |  |  |
| Diesel engine发动机 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Model型号 |  |  |  |  |  |  |  |  | Cummins QSM11-C330 康明斯QSM11-C330发动机 |  |  |
| 2 | Type型式 |  |  |  |  |  |  |  |  | EFI,turbocharger,air-to-aircooling;temperaturecontrolindependentcoolingsystem 电喷、涡轮增压、空空中冷；温控独立散热系统 |  |  |
| 3 | Rated output额定功率 |  |  |  |  |  |  |  |  | 246(kW) |  |  |
| 4 | Rated speed额定转速 |  |  |  |  |  |  |  |  | 2100(r/min) |  |  |
| 5 | Max. Torque最大扭矩 |  |  |  |  |  |  |  |  | 1674(N.m)/(1400r/min) |  |  |
| Transmission case变速箱 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Model型号 |  |  |  |  |  |  |  |  |  |  | ZF 6WG260 |
| 2 | Type型式:电液控制动力换档、自动变速箱，单手柄操作 |  |  |  |  |  |  |  |  |  |  | electro-hydraulic controlled power shift single handle operation，automatic transmission |
| 3 | Gear shift position 档位数 |  |  |  |  |  |  |  |  |  |  | 6 Forward and 3 reverse gears 前六后三 |
| Axle and Tire桥和轮胎 Rear axle balance box:Meritor |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | Max. drawing force（f=0.73）最大牵引力 |  |  | 200kN |  |  |  |  |  |
| 2 |  |  |  | Size of tire轮胎型号 |  |  | 18.00-25-28PR |  |  |  |  |  |
| Working Device Hydraulic System工作液压系统 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Model of oil pump油泵型号 |  |  |  |  |  |  | 丹佛斯FRR074BLS2520-3S1N2A1 |  |  |  |
| 2 |  | System pressure系统压力 |  |  |  |  |  |  | 25MPa |  |  |  |
| 3 |  | Model of distribution valve多路阀型号 |  |  |  |  |  |  | Walvoil DRFC22132 DPX100/5 Walvoil DRFC22133 DPX100/5 |  |  |  |
| 4 |  | Model of oil motor回转马达型号 |  |  |  |  |  |  | Eaton 604-1113 |  |  |  |
| Steering System转向系统 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Type型式 |  |  |  |  |  | Fore Tire deflexion前轮偏摆式转向 |  |  |  |  |  |
| 2 | Model of steering gear转向器型号 |  |  |  |  |  | Eaton Xcel45-320 |  |  |  |  |  |
| 3 | System pressure系统压力 |  |  |  |  |  | 16MPa |  |  |  |  |  |
| Brake System制动系统 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Type of traveling brak额行车制动系统型式 |  |  |  |  | Full Hydraulic Wet Brakes全液压湿式制动 |  |  |  |  |  |  |
| 2 | Model of brake pump制动泵型号 |  |  |  |  | 泊姆克HPT3-50/HPT1-22/9 |  |  |  |  |  |  |
| 3 | System pressure系统压力 |  |  |  |  | 6.9MPa |  |  |  |  |  |  |
| 4 | Type of parking brake停车制动型式 |  |  |  |  | Hydraulically Operated Disc Brake 液压操纵盘式制动 |  |  |  |  |  |  |
| Front wheel auxiliary drive system前轮辅助驱动系统 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Model of front wheel drive pump 前驱油泵型号 |  | 驱动泵FRR090CLS2620--3S1C2A1 |  |  |  |  |  |  |  |
| 2 |  |  | Model of front wheel drive motor 前驱马达型号 |  | 左前轮马达4073601313 右前轮马达4073601213 |  |  |  |  |  |  |  |