GKM90P矿用自卸车

GKM90P Mine Dump Truck

| GKM90P Off-road mining dump truck |  |
| --- | --- |
| 型号 Type | GKM90P |
| 驾驶室 Driving cab： | 可翻转式宽体偏置驾驶室 Reversible wide-body offset cab |
| 发动机Engine Type/ Horsepower:： | WEICHAI 430马力WP12G430E310国三 WEICHAI 420马力WD12G420E211国二 |
| 变速箱 Transmission Type: | FAST 7DS220 Manual (without liquid retardation) |
| 转向器 Steering Engine: | 全液压转向Full hydraulic power steering engine |
| 前/后车桥 Front/rear axle: | 19T前桥/ 35T铸钢双级减速中后桥（鼓式制动） 19Tcasting steel front axle/35Tcasting dual reduction middle/rear axle |
| 车架 Frame:： | 焊接式刚性箱型车架 450*140 Welded rigid cube type frame 450*140 |
| 上装描述 Description: | 高强耐磨板400 底16mm Q345钢板边12mm 后12mm、前举、海沃举升系统、矿用厢 steel plate, bottom plate 16mm, side plate 12mm, HYVA hydraulic system, mining square box 货厢内尺寸（mm）Inner dimension of body：5850*3200*1750 |
| 轮胎Tyre: | 14.00R25 Mining Tubed Wire Tire矿用有内胎钢丝胎轮胎 14.00-25 Mining Tubed Bias Tire 矿用有内胎斜交胎 |
| 悬挂 Front/rear suspension: | 前油气悬挂/后整体焊接式平衡悬挂，加强型可拆换式平衡轴 Front oil and gas suspension/rear integral welded balance suspension, reinforced removable balance shaft |
| 板簧 Leaf spring for front, middle, rear axle: | 前：油气弹簧/后：钢板弹簧14*120*27 Front: Oil-gas spring Rear: 14*120*27 |
| 制动系统 Breaking system: | 瑞立 SORL system |
| 其他 Others: | 油浴式空气滤清器、排气制动、大箱底板加热、淋水、剔石器，底盘防护，油箱防护，挡泥板，LED侧灯和尾灯，电子空调 Oil bath air filter, exhaust brake, box floor heating, drenching, stone picker, chassis protection, fuel tank protection, fenders, LED side and tail lights, electronic air conditioning |
| 整车质量 Curb Weight(kg): | 30000 |
| 额定载质量 Rated load(kg): | 60000 |
| 总质量 Rated total weight(kg): | 90000 |
| 外形尺寸 Overall Dimension(mm): | 9100×3450×4130 |
| 最大速度 Max speed(km/h): | 42 |
| 最小离地间隙 Min ground clearance(mm): | 380 |
| 最小转弯半径 Min turning radius(m): | ≤12m |
| 最大爬坡度 Max gradeability(%): | 33% |
| 接近角/离去角 Approach angle/Departure angle(°): | 32°/39° |
| 轴距 Axle base(mm): | 3800+1560 |
| 前后轮距 Wheel span (front /rear)(mm): | PX90 :2792/2526 |
| 油箱 Oil tank Capacity(L): | 450L/600L |
| 货箱容积 Body Capacity: | 32.8m³ |
| 备注：（CNY） | 出口左侧驾驶室加2000元，出口右侧驾驶室加4000元 集中润滑加9200元 倒车影像加2500元，360度环视影像加4200元 液力缓速器加20000元，加液力缓速器需增加电子散热器，电子散热器加5000元 淋水刹车加4800元，铜水箱加12500元 海运整车打蜡加1000元 货箱材质全提升至高强耐磨板400材质加价12000元 |