GKM95P矿用自卸车

GKM95P Mine Dump Truck

| GKM95P Off-road mining dump truck |  |
| --- | --- |
| 型号Type: | GKM95P |
| 驾驶室Driving Cab: | 矿用宽体驾驶室（左驾） Reversible wide-body offset cab （Left hand） |
| 发动机Engine Type/ Horsepower: | 玉柴520马力发动机 YuChai YC6K520-GT30 E3 |
| 变速箱 Transmission Type: | 艾里逊Allison 4800 ORS-RP （AT） |
| 转向器 Steering Engine: | 伊顿全液压转向系统EATON Full hydraulic power steering engine |
| 前/后车桥 Front/rear axle: | 25吨前桥35吨中后桥 25Tcasting steel front axle/35T casting dual reduction middle/rear axle |
| 车架 Frame: | 焊接式半刚性车架 480*140 Welded Semi-rigid cube type frame 480*140 |
| 货箱尺寸 Inner dimension of body(mm): | 6000*3400*1300 |
| 上装描述 Description: | Q345钢板，底板16mm边板12mm，海沃179液压系统，矿用方箱 Q345 steel plate, bottom plate 16mm, side plate 12mm, Hyward 191 hydraulic system, mining square box |
| 轮胎Tyre: | 16.00-25 矿用轮胎 Mining tires |
| 悬挂 Front/rear suspension: | 全焊接平衡悬挂，加强可拆卸平衡轴 Whole welded equalizing suspension, reinforced removable equalizing shaft |
| 板簧 Leaf spring for front, middle, rear axle: | 前：油气弹簧 后：钢板弹簧 Front: Oil-gas spring Rear: leaf spring |
| 制动系统 Breaking system: | 瑞立制动系统 SORL system,Front / mid / rear axle drum brake |
| 其他 Others: | 油浴式滤清器，排气制动，底板加热，排石器，底盘防护，继电器防水保护，后视镜加固，LED侧灯和尾灯 Oil bath filter, exhaust brake, floor heating, stone ejector, chassis protection, relay waterproof protection, mirror reinforcement, LED side and tail lights |
| 整车质量 Curb Weight(kg): | 34000 |
| 额定载质量 Rated load(kg): | 65000 |
| 总质量 Rated total weight(kg): | 99000 |
| 外形尺寸 Overall Dimension(mm): | 9100×3650×4150 |
| 最大速度 Max speed(km/h): | 50 |
| 最小离地间隙 Min ground clearance(mm): | 435 |
| 最小转弯半径 Min turning radius(m): | ≤12 |
| 最大爬坡度 Max gradeability(%): | 33 |
| 接近角/离去角 Approach angle/Departure angle(°): | 37/39 |
| 轴距 Axle base(mm): | 3800+1690 |
| 前后轮距 Wheel span (front /rear)(mm): | Front2792, Rear 2526 |
| 油箱 Oil tank Capacity(L): | 600L Steel oil tank |
| 货箱容积 Body Capacity: | 平装Flat 38.7m³ |