GKM65P矿用自卸车

GKM65P Mine Dump Truck

| GKM65P Off-road mining dump truck |  |
| --- | --- |
| 型号 Type | GKM65P |
| 驾驶室 Driving cab： | 可翻转式宽体偏置驾驶室 Reversible wide-body offset cab |
| 发动机Engine Type/ Horsepower:： | WEICHAI 380马力国三 |
| 变速箱 Transmission Type: | FAST 7DS220 Manual (without liquid retardation) |
| 转向器 Steering Engine: | 液压助力Hydraulic power |
| 前/后车桥 Front/rear axle: | 13T 前桥 / 25T 铸钢双级减速中后桥 13T front axle/ 25T casting dual reduction tandem axle |
| 车架 Frame:： | 焊接式刚性箱型车架 450*130 Welded rigid cube type frame 450*130 |
| 上装描述 Description: | 底板 16mm 边板 12mm、前举、海沃 Hyva 油缸、方箱 Bottom 16mm, side 12mm, front hoisting, Hyva cylinder, square box 货厢内尺寸（mm）Inner dimension of body：5600*2850*1550 |
| 轮胎Tyre: | 14.00R25 Mining Tubed Wire Tire矿用有内胎钢丝胎轮胎 14.00-25 Mining Tubed Bias Tire 矿用有内胎斜交胎 |
| 悬挂 Front/rear suspension: | 钢板弹簧 Steel plate springs |
| 制动系统 Breaking system: | 瑞立 SORL system |
| 其他 Others: | 三级沙尘空滤、排气制动、大箱底板加热、排石器、底盘防护、电瓶防护、油箱防护、挡泥板、LED侧灯和尾灯、电子空调 Triple sand air filter, exhaust brake, heated big box floor, stone evacuator, chassis protection, battery protection, fuel tank protection, fenders, LED side and tail lights, electronic air conditioning |
| 整车质量 Curb Weight(kg): | 23000 |
| 额定载质量 Rated load(kg): | 40000 |
| 总质量 Rated total weight(kg): | 63000 |
| 外形尺寸 Overall Dimension(mm): | 8700*3000*3700 |
| 最小离地间隙 Min ground clearance(mm): | 380 |
| 最小转弯半径 Min turning radius(m): | ≤12 |
| 最大爬坡度 Max gradeability(%): | 30 |
| 轴距 Axle base(mm): | 3650+1560 |
| 油箱 Oil tank Capacity(L): | 400L |
| 货箱容积 Body Capacity: | 25m³ |