M82 CRYSTAL PARTITION

Modules

Product files

| ### MODULE A | M2. Supply and installation of Dynamobel M82 Crystal demountable partition with visible profiling, module A: totally blind. Standard module width: 1,200 mm. Total thickness: 80 mm. Made with Sendzimir galvanised-steel inner and outer frames per European standard EN10.142-A1:1994. (DX51D+Z). Previously degreased, visible metal components, electrostatically painted with polyester powder and polymerised at 200ºC. Total paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Interior insulation with 50-mm thick, 30 Kg/m3 dense mineral wool. 13-mm laminated plaster panels coated with 0.3-mm vinyl laminate. Panels and frames secured by 16-mm wide, painted-steel splice profiles. Acoustic insulation: 45 dB per UNE 74040. Reinstallation and interchangeability of modules. Acoustic insulation: 45 dB. Quality assurance UNE EN ISO 9002 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE D Glazed | M2. Supply and installation of Dynamobel M82 Crystal demountable partition with visible profiling, module D: totally glazed. 1,200-mm wide between axes for a pane to choose between 8-, 10- or 12-mm thick. Total thickness: 80 mm. Made with Sendzimir galvanised-steel perimeter frames per European standard EN 10.142-A1:1994. (DX51D+Z). Previously degreased, visible metal components, electrostatically painted with polyester powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. The levelled panes are 4+4, 5+5 or 6+6 mm (to choose) joined by a sheet of PVB. The frames are secured to each side of the glass. The modules are joined by 16-mm wide, painted-steel splice profiles. The panes can be demounted from the structure independently thanks to the perimeter frames. Module reinstallation and interchangeability. Quality assurance UNE EN ISO 9002 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE D Glazed | M2. Supply and installation of Dynamobel M82 Crystal demountable partition with visible profiling, module D: totally glazed. 2,400-mm wide between axes with two 1,200-mm wide panes, 8-, 10- or 12-mm thick (to choose), joined by a Polymethyl methacrylate PMMA overlap joint. Total partition thickness: 80 mm. Made with Sendzimir galvanised-steel perimeter frames per European standard EN 10.142-A1:1994. (DX51D+Z). Previously degreased, visible metal components, electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. The levelled panes are 4+4, 5+5 or 6+6 mm (to choose) joined by a sheet of PVB. The frames are secured to the bottom runner and each side of the glass. The modules are joined by 16-mm wide, painted-steel splice profiles. The panes can be demounted from the structure independently thanks to the perimeter frames. Module reinstallation and interchangeability. Quality assurance UNE EN ISO 9002 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE D Glazed | M2. Supply and installation of Dynamobel M82 Crystal demountable partition with visible profiling, module D: totally glazed. 3,600-mm wide between axes with three 1,200-mm wide panes, 10- or 12-mm thick (to choose), joined by a Polymethyl methacrylate PMMA overlap joint. Total partition thickness: 80 mm. Made with Sendzimir galvanised-steel perimeter frames per European standard EN 10.142-A1:1994. (DX51D+Z). Previously degreased, visible metal components, electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. The levelled panes are 5+5 or 6+6 mm (to choose) joined by a sheet of PVB. The frames are secured to the bottom runner, one on each side of the glass. The modules are joined by 16-mm wide, painted-steel splice profiles. The panes can be demounted from the structure independently thanks to the perimeter frames. Module reinstallation and interchangeability. Quality assurance UNE EN ISO 9002 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE D Glazed | M2. Supply and installation of Dynamobel M82 Crystal demountable partition with visible profiling, module D: totally glazed. Unlimited-length sections with three or more, as required, 1,200-mm wide panes, 12-mm thick, joined by Polymethyl methacrylate PMMA overlap joints. Total partition thickness: 80 mm. Made with Sendzimir galvanised-steel perimeter frames per European standard EN 10.142-A1:1994. (DX51D+Z). Previously degreased, visible metal components, electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. The levelled panes are 6+6 mm joined by a sheet of PVB. The frames are secured to the bottom runner, one on each side of the glass. The modules are joined by 16-mm wide, painted-steel splice profiles. The panes can be demounted from the structure independently thanks to the perimeter frames. Module reinstallation and interchangeability. Quality assurance UNE EN ISO 9002 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE G Door | M2. Supply and installation of Dynamobel M82 Crystal demountable partition with visible profiling, module G: glazed, floor-to-ceiling door, 909-mm wide between axes. Total partition thickness: 80 mm. Made with Sendzimir galvanised-steel perimeter frames per European standard EN 10.142-A1:1994. (DX51D+Z). Previously degreased, visible metal components, electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. 10-mm thick, 830-mm wide, Clarit tempered-glass door leaf, with matching hinges and latch. The modules are joined by 16-mm wide, painted-steel splice profiles. Module reinstallation and interchangeability. Quality assurance UNE EN ISO 9002 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |

| ### MODULE W Door | M2. Supply and installation of Dynamobel M82 Crystal demountable partition with visible profiling, module W: glazed, floor-to-ceiling, double-leaf door, 1,742-mm wide between axes. Total partition thickness: 80 mm. Made with Sendzimir galvanised-steel perimeter frames per European standard EN 10.142-A1:1994. (DX51D+Z). Previously degreased, visible metal components, electrostatically painted with polyester or epoxy powder and polymerised at 200ºC. Paint thickness of 50/60 micrometres and colour tolerance per DIN 5033. Two 10-mm thick, 830-mm wide, Clarit tempered-glass door leaves, with matching hinges and latch. The modules are joined by 16-mm wide, painted-steel splice profiles. Module reinstallation and interchangeability. Quality assurance UNE EN ISO 9002 certified by LLOYD’S. Includes all parts necessary for installation. Supplied and installed by manufacturer-approved distributor. |
| --- | --- |