Communication Protocol

for Electronic Load and PC

- Com Port Set

  - Baud Rate：9600

  - Data Bit：8

  - Stop Bit：1

  - Check：None

- Frame Format

The length of the frame is 26 (compatible with Fab), the format is as the following：

| AAH | Address | Command | Relative information of Bit 4 - 25 | Check |
| --- | --- | --- | --- | --- |

Explanation：

- The lock head is AAH and occupies one byte.

- The range of the Address is 0-FEH, and occupies one byte.

- The range of the Command is 90H-A0H and occupies one byte. The contents of the command is as the following:

  - 90H------To set the max current, the max power and the level of the voltage.

  - 91H------To read the current value, the voltage value, the power value and the state of the load. The state of the load includes the ON/OFF state of the load, over-voltage state, over-power state, over-heat state and mark of voltage polarity.

  - 92H------To control the ON/OFF of the load

  - 93H------Programming odd step1-5

  - 94H------Programming odd step 6-10

  - 95H------Start of programming output

  - 96H------Stop of programming output

- Byte 4 to byte 25 is the information contents.

- Byte 26 is the Check and is the accumulating sum of the former 25 bytes.

- The range of the voltage is 0-360V, and can be expressed in integer 0-360000.

- The range of the current is 0-30A, and can be expressed in integer 0-30000.

- The range of the power is 0-200W, and can be expressed in integer 0-2000.

- The range of the resistance is 0-500Ω, and can be expressed in integer 0-50000.

- Instruction of the Command

1. To set the max current, the max power and the level of the set value (90H)

| Byte 1 | AAH |
| --- | --- |
| Byte 2 | Address (0—FEH) |
| Byte 3 | Command (90H) |
| Byte 4 | Low byte of the max current |
| Byte 5 | High byte of the max current |
| Byte 6 | Low byte of the max power |
| Byte 7 | High byte of the max power |
| Byte 8 | New address of the Load |
| Byte 9 | Set mode of the electronic load: 1—current, 2—power, 3—resistance |
| Byte 10 | Low byte of the electronic load set value |
| Byte 11 | High byte of the electronic load set value |
| Byte 12 - 25 | System Reserved |
| Byte 26 | Check |

The set values of the current, the power and the load are all expressed by two bytes. Low byte is in the front and the high byte is at the back. For example:

The current value 3589H is expressed as the following:

| 89H | 35H |
| --- | --- |

2. To read the current, voltage, power and resistance of the electronic load (91H)

| Byte 1 | AAH |
| --- | --- |
| Byte 2 | Address (0—FEH) |
| Byte 3 | Command (91H) |
| Byte 4 | Low byte of the current |
| Byte 5 | High byte of the current |
| Byte 6 | Low byte of the low character of the voltage |
| Byte 7 | High byte of the low character of the voltage |
| Byte 8 | Low byte of the high character of the voltage |
| Byte 9 | High byte of the high character of the voltage |
| Byte 10 | Low byte of the power |
| Byte 11 | High byte of the power |
| Byte 12 | Low byte of the max current |
| Byte 13 | High byte of the max current |
| Byte 14 | Low byte of the max power |
| Byte 15 | High byte of the max power |
| Byte 16 | Low byte of the resistance value |
| Byte 17 | High byte of the resistance value |
| Byte 18 | Output state of the electronic load |
| Byte 19 - 25 | System reserved |
| Byte 26 | Check |

Output state of the electronic load is expressed by one byte and each digit unit is defined as the following:

From High to Low

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Bit 0: Being Controlled mode, 0 is for the keyboard controlling and 1 is for the PC controlling.

Bit 1: Output state. 0 is for the OFF of the output and 1 is for the ON of the output.

Bit 2: Polarity mark of the input voltage. 0 is for the right voltage polarity and 1 is for the wrong voltage polarity.

Bit 3: Over-heat mark of the input. 0 is for normal and 1 is for over-heat.

Bit 4: Over-voltage mark of the input. 0 is for normal 1 is for over-voltage.

Bit 5: Over-power mark of the input. 0 is for normal and 1 is for over-power.

Notes: The frame format of the electronic answering the PC is the same as the above format.

3. To control the output state of the electronic load (92H)

| Byte 1 | AAH |
| --- | --- |
| Byte 2 | Address (0-FEH) |
| Byte 3 | Command (92H) |
| Byte 4 | State of the electronic load |
| Byte 5- 25 | System Reserved |
| Byte 26 | Check |

The state of the load is expressed by one byte and the each bit unit is defined as the following:

From High to Low

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Bit 0: To control the output of the electronic load. 0 is for OFF and 1is for ON.

Bit 1: To modify the controlling mode of the electronic load. 0 is for the keyboard controlling and 1 is for the PC controlling.

4. Programming Odd Step 1-5 (93H)

| Byte 1 | AAH |
| --- | --- |
| Byte 2 | Address (0—FEH) |
| Byte 3 | Command (93H) |
| Byte 4 | Set mode of the electronic load output |
| Byte 5 | Programming step number (Step 1-10) |
| Byte 6 | Low byte of the output set of Odd step 1 |
| Byte 7 | High byte of the output set of Odd step 1 |
| Byte 8 | Low byte of the time of Odd step 1 |
| Byte 9 | High byte of the time of Odd step 1 |
| Byte 10 | Low byte of the output set of Odd step 2 |
| Byte 11 | High byte of the output set of Odd step 2 |
| Byte 12 | Low byte of the time of Odd step 2 |
| Byte 13 | High byte of the time of Odd step 2 |
| Byte 14 | Low byte of the output set of Odd step 3 |
| Byte 15 | High byte of the output set of Odd step 3 |
| Byte 16 | Low byte of the time of Odd step 3 |
| Byte 17 | High byte of the time of Odd step 3 |
| Byte 18 | Low byte of the output set of Odd step 4 |
| Byte 19 | High byte of the output set of Odd step 4 |
| Byte 20 | Low byte of the time of Odd step 4 |
| Byte 21 | High byte of the time of Odd step 4 |
| Byte 22 | Low byte of the output set of Odd step 5 |
| Byte 23 | High byte of the output set of Odd step 5 |
| Byte 24 | Low byte of the time of Odd step 5 |
| Byte 25 | High byte of the time of Odd step 5 |
| Byte 26 | Check |

5. Programming Odd Step 6-10 (94H)

| Byte 1 | AAH |
| --- | --- |
| Byte 2 | Address (0—FEH) |
| Byte 3 | Command (94H) |
| Byte 4 | Low byte of the output set of Odd step 6 |
| Byte 5 | High byte of the output set of Odd step 6 |
| Byte 6 | Low byte of the time of Odd step 6 |
| Byte 7 | High byte of the time of Odd step 6 |
| Byte 8 | Low byte of the output set of Odd step 7 |
| Byte 9 | High byte of the output set of Odd step 7 |
| Byte 10 | Low byte of the time of Odd step 7 |
| Byte 11 | High byte of the time of Odd step 7 |
| Byte 12 | Low byte of the output set of Odd step 8 |
| Byte 13 | High byte of the output set of Odd step 8 |
| Byte 14 | Low byte of the time of Odd step 8 |
| Byte 15 | High byte of the time of Odd step 8 |
| Byte 16 | Low byte of the output set of Odd step 9 |
| Byte 17 | High byte of the output set of Odd step 9 |
| Byte 18 | Low byte of the time of Odd step 9 |
| Byte 19 | High byte of the time of Odd step 9 |
| Byte 20 | Low byte of the output set of Odd step 10 |
| Byte 21 | High byte of the output set of Odd step 10 |
| Byte 22 | Low byte of the time of Odd step 10 |
| Byte 23 | High byte of the time of Odd step 10 |
| Byte 24 | Output mode of the programming (0 is for mono-time and 1 is for repetition.) |
| Byte 25 | System Reserved |
| Byte 26 | Check |

6. Start of the Programming Output (95H)

| Byte 1 | AAH |
| --- | --- |
| Byte 2 | Address (0-FEH) |
| Byte 3 | Command (95H) |
| Byte 4 - 25 | System Reserved |
| Byte 26 | Check |

7. Stop of the Programming Output (96H)

| Byte 1 | AAH |
| --- | --- |
| Byte 2 | Address (0-FEH) |
| Byte 3 | Command (96H) |
| Byte 4 - 25 | System Reserved |
| Byte 26 | Check |