BLINDER 4 IP

（WW+CW）

USER MANUAL

KEEP THIS USER MANUAL FOR FUTURE NEEDS

# MENU：

A            B          C         D

A． Menu

B． Up

C． Down

D． Enter

Operation instructions: Press the function key A to circulate 10 different function effects, the first two digits of the digital tube represent the current function (refer to the function table). The last two digits represent the location of the function. Address code, or speed, parameters. Press B or C to change its parameter value. Press D to confirm.

# DMX DISPLAY （Press D to confirm all functions selected）

| NO. | DISPLAY | DETAIL |
| --- | --- | --- |
| 1 | A001 | DMX address，（001—512） B 、C Key to add or subtract address values |
| 2 | 6/12/8 /4/2CH | Multiple channel modes to choose |
| 3 | CC00 | Jumping （00—12） B 、C to change effect |
| 4 | LL50 | Jumping speed （00—12） B 、C to change effect speed |
| 5 | CP 99 | fade，（01—99） B 、C fade,pulse speed |
| 6 | DE99 | Strobe（01—99） B、C to change fade,pulse speed |
| 7 | BEBE | Sound |
| 8 | o255 | WW color, （000—255） B 、C change |
| 9 | Y255 | CW color，（000—255） B 、C change |
| 10 | n255 | Strobe，（000—255） B 、C change |
| 11 | ON/OFF | Display saver on/off |

FUNCTION DISPLAY (D to save)

| NO. | DISPLAY | DETAIL |
| --- | --- | --- |
| 1 | R235 | COB1 WW current 20-255,B、C to change |
| 2 | G235 | COB1 CW current 20-255,B、C to change |
| 3 | B235 | COB2 WW current 20-255,B、C to change |
| 4 | o235 | COB2 CW current 20-255,B、C to change |
| 5 | Y255 | COB3 WW current 20-255,B、C to change |
| 6 | P235 | COB3 CW current 20-255,B、C to change |
| 7 | A235 | COB4 WW current 20-255,B、C to change |
| 8 | n235 | COB4 CW current 20-255,B、C to change |
| 9 | T080 | DMX dimmer speed 0-255,B、C to change |
| 10 | C-60 | Temperature control sets the temperature 40-70 degree,B、C to change |

# DMX512 CHANNEL (6CH)

| NO. | FUNCTION | DETAIL |
| --- | --- | --- |
| 1CH | Dimmer | COB dimmer,liner dimmer,from the dark to bright |
| 2CH | Strobe | Strobe from slow to fast |
| 3CH | Function | 0-5 Null 6--20： Jumping 1； 21—35: Jumping 2； 36—50: Jumping 3； 51—65: Jumping 4； 66—80: Jumping 5； 81—95: Jumping 6； 96—105: Jumping 7； 106—110: Jumping 8； 111—125: Jumping 9； 126—140: Jumping 10； 141—155: Jumping 11； 156—170: Jumping 12； 170—185: Jumping 1； 186—200: Jumping 2； 201—215: Jumping 3； 216—230: Strobe ; 231—255: Strobe by sound |
| 4CH | Function Speed | Function speed, from slow to fast |
| 5CH | WW color dimmer | WW color dimmer,liner dimmer,from the dark to bright |
| 6CH | CW color dimmer | CW color dimmer,liner dimmer,from the dark to bright |

# DMX512 CHANNEL (12CH)

| NO. | FUNCTION | DETAIL |
| --- | --- | --- |
| 1CH | Dimmer | COB dimmer,liner dimmer,from the dark to bright |
| 2CH | Strobe | Strobe from slow to fast |
| 3CH | Function | 0-5 Null 6--20： Jumping 1； 21—35: Jumping 2； 36—50: Jumping 3； 51—65: Jumping 4； 66—80: Jumping 5； 81—95: Jumping 6； 96—105: Jumping 7； 106—110: Jumping 8； 111—125: Jumping 9； 126—140: Jumping 10； 141—155: Jumping 11； 156—170: Jumping 12； 170—185: Jumping 1； 186—200: Jumping 2； 201—215: Jumping 3； 216—230: Strobe ; 231—255: Strobe by sound |
| 4CH | Function Speed | Function speed, from slow to fast |
| 5CH | WW 1 color dimmer | WW 1 color dimmer,liner dimmer,from the dark to bright |
| 6CH | CW 1 color dimmer | CW1 color dimmer,liner dimmer,from the dark to bright |
| 7CH | WW 2 color dimmer | WW 2 color dimmer,liner dimmer,from the dark to bright |
| 8CH | CW 2 color dimmer | CW 2 color dimmer,liner dimmer,from the dark to bright |
| 9CH | WW 3 color dimmer | WW 3 color dimmer,liner dimmer,from the dark to bright |
| 10CH | CW 3 color dimmer | CW 3 color dimmer,liner dimmer,from the dark to bright |
| 11CH | WW 4 color dimmer | WW 4 color dimmer,liner dimmer,from the dark to bright |
| 12CH | CW 4 color dimmer | CW 4 color dimmer,liner dimmer,from the dark to bright |

# DMX CHANNEL (8CH)

| NO. | FUNCTION | DETAIL |
| --- | --- | --- |
| 1CH | WW 1 color dimmer | WW 1 color dimmer,liner dimmer,from the dark to bright |
| 2CH | CW 1 color dimmer | CW1 color dimmer,liner dimmer,from the dark to bright |
| 3CH | WW 2 color dimmer | WW 2 color dimmer,liner dimmer,from the dark to bright |
| 4CH | CW 2 color dimmer | CW 2 color dimmer,liner dimmer,from the dark to bright |
| 5CH | WW 3 color dimmer | WW 3 color dimmer,liner dimmer,from the dark to bright |
| 6CH | CW 3 color dimmer | CW 3 color dimmer,liner dimmer,from the dark to bright |
| 7CH | WW 4 color dimmer | WW 4 color dimmer,liner dimmer,from the dark to bright |
| 8CH | CW 4 color dimmer | CW 4 color dimmer,liner dimmer,from the dark to bright |

# DMX CHANNEL (4CH)

| NO. | FUNCTION | DETAIL |
| --- | --- | --- |
| 1CH | Dimmer | COB dimmer,liner dimmer,from the dark to bright |
| 2CH | WW dimmer | WW dimmer,liner dimmer,from the dark to bright |
| 3CH | CW dimmer | CW dimmer,liner dimmer,from the dark to bright |
| 4CH | Strobe | Strobe from slow to fast |

# DMX CHANNEL (2CH)

| NO. | FUNCTION | DETAIL |
| --- | --- | --- |
| 1CH | WW dimmer | WW dimmer,liner dimmer,from the dark to bright |
| 2CH | CW dimmer | CW dimmer,liner dimmer,from the dark to bright |

# RDM

| NO. | FUNCTION | DETAIL |
| --- | --- | --- |
| 1 | Feedback (address code) | Can be changed freely |
| 2 | Feedback (channel mode) | Can be changed freely |
| 3 | Feedback(device temperature) | Can’t be changed freely |

TECHNICAL PARAMETER

Voltage: AC100~240V,50/60HZ

Power consumption: 400W

Light source: 4pcs 100W WW+CW

LED control: individual controllable

DMX channels: 6/12/8//4/2CH

Color temperature:3200K~7000 Adjustable

Dimmer: 16bit 0~100% linear dimmer

Beam angle: 38 degree

Work temperature: -30degreee~50degree

Control model: DMX512,Auto,Master-Slave,Sound active

Strobe: 1~30HZ

House: Metal,black

Connection model: DMX in/out, Power IN/OUT

IP grade: IP65

Size: 394*245*392

Weight: 17kg

FIXTURE SIZE

REMARK

The product has perfect performance and intergrity packing. All users should be strictly comply with the warning

and operating instructions as stated. Or we aren’t in charge of any result by misusing.

Any damage resulting by misuse is not within the Company’s warranty.

Any fault or problem caused by neglecting the manual is also not in the charge of dealers.

Errors and omissions for every information given in this manual excepted.

All information is subject to change without prior notice.

Thank you again for your patronage!

We will be offering sincere service as always,

Wish you have a good lighting journey!