BLINDER 2

(WW+CW)

USER MANUAL

KEEP THIS USER MANUAL FOR FUTURE NEEDS

MENU

1.Keystroke diagram

MENU： function key

UP： additional key

DOWN： Subtract key

ENTER： Confirmation key

## 2.  Button Description MENU function key:

Press the function  key  MENU will  loop  out of the table  (1) function, digital tube in front of 1-2 to represent the current function. The latter 2-3  digits  represent  the  speed,  effect  and  other  parameters  of  the function;  the  parameter  values  can  be  modified  by  pressing  UP  or DOWN  key;  and  the  parameter  value  can  be  confirmed  by  pressing ENTER key.

UP additional key:

Press the key can be added, the length of more than 2 seconds if added continuously;

DOWN Subtract key:

According to the invention, the key can be reduced, and the length can be reduced continuously for more than 2 seconds;

ENTER Confirmation key:

Press the key to save the currently set parameter function, the next boot automatically run the save function.

| NO | DIS | VALUE | FUNCTION |
| --- | --- | --- | --- |
| 1 | d001 | 001- 512 | DMX512 Address code setting -4 channel mode |
| 2 | A001 | 001- 512 | DMX512 Address code setting -8 channel mode |
| 3 | U255 | 000 - 255 | Cold white dials, from dark to bright. |
| 4 | C255 | 000 - 255 | Warm white dials from dark to bright. |
| 5 | FH99 | 01 - 99 | Stroboscopic, from slow to fast. |
| 6 | CC99 | 01 - 99 | Jump, from slow to fast. |
| 7 | DE99 | 01 - 99 | Gradient, from slow to fast. |
| 8 | CP99 | 01 - 99 | Pulse change, from slow to fast. |
| 9 | S0UD | - | Sound control |

CHANNEL LIST

| CH 8 | FUNCTION | DESCRIPTION |
| --- | --- | --- |
| CH1 | Dimming | dimming， From dark to bright |
| CH2 | CW1 dimming | WW1 dimming， From dark to bright |
| CH3 | WW2 dimming | WW2 dimming， From dark to bright |
| CH4 | CW1 dimming | WW1 dimming， From dark to bright |
| CH5 | WW2 dimming | WW2 dimming， From dark to bright |
| CH6 | Total strobe | Total strobe, slow to fast |
| CH7 | Function | 0-50:DMX dimming; 51-100: common color output (8); 101-150: jump; 151-200: gradual; 201-250 pulse; 251-255: voice control |
| CH8 | Function speed | Functional speed, from slow to fast |

| CH 4 | FUNCTION | DESCRIPTION |
| --- | --- | --- |
| CH1 | CW1 dimming | WW1 dimming， From dark to bright |
| CH2 | WW2 dimming | WW2 dimming， From dark to bright |
| CH3 | CW1 dimming | WW1 dimming， From dark to bright |
| CH4 | WW2 dimming | WW2 dimming， From dark to bright |

TECHNICAL PARAMETER

Voltage: AC100~240V,50/60HZ

Power consumption: 200W

Light source: 2pcs 100W Warm white

LED control: individual controllable

DMX channels: 4/8CH

Color temperature:3200K~7000 Adjustable

Dimmer: 16bit 0~100% linear dimmer

Beam angle: 38 degree

Work temperature: -30degreee~50degree

Control model: DMX512,Auto,Master-Slave,Sound active

Strobe: 1~30HZ

House: Metal,black

Connection model: DMX in/out, Power IN/OUT

IP grade: IP20

Size: 375*301*180MM

Weight: 3kg

FIXTURE SIZE

REMARK

The product has perfect performance and intergrity packing. All users should be strictly comply with the warning

and operating instructions as stated. Or we aren’t in charge of any result by misusing.

Any damage resulting by misuse is not within the Company’s warranty.

Any fault or problem caused by neglecting the manual is also not in the charge of dealers.

Errors and omissions for every information given in this manual excepted.

All information is subject to change without prior notice.

Thank you again for your patronage!

We will be offering sincere service as always,

Wish you have a good lighting journey!