BLINDER 4

（WW+CW）

USER MANUAL

KEEP THIS USER MANUAL FOR FUTURE NEEDS

- DISPLAY MENU

A．Function key B．UP key C．Down key

D．Enter key

A	B	C	D

Operate introduce: Press function key “A” will circulate seven different function effects, The line of words upside LCD stand for the present working function ( please refer to the function table.), the under line stands for LCD shows what address code or speed parameter of this function is at.

- DISPLAY MENU FUNCTION ( Press D key to confirm after choosing certain function)

| NO. | DISPLAY | FUNCTION |
| --- | --- | --- |
| 1 | A002 | 12 channel address code, UP/DOWN key selects the address code (A001 ~ A512), press the confirmation key (ENTER key) to save the address code |
| 2 | L001 | 4 channel mode, UP/DOWN key selects the address code (L001 ~ L512), press the confirmation key (ENTER key) to save the address code |
| 3 | d002 | 2 channel mode, the UP/DOWN key selects the address code (d001 ~ d512), press the confirmation key (ENTER key) to save the address code |
| 4 | M000 | macro |
| 5 | S000 | Functional mode speed |
| 6 | P000 | The frequency flash is from slow to fast |
| 7 | R255 | (r000-r255) up, manually modify the bulb 1 cold white brightness |
| 8 | G255 | (G000 - G255) upward, downward manually modify the lamp bead 1 warm white brightness |
| 9 | B255 | (B000 - B255) upward, manually modify the lamp bead 2 cold white brightness |
| 10 | W255 | (w00-w255) upward, downward manually modify the lamp ball 2 warm and white brightness |
| 11 | Y255 | (y000-y255) upward, downward manually modify the bulb 3 cold white brightness |
| 12 | I255 | (i000-i255) upward, downward manually modify the bulb 3 warm white brightness |
| 13 | F255 | (F000 - F255) up and down manually modify the bulb 4 cold white brightness |
| 14 | H255 | (H000 - H255) upward, downward manually modify the bulb 4 warm white brightness |

- CHANNEL LIST

12CH Description

| NO. | FUNCTION | DESCRIPTION |
| --- | --- | --- |
| 1CH | 000-255 | The total dimming |
| 2CH | 000-255 | 0-9 no stroboscopic 10-255 strobes speed from slow to fast |
| 3CH | 250-255 | macro |
| 4CH | 000-255 | Built-in mode speed adjustment |
| 5CH | 000-255 | R - light bulb 1 cold white linear dimming, from dark to light |
| 6CH | 000-255 | G - light bulb 1 warm white linear dimming, from dark to light |
| 7CH | 000-255 | B-lamp 2 cold white linear dimming, from dark to light |
| 8CH | 000-255 | W - bulb 2 warm white linear dimming, from dark to light |
| 9CH | 000-255 | Y - light bulb 3 cold white linear dimming, from dark to light |
| 10CH | 000-255 | I - light bulb 3 warm white linear dimming, from dark to light |
| 11CH | 000-255 | F - bulb 4 cold white linear dimming, from dark to light |
| 12CH | 000-255 | H - bulb 4 warm white linear dimming, from dark to light |

4CH Description

| NO. | FUNCTION | DESCRIPTION |
| --- | --- | --- |
| 1CH | 000-255 | Tuned light |
| 2CH | 000-255 | R/G/Y/F brightness is adjusted from dark to light |
| 3CH | 000-255 | B/W/I/H brightness is adjusted from dark to light |
| 4CH | 000-255 | 0-9 no stroboscopic 10-255 strobes speed from slow to fast |

2CH Description

| NO. | FUNCTION | DESCRIPTION |
| --- | --- | --- |
| 1CH | All Dimmer | From dark to bright |
| 2CH | 000-255 | 0-9 no stroboscopic 10-255 strobes speed from slow to fast |

TECHNICAL PARAMETER

Voltage: AC100~240V,50/60HZ

Power consumption: 400W

Light source: 4pcs 100W Warm white

LED control: individual controllable

DMX channels: 12/4/2CH

Color temperature:3200K~7000 Adjustable

Dimmer: 16bit 0~100% linear dimmer

Beam angle: 38 degree

Work temperature: -30degreee~50degree

Control model: DMX512,Auto,Master-Slave,Sound active

Strobe: 1~30HZ

House: Metal,black

Connection model: DMX in/out, Power IN/OUT

IP grade: IP20

Size:36x38x20CM

Weight: 9.0K

FIXTURE SIZE

REMARK

The product has perfect performance and intergrity packing. All users should be strictly comply with the warning

and operating instructions as stated. Or we aren’t in charge of any result by misusing.

Any damage resulting by misuse is not within the Company’s warranty.

Any fault or problem caused by neglecting the manual is also not in the charge of dealers.

Errors and omissions for every information given in this manual excepted.

All information is subject to change without prior notice.

Thank you again for your patronage!

We will be offering sincere service as always,

Wish you have a good lighting journey!